# 3GPP TSG-RAN WG4 Meeting #114                                                                R4-2500413

Athens, Greece, 17-21 February, 2025

Title: 	Remaining RF issues for supporting 3MHz CBW in NTN bands

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.2

Document for:	Discussion

# Introduction

RAN4 113 meeting the WF[1] was agreed, and some RF requirements are remained as FFS in this document, those open issues will be addressed.

# Discussion

Issue 2-1-0: 2ms time limitation - WF: - FFS: The 2 ms time limitation may not be ignored in spectral analysis. A sufficient number of 2 ms averaging windows is needed to make simulation results reliable.

The 2ms averaging window needs to be respected for AMPR evaluations for NS_02N, NS_03N, NS_04N and NS_05N. In our evaluations[2],[3] this condition was considered.

Observation 1: Our A-MPR evaluations for NS_02N, NS_03N, NS_04N and NS_05N include the 2ms averaging window.

Issue 2-1-2: A-MPR for NS_03N - WF: - Discuss further NS_03N A-MPR requirements for 3MHz CBW based on Table 1 and Table 2 Table 1: A-MPR regions for NS_03N Channel BW; Carrier Center Frequency; RB_start*12*SCS (MHz); LCRB*12*SCS (MHz); A-MPR. 3MHz; Option 1: 1611.5 <= fc < 1613 Option 2: fc < 1613.9 Option 3: 1611.5 <= fc < 1612.9; Option 1: >= 2.16 Option 2: >= 1.8 Option 3: >= 2.52; Option 1: A2 Option 2: A3 Option 3: A6. Table 2: A-MPR for NS_03N Modulation; Option 1: A2; Option 2: A3; Option 3: A6. DFT-s-OFDM; Pi/2 BPSK; 1.0; 1.5. QPSK; 3.5; 2.5; 1.5. 16QAM; 4.0; 3.0. 64QAM; 4.5; 3.5. 256QAM; 5; 4.5. CP-OFDM; QPSK; 4.5; 4.0; 3.5. 16QAM; 4.5; 4.0. 64QAM; 4.5; 4.0. 256QAM; 4.5; 4.0.

As there are 3 proposals for NS_03N, we can have an arithmetic averaging among the AMPR values. It should be mentioned that for PI/2 BPSK we did not need any the AMPR values therefore it was left blank in Option 1.

Proposal 1: Consider an arithmetic averaging among the AMPR values of NS_03N.

Issue 2-1-5: A-MPR for NS_24 - Way forward: - Discuss further A-MPR for NS_24 with 3MHz CBW based on Table 3 and Table 4. Table 3: A-MPR for NS_24 for 3MHz CBW Channel Bandwidth, MHz; Carrier Centre Frequency, Fc, MHz; Region A. RBend*12*SCS MHz; LCRB*12*SCS MHz; A-MPR. 3MHz; 1997.5 < Fc ≤ 2000.5; Option 1: ≥2.16 Option 2: ≥1.44; Option 1: A3 Option 2: A8. 3MHz; 2000.5 < Fc ≤ 2002; Option 1: ≥2.16 Option 2: ≥1.44 Option 3: [≥1.44]; Option 1: A4 Option 2: A8 Option 3: [A3]. 3MHz; 2002 < Fc ≤ 2003.5; [≥1.44]; Option 1: A4 Option 2: A8. NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies. Table 4: A-MPR for modulation and waveform type Modulation/Waveform; Option 1: A3; Option 1: A4; Option 2: A8. Outer/Inner; Outer/Inner; Outer. DFT-s-OFDM PI/2 BPSK; ≤ [3.5]. DFT-s-OFDM QPSK; ≤ 3.5; ≤ 7; ≤ [4]. DFT-s-OFDM 16 QAM; ≤ 3.5; ≤ 8; ≤ [4.5]. DFT-s-OFDM 64 QAM; ≤ 3.5; ≤ 8.5; ≤ [5]. DFT-s-OFDM 256 QAM; ≤ 8; ≤ [5]. CP-OFDM QPSK; ≤ 4.5; ≤ 8.5; ≤ [6]. CP-OFDM 16 QAM; ≤ 4.5; ≤ 8.5; ≤ [6]. CP-OFDM 64 QAM; ≤ 4.5; ≤ 8.5; ≤ [6]. CP-OFDM 256 QAM; ≤ 8.5; ≤ [6.5]. NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 2.1-1.

The evaluations of different companies show that AMPR values for NS_24 is needed. Moreover, as its values are different than the ones of TN, it is better to rename it as NS_24N for NTN communications.

Proposal 2: Update NS_24 to NS_24N for NTN communications. Consider an arithmetic averaging among the AMPR values of NS_24N for every overlapping region.

Issue 2-2-1: Reference sensitivity - Option 1: - 2.2dB improvement compared with 5 MHz CBW; - Option 2: - 1.7dB improvement compared with 5 MHz CBW; - Encourage companies to consider Rx non-linearity in the reference sensitivity evaluations

In TS36.101-1 36% of the bands take 1.7dB improvement and 60% of the bands take 2.2dB improvement, so the majority requires 2.2dB. Moreover, it is true that due to IIP2, some bands such as B2, B3and B12 (all with small duplex distance) had 1.7dB improvement as indicated in R4-103187 and R4-103188. However, the duplex distance between the NTN bands are much larger than those bands. Therefore Option 1 is our preference.

Proposal 3: As the duplex distance of NTN bands are large enough, 2.2dB improvement compared with 5 MHz CBW shall be considered.

# Conclusions

Observation 1: Our AMPR evaluations for NS_02N, NS_03N, NS_04N and NS_05N include the 2ms averaging window.

Proposal 1: Consider an arithmetic averaging among the AMPR values of NS_03N.

Proposal 2: Update NS_24 to NS_24N for NTN communications. Consider an arithmetic averaging among the AMPR values of NS_24N for every overlapping region

Proposal 3: As the duplex distance of NTN bands are large enough, 2.2dB improvement compared with 5 MHz CBW shall be considered.

# Reference

[1]	“R4-2419789, Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi.”

[2]	“R4-2416255, A-MPR evaluations of NS_03N, NS_N04N, NS_05N and NS_24 for 3MHz CBW for NTN communication, Huawei, HiSilicon.”

[3]	“R4-2419156, Remaining RF issues for supporting 3MHz CBW in NTN bands, Huawei, Hisilicon.”