3GPP TSG-RAN WG4 Meeting #114	R4-2500423

Athens, Greece, 17th – 21st February 2025

Source: 	Nokia

Title: 					n68 NS_26 and NS_36 A-MPR proposals

Agenda Item: 			6.17.1

Document for:	Approval

# 1	Introduction

We provide A-MPR proposals for band n68 and network signalling values NS_26 and NS_36. Our proposals are based on our simulation results [1] and agreements made in the previous meeting [2].

# 2	Simulation assumptions and scenario

Here we provide the simulation scenario and parameters that were used in the A-MPR simulations [1] leading to the proposals.

## 2.1	Simulation parameters

- Operating band n68 (UL 698 MHz – 728 MHz)

- Channel bandwidths: 5 MHz, 10 MHz and 15 MHz

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission masks from NS_26 and NS_36 are simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

- Extreme conditions duplex filter in Table 1 is used for both NS-values.

## 2.2	Duplex filter

According to the WF from RAN4#112-bis [3], we use the extreme conditions duplex filter parameters in Table 1. We use nearest-neighbor interpolation between the given frequencies.

Table 1: Duplex filter.

|  | Vendor B SAW 15MHz passband |  |
| --- | --- | --- |
| MHz | Extreme | Normal |
| 693.5 | 0.0 | 10.0 |
| 692.5 | 4.0 | 16.0 |
| 691.5 | 10.0 | 20.0 |
| 690.5 | 16.0 | 26.0 |
| 689.5 | 20.0 | 30.0 |
| 688.5 | 26.0 | 30.0 |
| 687.5 | 30.0 | 30.0 |
| 686.5 | 30.0 | 30.0 |

## 2.3	NS_26 mask

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |
| 686 ≤ f ≤ 694 | -25 | 8MHz |

## 2.4	NS_36 mask

According to the WF from RAN4#112-bis [3] we use the “normal conditions” limit.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz and 15 MHz |  |
| 470 ≤ f ≤ 694 | -42 | 8MHz |

# 3	A-MPR proposals

We presented simulation results for the previous meeting [1]. In this contribution we provide A-MPR proposals based on those results taking into account the agreements on the A-MPR regions made in the previous meeting [2]. To make the proposals applicable to both 15 kHz and 30 kHz SCSs we have converted the RB allocations to frequencies where needed.

## 3.1	NS_26

Proposal 1: RAN4 shall consider defining A-MPR for n68 PC3 NS_26 as shown in table 1.

Table 1: The proposed A-MPR regions for n68 PC3 NS_26.

| Channel bandwidth (MHz) | Rb_start*12*SCS MHz | L_crb | DFT-s-OFDM |  |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PI/2-BPSK | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| 5 | ≤ 0.18 | ≥ 1 |  |  |  |  |  |  |  |  |  |
| 10 | ≤ 2.16 | ≥ 1 | 2 | 3 | 3 | 3.5 |  | 4.5 | 4.5 | 4.5 |  |
| 15 | ≤ 4.5 | ≥ 1 | 2.5 | 3 | 3 | 3.5 |  | 4.5 | 4.5 | 4.5 |  |

## 3.2	NS_36

We have made small modifications in the A-MPR regions marked in yellow color, and they are explained below.

CBW=15 MHz, A1, RB_start column:

≥ changed to > to avoid overlapping regions.

CBW=15 MHz, A2, RB_start column:

≥ changed to > to avoid overlapping regions.

CBW=15 MHz, A2, L_crb column, second last condition:

Increased the constant term to avoid a gap in the regions.

CBW=15 MHz, A2, L_crb column, last condition:

Increased the constant term to cover the whole CIM3 region.

Proposal 2: RAN4 shall consider defining A-MPR for n68 PC3 NS_36 as shown in table 2.

Table 2: The proposed A-MPR regions for n68 PC3 NS_36.

| CBW |  | Rb_start* 12*SCS MHz | L_crb*12*SCS MHz | DFT-s-OFDM |  |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PI/2-BPSK | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| 5 MHz | A1 | ≤ 1.08 | > 0 | 7.5 | 8 | 8.5 | 8.5 | 8.5 | 9 | 9 | 9 | 9 |
|  | A2 | ≥ 1.08 and ≤ 2.7 | ≥ 1.8 | 4 | 4.5 | 4.5 | 4.5 |  | 5 | 5 | 5 |  |
| 10 MHz | A1 | ≤ 2.16 | > 0 | 12.5 | 13 | 13.5 | 13.5 | 13.5 | 15 | 15 | 15 | 15 |
|  |  | ≥ 2.34 and ≤ 3.78 | ≥ 12*SCS*RB_start + 1.62 |  |  |  |  |  |  |  |  |  |
|  | A2 | ≥ 2.34 and ≤ 4.32 | ≥ 0.5*12*SCS*RB_start + 0.63 and < 12*SCS*RB_start + 1.62 | 3.5 | 5 | 9 | 10 | 10.5 | 18 | 18 | 18 | 18 |
|  |  | > 4.32 | ≥ -12*SCS*RB_start + 7.2 |  |  |  |  |  |  |  |  |  |
| 15 MHz | A1 | ≤ 4.14 | > 0 | 12.5 | 13 | 13.5 | 13.5 | 13.5 | 15.5 | 15.5 | 15.5 | 15.5 |
|  |  | > 4.14 and ≤ 6.12 | ≥ 12*SCS*RB_start + 2.16 |  |  |  |  |  |  |  |  |  |
|  | A2 | > 4.14 and ≤ 7.2 | ≥ 0.5*12*SCS*RB_start + 0.63 and < 12*SCS*RB_start + 2.16 | 5 | 6 | 9 | 10 | 10 | 18 | 18 | 18 | 18 |
|  |  | > 7.2 | ≥ -12*SCS*RB_start + 11.0 |  |  |  |  |  |  |  |  |  |

# 4	Conclusions

We have made the following A-MPR proposals based on our simulation results.

Proposal 1: RAN4 shall consider defining A-MPR for n68 PC3 NS_26 as shown in table 1.

Table 1: The proposed A-MPR regions for n68 PC3 NS_26.

| Channel bandwidth (MHz) | Rb_start*12*SCS MHz | L_crb | DFT-s-OFDM |  |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PI/2-BPSK | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| 5 | ≤ 0.18 | ≥ 1 |  |  |  |  |  |  |  |  |  |
| 10 | ≤ 2.16 | ≥ 1 | 2 | 3 | 3 | 3.5 |  | 4.5 | 4.5 | 4.5 |  |
| 15 | ≤ 4.5 | ≥ 1 | 2.5 | 3 | 3 | 3.5 |  | 4.5 | 4.5 | 4.5 |  |

Proposal 2: RAN4 shall consider defining A-MPR for n68 PC3 NS_36 as shown in table 2.

Table 2: The proposed A-MPR regions for n68 PC3 NS_36.

| CBW |  | Rb_start* 12*SCS MHz | L_crb*12*SCS MHz | DFT-s-OFDM |  |  |  |  | CP-OFDM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PI/2-BPSK | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| 5 MHz | A1 | ≤ 1.08 | > 0 | 7.5 | 8 | 8.5 | 8.5 | 8.5 | 9 | 9 | 9 | 9 |
|  | A2 | ≥ 1.08 and ≤ 2.7 | ≥ 1.8 | 4 | 4.5 | 4.5 | 4.5 |  | 5 | 5 | 5 |  |
| 10 MHz | A1 | ≤ 2.16 | > 0 | 12.5 | 13 | 13.5 | 13.5 | 13.5 | 15 | 15 | 15 | 15 |
|  |  | ≥ 2.34 and ≤ 3.78 | ≥ 12*SCS*RB_start + 1.62 |  |  |  |  |  |  |  |  |  |
|  | A2 | ≥ 2.34 and ≤ 4.32 | ≥ 0.5*12*SCS*RB_start + 0.63 and < 12*SCS*RB_start + 1.62 | 3.5 | 5 | 9 | 10 | 10.5 | 18 | 18 | 18 | 18 |
|  |  | > 4.32 | ≥ -12*SCS*RB_start + 7.2 |  |  |  |  |  |  |  |  |  |
| 15 MHz | A1 | ≤ 4.14 | > 0 | 12.5 | 13 | 13.5 | 13.5 | 13.5 | 15.5 | 15.5 | 15.5 | 15.5 |
|  |  | > 4.14 and ≤ 6.12 | ≥ 12*SCS*RB_start + 2.16 |  |  |  |  |  |  |  |  |  |
|  | A2 | > 4.14 and ≤ 7.2 | ≥ 0.5*12*SCS*RB_start + 0.63 and < 12*SCS*RB_start + 2.16 | 5 | 6 | 9 | 10 | 10 | 18 | 18 | 18 | 18 |
|  |  | > 7.2 | ≥ -12*SCS*RB_start + 11.0 |  |  |  |  |  |  |  |  |  |

# 5	References

[1] R4-2419196, n68 NS_26 and NS_36 A-MPR simulations, Nokia

[2] R4-2420352, WF on A-MPR values for NS_36 and NS_26 for n68, Ericsson

[3] R4-2417080, WF on A-MPR for NR band n68, Ericsson

# 6	Appendix