3GPP TSG-RAN WG4 Meeting #114	R4-2500465

Athens, Greece, February 17 – 21, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for on-demand SSB SCell operation

Agenda item:	7.25.2.1

Document for:	Discussion

- Introduction

At RAN4#113 meeting, the R19 WI on network energy saving was further discussed and the agreements were captured in the WF [1]. In this contribution, we continue the discussion on the RRM impact from on-demand SSB SCell operation.

- General OD-SSB requirements

RAN1 LS

In last meeting, RAN4 sent reply LS to RAN1 confirming the timing from OD-SSB transmission perspective [2]. Meanwhile RAN4 will continue the discussion on the UE processing/preparation time for OD-SSB reception.

Issue 1-1-1: RAN1 LS Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

In our understanding, the UE processing time was raised because UE needs additional time to get prepared for receiving the first OD-SSB. It only applies to OD-SSB reception but not regular SSBs as OD-SSB may be activated at any unexpected time. But this also means that in some cases if the UE has been prepared when receiving OD-SSB activation command, the UE processing time may not be needed.

For Case 1, only OD-SSB may be transmitted in an SCell. The UE would likely turn off the receiver while OD-SSB is not activated. It may take time to prepare for the first OD-SSB reception after receiving activation indication. But if OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time.

For Case 2, always-on SSB are being transmitted in addition to OD-SSB in an SCell. The UE is assumed to keep monitoring always-on SSBs which may provide the reference for OD-SSB reception. For example, if OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

Observation 1: If OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time when it is activated again.

Observation 2: If OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

Hence, we would like RAN4 to discuss the applicability of the UE processing time. The processing/preparation time is needed only when necessary and it shall not apply at least in above cases.

- The processing/preparation time for OD-SSB reception is needed only when necessary and it shall not apply at least in the following cases:

- If OD-SSB has been activated recently, or

- If OD-SSB is transmitted on the same frequency with always-on SSB.

L1 measurement in OD-SSB SCell

In RAN1#118 meeting, it was further concluded to at least support L1 measurement based on OD-SSBs. It is not clear in which OD-SSB scenarios this would happen. In our understanding, UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement is not performed before SCell activation. We believe the same measurement assumption shall apply when OD-SSB is in operation.

Observation 3: A UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement is not performed before SCell activation.

With the RAN1 agreement, this implies the OD-SSB will be triggered after SCell activation procedure is completed hence leads to scenario #3B. However, the scenarios other than scenario #2 and #2A were not concluded explicitly in RAN1. We would like to wait for further RAN1 clarification on the applicable scenarios e.g. scenario #3B for L1 measurement before discussing the L1 measurement requirements based on OD-SSB in RAN4.

Agreements (RAN1#118) [Signalling] Agreement​ Update the previous RAN1 agreement as follows.​ - At least support L1 measurement based on on-demand SSB ​ - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.​ - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB​ - The support of LTM is a separate discussion point​

- Wait for further RAN1 clarification on the applicable scenarios i.e. scenario #3B for L1 measurement, before discussing the L1 measurement requirements based on OD-SSB in RAN4.

Accuracy of OD-SSB based measurement

While OD-SSB is introduced for network energy saving, this should not impact the measurement performance. The legacy measurement accuracy for SSB-based measurement shall still apply to OD-SSB based measurement.

- The legacy measurement accuracy for SSB-based measurement shall apply to OD-SSB based measurement.

OD-SSB impact on neighbor cell measurement

In RAN4#112bis meeting, it was concluded to reuse the legacy measurement framework irrespective of OD-SSB, but there were different understandings if existing measurement requirement can still apply.

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neighboring cell, the legacy measurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. - This agreement can be revisited if it is not aligned with further agreement in other WGs.

In our view, we can see two cases:

- OD-SSB is configured in a neighbor cell on a serving CC, and SCell is legacy cell.

In this case, UE is required to perform measurements on the serving CC i.e. the servingCellMO including all the cells. As UE does not know if the OD-SSB is triggered in neighbor cell, it just measures the MO based on SMTC and may or may not be able to detect the OD-SSB neighbor cell. But if OD-SSB is activated in the neighbor cell, the existing measurement requirement shall still apply to the OD-SSB neighbor cell.

- OD-SSB cell is configured in an SCell, and neighbour cells are legacy SCells

In this case, it was agreed a UE is not required to measure OD-SSB before OD-SSB is activated. Hence the measurement requirement for deactivated SCell (being discussed in section 3 and 4) is applicable to OD-SSB SCell only if OD-SSB is activated. But this shall not impact the measurement on the legacy neighbor cells in the servingCellMO. The UE is still required to perform the intra-frequency measurement on the serving MO and existing measurement requirement for neighbor cells on the MO still apply.

- If OD-SSB is configured in a neighbor cell on a serving CC with legacy SCell, the existing measurement requirement based on SMTC shall apply to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- If there is an OD-SSB SCell on the serving CC, the existing measurement requirement for neighbor cells still apply to the legacy neighbor cells on the CC.

OD-SSB type

While RAN4 agreed to take NCD-SSB as a starting point for OD-SSB, there seem to be different understanding what NCD-SSB is. We understood NCD-SSB means it is not associated with SIB1 and generally there is no limitation on the frequency location. That is, it may or may not be located on synchronization raster as shown in Fig.1.

Fig. 1 OD-SSB SCell operation in and off sync raster (taking case 1 as example)

Observation 4: OD-SSB may or may not be located on a synchronization raster.

As in Case 1, there is only OD-SSB i.e. no always-on CD-SSB available in the cell for measurement, the UE is required to measure OD-SSB once OD-SSB transmission is triggered. This requirement is independent of the OD-SSB location on synchronization raster or not. We don’t see any impact on RRM requirements in Case 1 because of OD-SSB being located on sync raster or not.

For Case 2, RAN1 is discussing different alternatives of the frequency location of OD-SSB pending on if always-on SSB is on a synchronization raster where always-on SSB is CD-SSB. This may lead to different relation between always-on SSB and OD-SSB pending on the presence of synchronization raster. We should wait for RAN1 to further clarify the feasible scenarios before identifying the impact to RRM requirements due to location on synchronization raster.

- For Case 1, there is no impact on RRM requirements because of OD-SSB being located on the sync raster or not.

- For Case 2, wait for RAN1 to further clarify the feasible scenarios before identifying the impact to RRM requirements due to location on synchronization raster.

- OD-SSB based deactivated SCell measurement (Case 1)

In last meeting, the UE measurement behavior on the deactivated OD-SSB SCell was further clarified. In general, a UE is expected to perform a fast measurement based on OD-SSB periodicity within a fast mode window when it is activated. Afterwards, UE can fall back to legacy deactivated SCell measurement while OD-SSB is being activated. We will continue discussing the details based on the latest agreements.

Deactivated SCell measurement within the fast mode window

For the measurement within the fast mode window, some company proposed to define a lower bound or to apply CSSF for the measurement period due to UE implementation restriction.

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement - Option 1: The same CSSF as legacy - Option 2: - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements. - RRM requirements on PSCC and other SCCs are not applicable during the time-window. - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed. Other option is not precluded. Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

With the lower bound, the UE may relax the measurement on the deactivated SCell. For instance, the OD-SSB is being transmitted with a periodicity of 5ms while the UE is measuring OD-SSB every 20ms assuming a low bound of 100ms. From network point of view, it is not energy efficient to transmit OD-SSB that often if they are not measured by the UE. Besides, UE is assumed be able to process SSB within 2ms during SCell activation, we wonder why additional lower bound is needed for processing OD-SSB based measurement. The lower bound needs to be well justified as it may reduce the network energy saving gain from OD-SSB transmission.

- The lower bound needs to be well justified as it may reduce the network energy saving gain from OD-SSB transmission.

Due to the same concern, UE vendors would like to apply CSSF as an alternative if lower bound is not to be introduced. This can also relax the UE measurement when multiple carriers are to be measured. In our view, the measurement in fast mode is to prioritize the OD-SSB based measurement where measCycleSCell and DRX cycle are not considered. Hence, this fast measurement shall also be prioritized over the measurements on legacy carriers. The CSSF for the measurement within fast mode window shall not count the number of legacy carriers configured with measurement. But if there are multiple OD-SSB SCells to be measured within the fast mode window, the number of OD-SSB SCells needs to be counted when deriving CSSF. To account the workload of the discussion and finalize the work item, we suggest focusing on single OD-SSB SCell in this release, hence no CSSF applies.

Observation 5: The CSSF for the measurement within fast mode window shall not count the number of legacy carriers configured with measurement.

- For the measurement within fast mode window, the CSSF does not  apply assuming there is one OD-SSB SCell.

Regarding to the time window for fast measurement, the following two conditions were agreed to determine the time window for fast mode measurement.

Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed

The number of OD-SSB samples provides a maximum time period which ensures the UE is able to acquire a qualified measurement result based on OD-SSB. As the number of OD-SSB samples may be scaled by CSSF, we think it can be captured in the time to identify the OD-SSB SCell e.g. Tidentify_intra_without_index or Tidentify_intra_with_index can be used as a reference by replacing SMTC with OD-SSB periodicity.

The other condition is a valid measurement reporting for the OD-SSB SCell. With this explicit indication, network may determine to stop or slow down the transmission of OD-SSB. In case the UE sends measurement reporting before the end of the cell identification time, this would provide more benefit on network energy saving. Hence, both conditions can be considered and the time window is based on the smaller value between the cell identification time and the time to send the first valid measurement reporting for the OD-SSB SCell.

- The number of OD-SSB samples is captured in the time to identify the OD-SSB SCell e.g. Tidentify_intra_without_index or Tidentify_intra_with_index can be used as a reference by replacing SMTC to OD-SSB periodicity.

- The time window for fast measurement is based on the smaller value between the cell identification time and the time to send the first valid measurement reporting for the OD-SSB SCell.

Deactivated SCell measurement after the fast mode window

After the fast mode window, it was agreed the legacy deactivated SCell measurement requirement shall apply if OD-SSB is activated.

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)? Sub-issue 5: What’s the UE behavior when OD-SSB further activation/reconfiguration after T2? FFS.

Another issue raised was the minimum interval T2 which is the time period between OD-SSB is deactivated and OD-SSB is again activated i.e. stage 4 in the figure. In our view, the activation and deactivation of OD-SSB is fully up to network decision. We should not specify any requirements restricting the network configuration. The minimum internal T2 shall not be discussed in RAN4.

- Do not discuss the minimum interval T2 between OD-SSB is deactivated and OD-SSB is again activated in RAN4.

What may be discussed is the impact on the measurement requirement from the internal T2. In particular, if the UE has acquired a measurement result based on OD-SSB which was previously activated, the time window for fast measurement can be shorter when OD-SSB is activated again if the cell remained detected which depends on the length of T2. We would see this as an optimization to the fast mode window, but this would complicate the OD-SSB based measurement requirement. RAN4 can discuss if such optimization is needed for OD-SSB based deactivated SCell measurement.

Observation 6: The time window for fast measurement can be shorter when OD-SSB is activated again, if the cell remained detected which depends on the length of T2.

- RAN4 to discuss if the optimization to the fast mode time window based on T2 is needed.

- OD-SSB based deactivated SCell measurement (Case 2)

Relation between Always-on SSB and OD-SSB

In Case 2, it is assumed there is some regular or always-on SSBs in the SCell and OD-SSB may be triggered in addition. At RAN4#112bis meeting, an LS was sent to RAN1 asking about the feasible scenarios in terms of the relation between always-on SSB and OD-SSB. In the Reply LS [2], RAN1 provides initial answers to RAN4 questions as cited below.

Obj 1 Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Fig. 2 OD-SSB SCell operation in Case 2 and Scenario #2/#2A

Regarding to the time location, RAN1 indicated OD-SSB periodicity is likely to be equal or smaller than that of always-on SSB. But they didn’t answer to the time offset between OD-SSB and always-on SSB. Based on offline discussion, the UE may not be able to receive SSBs if the intervals between consecutive SSBs are varying from time to time. Hence it would be more energy efficient to assume the same time offset between them as exampled in Fig.2 Case 2-1.

Besides, we understood RAN1 is discussing the case where OD-SSB and always-on SSB have the same periodicity with a time shift of a half periodicity i.e. in Fig.2 Case 2-2. In this case, the combination of two types of SSB achieves half of the SSB periodicity which can reduce the measurement delay.

There is one more interesting Case 2-3 considering different frequency location between OD-SSB and always-on SSB. Since this was not confirmed by RAN1, we can prioritize the Case 2-1 and Case 2-2 in RAN4 discussion before further information is provided by RAN1.

- RAN4 to prioritize the discussion in Case 2-1 and Case 2-2 before any further information is received from RAN1.

- Case 2-1: different periodicity with no time offset

- Case 2-2: same periodicity with a time offset of half periodicity

Deactivated SCell measurement

When OD-SSB is triggered on a deactivated SCell, it needs to be discussed how a UE would perform the measurements and how to define the measurement requirement while both OD-SSB and regular SSBs are present. The following options were proposed in last meeting.

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered - Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 1-1: The measurement periodicity is OD-SSB periodicity - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB - Option 2: UE always follow legacy requirements based on measCycleSCell - Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

The deactivated SCell measurement in Case 2 is different from Case 1 in that there is always-on SSB before OD-SSB is activated. Having the prioritized Case 2-1/Case 2-2 in mind, as OD-SSB and always-on SSB are on the same frequency, the UE may have acquired a measurement result based on always-on SSB when OD-SSB is activated. In this case, the UE may skip the measurement on OD-SSB and continue with the legacy measurement based on always-on SSB. This can be formulated as the conditions that the UE has been measuring always-on SSB for a certain time period, or the UE has sent a measurement reporting for the OD-SSB SCell.

Otherwise, it shall follow the measurement procedure as discussed for Case 1, i.e. start the fast measurement within a fast mode window and fall back to legacy measurement on deactivated SCell. But the deactivated SCell measurement period after the fast mode window is based on always-on SSB instead of OD-SSB so that network may stop or deactivate the OD-SSB transmission for energy saving.

Observation 7: If OD-SSB and always-on SSB are on the same frequency, the UE may have acquired a measurement result based on always-on SSB when OD-SSB is activated.

- UE may skip the measurement on OD-SSB if the UE has been measuring always-on SSB for a certain time period, or the UE has sent a measurement reporting for the OD-SSB SCell. Otherwise, the UE follows the measurement procedure as discussed for Case 1, i.e. start the fast measurement within a fast mode window and fall back to legacy measurement on deactivated SCell.

- For the deactivated SCell measurement after the fast mode window in Case 2, the measurement period is based on always-on SSB instead of OD-SSB.

If the OD-SSB is on a different frequency from always-on SSB e.g. Case 2-3 in Fig.2, from existing configuration point of view they would reside in different measurement object (MO)s as no MO can have multiple frequencies of SSB associated. This seems to be easiest approach in both RAN2 and RAN4. So we think the measurements and reporting on OD-SSB and always-on SSB are up to network configuration and can follow legacy behavior. As Case 2-3 is still under discussion in RAN1, we can discuss the details when this is considered as feasible from RAN1 perspective.

- For the case where OD-SSB is on a different frequency from always-on SSB e.g. Case 2-3, RAN4 to wait for the RAN1 conclusion on the feasibility before discussing the measurement requirement.

- OD-SSB based SCell activation delay

Known/unknown condition

On SCell activation delay, the following was agreed on the known/unknown condition for OD-SSB SCell.

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

One open issue about known/unknown condition is the time period during which the UE has sent a measurement reporting before receiving SCell activation command. As we agreed to follow legacy deactivated SCell measurement period after the fast mode window, the validity of the measurement reporting can still refer to the legacy time period e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell, irrespective if the measurement reporting is based on OD-SSB or always-on SSB. This can minimize the impact on known condition and keep the SCell in known state.

- In the known condition, the time period during which the UE has sent a measurement reporting before receiving SCell activation command is the same as that in legacy SCell activation e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell.

Another issue being raised is to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. We understood this problem is valid only if OD-SSB is on a different frequency from always-on SSB. As discussed before, if we prioritize the case with the same frequency, it does not matter if the known condition is determined based on always-on SSB and/or OD-SSB and the SCell shall be activated as a known SCell.

- If the OD-SSB is on the same frequency with always-on SSB, the SCell shall be activated as a known SCell no matter if the known condition is determined based on always-on SSB and/or OD-SSB.

- Conclusion

In this contribution, we discussed the RRM requirements for OD-SSB SCell operation based on RAN1/2 progress and presented our views on the potential RAN4 impact. The following observations and proposals are made:

Observation 1: If OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time when it is activated again.

Observation 2: If OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

- The processing/preparation time for OD-SSB reception is needed only when necessary and it shall not apply at least in the following cases:

- If OD-SSB has been activated recently, or

- If OD-SSB is transmitted on the same frequency with always-on SSB.

Observation 3: A UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement is not performed before SCell activation.

- Wait for further RAN1 clarification on the applicable scenarios i.e. scenario #3B for L1 measurement, before discussing the L1 measurement requirements based on OD-SSB in RAN4.

- The legacy measurement accuracy for SSB-based measurement shall apply to OD-SSB based measurement.

- If OD-SSB is configured in a neighbor cell on a serving CC with legacy SCell, the existing measurement requirement based on SMTC shall apply to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- If there is an OD-SSB SCell on the serving CC, the existing measurement requirement for neighbor cells still apply to the legacy neighbor cells on the CC.

Observation 4: OD-SSB may or may not be located on a synchronization raster.

- For Case 1, there is no impact on RRM requirements because of OD-SSB being located on the sync raster or not.

- For Case 2, wait for RAN1 to further clarify the feasible scenarios before identifying the impact to RRM requirements due to location on synchronization raster.

- The lower bound needs to be well justified as it may reduce the network energy saving gain from OD-SSB transmission.

Observation 5: The CSSF for the measurement within fast mode window shall not count the number of legacy carriers configured with measurement.

- For the measurement within fast mode window, the CSSF does not  apply assuming there is one OD-SSB SCell.

- The number of OD-SSB samples is captured in the time to identify the OD-SSB SCell e.g. Tidentify_intra_without_index or Tidentify_intra_with_index can be used as a reference by replacing SMTC to OD-SSB periodicity.

- The time window for fast measurement is based on the smaller value between the cell identification time and the time to send the first valid measurement reporting for the OD-SSB SCell.

- Do not discuss the minimum interval T2 between OD-SSB is deactivated and OD-SSB is again activated in RAN4.

Observation 6: The time window for fast measurement can be shorter when OD-SSB is activated again, if the cell remained detected which depends on the length of T2.

- RAN4 to discuss if the optimization to the fast mode time window based on T2 is needed.

- RAN4 to prioritize the discussion in Case 2-1 and Case 2-2 before any further information is received from RAN1.

- Case 2-1: different periodicity with no time offset

- Case 2-2: same periodicity with a time offset of half periodicity

Observation 7: If OD-SSB and always-on SSB are on the same frequency, the UE may have acquired a measurement result based on always-on SSB when OD-SSB is activated.

- UE may skip the measurement on OD-SSB if the UE has been measuring always-on SSB for a certain time period, or the UE has sent a measurement reporting for the OD-SSB SCell. Otherwise, the UE follows the measurement procedure as discussed for Case 1, i.e. start the fast measurement within a fast mode window and fall back to legacy measurement on deactivated SCell.

- For the deactivated SCell measurement after the fast mode window in Case 2, the measurement period is based on always-on SSB instead of OD-SSB.

- For the case where OD-SSB is on a different frequency from always-on SSB e.g. Case 2-3, RAN4 to wait for the RAN1 conclusion on the feasibility before discussing the measurement requirement.

- In the known condition, the time period during which the UE has sent a measurement reporting before receiving SCell activation command is the same as that in legacy SCell activation e.g. max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell.

- If the OD-SSB is on the same frequency with always-on SSB, the SCell shall be activated as a known SCell no matter if the known condition is determined based on always-on SSB and/or OD-SSB.

- References

[1]	R4-2420098, “WF on Netw_Energy_NR_enh”, Ericsson

[2]	R1-2500020, Reply LS on timeline for On-demand SSB operation on SCell, RAN4, RAN1#120, Athens, Greece, February 17th – 21st, 2025