3GPP TSG-RAN WG4 Meeting #114	R4-2500470

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for intra-band non-collocated scenarios Phase2

Agenda item:	7.5.3

Document for:	Discussion

Introduction

At RAN4#113 meeting, the CR work split was agreed regarding to the impact on RRM requirements in non-collocated intra-band scenarios for Type 4 UEs [1]. However, the BS signalling and initial operation are still under discussion in RF, and the R19 first specification is not available. To facilitate the coming discussion in RRM, we bring the text proposal for MRTD/MTTD clauses in NR-CA scenarios based on our understanding.

Text proposal

### <Start of Change 1>

### 7.5.4	Minimum requirements for NR Carrier Aggregation

The UE shall be capable of handling at least a relative transmission timing difference between slot timing of all pairs of TAGs in FR1 and/or FR2-1 as shown in table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode, or

-	configured with more than one sTAG for inter-band NR carrier aggregation in EN-DC or NE-DC mode.

The UE shall be capable of handling at least a relative transmission timing difference between subframe timing of all pairs of TAGs between FR1 and FR2-2 as shown in table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference as shown in table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 or [intraBandNR-CA-non-collocatedType4-r19]and neither nonCollocatedTypeNR-CA-r18 nor [nonCollocatedTypeNR-CAType4-r19] is provided.

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2-1 | 26.1 |
| Between FR1 and FR2-2 | 26.1 |
| NOTE1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

### <End of Change 1>

### <Start of Change 2>

### 7.6.4	Minimum requirements for NR Carrier Aggregation

For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of intraBandNR-CA-non-collocated-r18 or [intraBandNR-CA-non-collocatedType4-r19] shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1 below.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference as shown in table 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 or [intraBandNR-CA-non-collocatedType4-r19] and neither nonCollocatedTypeNR-CA-r18 nor [nonCollocatedTypeNR-CAType4-r19] is  provided. Otherwise, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1 below if UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, or if UE indicates that it is capable of [intraBandNR-CA-non-collocatedType4-r19] and [nonCollocatedTypeNR-CAType4-r19] is provided.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| NOTE 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

For inter-band NR carrier aggregation,

-	the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers in FR1 and FR2-1 to be aggregated at the UE receiver as shown in table 7.6.4-2 below.

-	the UE shall be capable of handling at least a relative receive timing difference between subframe timing of all pairs of carriers in FR1 and FR2-2 to be aggregated at the UE receiver as shown in table 7.6.4-2 below.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 note1 |
| Between FR1 and FR2-1 | 25 |
| Between FR1 and FR2-2 | 25 |
| NOTE1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

### <End of Change 1>

Proposal 1: To wait for RF conclusion on BS signaling and initial operation type for Type 4 UE.

Proposal 2: To discuss the text proposal on MRTD/MTTD.

Conclusion

In this paper we have made the text proposals for the intra-band non-collocated scenarios for Type 4 capable UEs:

Proposal 1: To wait for RF conclusion on BS signaling and initial operation type for Type 4 UE.

Proposal 2: To discuss the text proposal on MRTD/MTTD.

References

[1] R4-2420090, WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2, Huawei, RAN4#113