3GPP TSG-RAN4 Meeting #114	R4-2500490

# Athens, Greece,  16th – 21st February, 2025

Title:	Ambient IoT device requirements

Source:	Qualcomm Incorporated

Agenda item:	7.24.2.2

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

WI for Ambient IoT in Rel-19 was approved in RAN#106.  The scope is topology 1 and device 1. In this paper we discuss some requirements for the the Device and how to define them. Especially and for example reference sensitivity for the backscattering type devices may not be that straightforward since that device will need two signals to operate. In addition the A IoT Device will need energizing signal to function.

# 2. 	Discussion

WF from last meeting [2] outlines the following RF requirements needed for the device 1 TX.

Transmit output power: NO for maximum output power, YES for backscattering power or power backscattering loss

Modulation quality requirements: TBD

Output RF spectrum emissions: Occupied bandwidth and SEM and Spurious Emissions as TBD

Unwanted emissions: TBD

And for RX:

Reference sensitivity

Maximum input power: TBD

Interference rejection: TBD

Despite the agreements what to specify and what not, there was not a lot of discussion how to specify the above mentioned items and in the following we intend to clarify definitions and propose how they can be specified.

## 2.1 	Transmit output power considerations

For example, it is not clear what does it mean exactly that for “Transmit output power: NO for maximum output power, YES for backscattering power or power backscattering loss”. It seems the intention is to specify backscattering loss for the device but gain or loss alone is difficult to specify so that it can be tested. Gain is the ratio of two powers, or voltages, or in other words, there is an expectation of returned power compared to the transmitted power.

Observation 1: Specifying testable backscattering loss without specifying either the input or the output power is difficult.

It may be beneficial to combine the maximum output power with proper input signal requirement.

Proposal 1: Define a transmit output power requirement so that for specific input power, a predefined output power is expected from the device.

If Proposal 1 will define the output power and back scattering loss requirement for the Ambient IoT Device. But in order to detail this requirement, we will need to understand better the definition of input signal. Only when we understand what is the requirement, we can agree the numerical values for the requirement.

Observation 2: Good definition of the requirement is needed before companies can agree the values for the requirement

## 2.2	Maximum input level

Agreement was that also maximum input power is specified as TBD. Maximum input level in LTE and NR is the maximum input level of any signal to the UE RX input port. This would be a good way forward also for A IoT Device.

Proposal 2: Define maximum input power for A-IoT Device to apply to all input signals

Further discussion can be had if the maximum input level is also applicable for the sum power of all the signals.

## 2.3	Reference sensitivity

Similarly to the transmit output power, the input side will need some more detailed definitions.  Device will need three input signals for it to operate:

- Energizing signal

- CW for backscattering

- R2D signal that triggers device to respond

We can assume from the WF [2] that the backscattering loss is the ratio of CW power to the D2R signal power and the Reference Sensitivity could be referred to that but the Device will not respond without successful reception of PRDCH signal. The Reference sensitivity can be defined in two ways:

- Define Reference Sensitivity power level as the input power of the CW signal and PRDCH level as side condition

- Define Reference Sensitivity power level as the input power of the PRDCH and CW signal level as side condition

The levels should be chosen in such way that the side condition does not limit the performance of the main parameter.

Observation 3: The metric for the reference sensitivity power level should be the one most critical from the system performance point of view

Critical parameter can be R2D signal power or availability of the CW signal power, or maybe even availability of the energy, needs to be decided in RAN4. It is also possible for RAN4 to define more than one reference sensitivity like requirement where each parameter is tested when the others are not limiting the performance. For example, one requirement is that CW level is well above sufficient and the D2R signal level is set to the lowest level when device still responds. The other is when R2D signal level is well above sufficient but the CW level is set to the lowest level when device responds acceptably. The graphical representation of signal conditions for this requirement is as in Figure 1:

Figure 1. Two Reference sensitivity test conditions

The maximum input level need to be respected in the Reference Sensitivity test conditions so care needs to be taken to specify test conditions for all three signals.

## 2.4	Modulation quality

Transmit Modulation Quality in RAN4 contains EVM, carrier leakage and in-band emissions. Transmit Modulation quality belongs in to a clause of Transmit Signal Quality. What the agreement means exactly may need some more discussion since e.g carrier leakage maybe difficult to specify for the Device.  EVM is possible to specify but the actual value may need to be discussed since the D2R signal is using modulation that is new to 3GPP and also the channel conditions are different than for handheld mobile and EVM is defined based on 5% capacity impact to system performance.

Observation 4: Definition EVM for backscattering device with OOK modulation may need further discussion

Since the Device modulates incoming CW, any impurity in the CW itself should not be counted as the EVM error of the device so one way to define the requirement can be adopted from Frequency error where the reference is the incoming signal.

Observation 5: EVM of D2R signal is dependent on the quality of the CW

Whether to define EVM or not, our view is that Device’s ability to modulate properly would need to be specified.

Proposal 3: Define EVM for the Ambient IoT device

On the other requirements, level of SFO is critical for RAN1 design [3] and it can be considered as frequency error equivalent. RAN1 is discussing if SFO calibration procedure is defined to reduce SFO to 1 % or if the system is built to withstand 10 % in all conditions. RAN4 should specify SFO requirement in both cases.

Proposal 4: As part of modulation quality RAN4 will specify SFO error requirement for the A-IoT Device.

For further details RAN4 needs to wait for RAN1 conclusion if a calibration routine is included in the system design but RAN4 can discuss how SFO can be verified and how to write a requirement for it. One possibility is to check the spectrum of the device. The offset Δf, or otherwise knows as small frequency offset, from CW defines the location of D2R signal. RAN1 is discussing up to 16Δf offsets and error up to 10% is fairly easy to detect with good margin. If all devices will support 16Δf offset or only some is yet to be determined by RAN1.

Figure 2. D2R Spectrum and location

An other possibility is to observe the sampling itself in BB.

## 2.5	Selectivity

As discussion in our companion paper [4], there is degraded performance if NR UE moves to indoor where A-IoT is deployed. There are two problematic mechanisms where device requirements could be impacted.

First one is interference from NR UL to R2D signal. This was discussed in the co-ex study that resulted in to the conclusions in [4]. Total interference is combination of blocking and emissions falling to inband. This mechanism can be reduced by the Device selectivity in terms of rejection of aggressor signal.

An other mechanism what was not discussed is also related selectivity but in this case the Device may start modulating the NR UL signal instead of the CW from the A IoT deployment. It could be called the backscattering selectivity.

Example band choses in n8 and the n8 UL is only 10 MHz away from the DL. It is unlikely that the A IoT device is so selective that it would work with 925 MHz CW but reject 935 MHz signal. In Figure 3 we outline the principle, in order for the CW resonator to attenuate n8 UL by 3 dB it needs to have a Q of 16.

Figure 3. CW selectivity scenario and 3 dB BW of the backscattering resonator

What this NR UL aggressor causes in the A IOT Device is unknown. One possibility is that it will it modulate it as two tone signal creating wide spectra of D2R signals and what reader then does when this signal enters the receiver is unknown.

Observation 6: Does RAN4 need to specify a selectivity requirement for A IoT Device need to be discussed.

## 2.6 	OTA aspect consideration

As agreed in [2] the reference plane is OTA. If the Device antenna has spatial characteristics or not is not clear at this time. The numerical values for the above discussed parameters should be defined with an antenna design as part of the Device. It means, the values are defined as equivalent electrical field strengths and can be converted to power with a known reference antenna, such as 0 dBi and unit can be power.

Proposal 5:   Define reference values as dBm and referred to 0 dBi antenna.

How the antenna directionality will be taken in to account in the requirement setting is not clear. One possibility is to assume antennas are omnidirectional and ignore this aspect or it also means that the antenna aspects then are part of the implementation and in testability the Device orientation can be random.

Observation 7: How to accommodate imperfections in the Device antenna design in RF requirements need to be discussed.

We have provided a companion paper to further discuss OTA testability aspects [5].

# Conclusion

We discussed various device requirements and made the following 7 observations

Observation 1: Specifying testable backscattering loss without specifying either the input or the output power is difficult.

Observation 2: Good definition of the requirement is needed before companies can agree the values for the requirement

Observation 3: The metric for the reference sensitivity power level should be the one most critical from the system performance point of view

Observation 4: Definition EVM for backscattering device with OOK modulation may need further discussion

Observation 5: EVM of D2R signal is dependent on the quality of the CW

Observation 6: Does RAN4 need to specify a selectivity requirement for A IoT Device need to be discussed.

Observation 7: How to accommodate imperfections in the Device antenna design in RF requirements need to be discussed.

And we made five proposals

Proposal 1: Define a transmit output power requirement so that for specific input power, a predefined output power is expected from the device.

Proposal 2: Define maximum input power for A-IoT Device to apply to all input signals

Proposal 3: Define EVM for the Ambient IoT device

Proposal 4: As part of modulation quality RAN4 will specify SFO error requirement for the A-IoT Device.

Proposal 5:  Define reference values as dBm and referred to 0 dBi antenna.

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

[4]	R4-2500402, “Discussion on Ambient IoT deployment scenario in Rel-19”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2500401, “Discussion on Ambient IoT Testability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025