3GPP TSG-RAN4 Meeting #114	R4-2500492

# Athens, Greece,  16th – 21st February, 2025

Title:	Initial aspects of RX Switching

Source:	Qualcomm Incorporated

Agenda item:	7.8.2

Release:	Rel-19

WI: 	NR_LBCA_Sw-Core

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

RAN#106 approved a new WI for low band carrier aggregation via switching. The intention of this WI is to enable carrier aggregation for low bands that can not be aggregated due to channel conditions with bands from other band groups and which have challenges in RF domain such as very large relative bandwidth such as the example band combination n5 + n29 has.

# 2. 	Discussion

## 2.1	Motivation and benefit analysis

The example band configuration has relative bandwidth of about 22%. This poses RF design some challenges resulting in to lower radiation efficiency of the antenna than if each band would be in use separately.  The framework in the WI introduces a switching pattern to the DL CA with the two possible states of the radio, labelled as “cases” as below in Figure 1.

Figure 1. States of radio for low band CA switching for n5 + n29. Example of Type in WI, referred here as Type 1

FDD system currently requires UE to monitor PDCCH on every slot. RAN1 changes are needed to define a framework to when UE monitors PDCCH on which band and how much where semi static UL grants such as HARQ feedbacks are postponed and placed. How far network wants to postpone these and how much to reduce latency is discussed in RAN1 and for now, RAN4 should wait for RAN1 to conclude their work on this. If the dwell time in each case is very long, the switching time may not be critical for the feature.

Having an outage for FDD band for some period of time means for that UE, the overall capacity is less than without the outage. If the UE is not receiving ½ of the DL slots, it means loss of ½ capacity or 3 dB.

Observation 1: RX switching feature with loss of every other slot for FDD band means 3 dB loss of capacity and latency.

However, the antenna radiation loss has not been discussed in RAN4. If it exceeds 3 dB with the 22% relative bandwidth compared to the tuning with individual bands, then this feature has positive gains.

Observation 2: Benefit analysis has not been concluded in 3GPP for this feature.

## 2.2	Side conditions

The WI outlines that this solution is targeted to the rural areas and the obvious solution aggregating it with other bands that use different antennas is not available since other bands are out of coverage. This means that for UE, these would be only bands to be aggregated with this feature and if RAN develops this feature with this assumption, it should be clearly included in the technical specifications. It also means that the FDD band is always PCell here.

Observation 3: For this solution, we can assume no other bands are aggregated in the same UE simultaneously and the FDD band is always configured as PCell.

This should be clearly stated as one of the assumptions for this work and also preferably communicated to RAN1. This side conditions was present in the motivating discussions earlier but it is not written down anywhere and our view is that it should be clearly agreed as a side condition for this feature since it has an impact on the implementation choices and therefore the switching time.

Proposal 1: CA band combination supporting this feature will not be combined with other bands to form higher order CA combinations

Combining this feature with more bands to form a higher order band combinations and configuring a third band as PCell will need re-evaluation of switching time and pattern design.

## 2.3	Switching time proposal

Any UE supporting inter-band carrier aggregation, contains at least two independent RX chains. UE supporting inter-band CA has the ability to receive both bands simultaneously.

Observation 2: Any current implementation supporting CA has the ability to receive two bands simultaneously.

For this feature, there are many ways to implement the receiver. Simplest way is for UE to keep receiver on for both bands and ignore the slots when the antenna tuning is not favouring the band.

UE can also reuse more of its receiver chain to optimise current consumption or enable this CA with a single receiver, but this may come with the penalty of prolonged switching time since then also LO needs to be re-tuned between the bands.

Figure 2. Impact of level of shared HW resources between the bands

What is the level of shared HW between the designs should be discussed. It seems from the motivation papers that the antenna is shared and then the switching time is antenna tuner switching time. Antenna tuning is a switch and switching times are relative fast, in the order of few 10’s of usec.

Proposal 2: Define RX switching time as a capability allowing also 0 usec and re-use TX switching feature switching period values 35, 140 and 210 usec.

For TX side, 3GPP RAN has developed a feature to switch TX between bands following scheduling commands. In this case, the TX chain does not need to be switched but only network needs to be aware that the UE is not able to transmit on certain slots. There is no need to define a switching period for TX since it can be aligned with the RX availability and RAN4 can assume than for which slots and symbols UE is receiving when in Case 1, also the TX for FDD band is able to transmit.

Only problem may come if RF filter design is such that the isolation from TX to SDL RX is not sufficient and any overlap between SDL RX and FDD TX degrades the performance. As these filters today for this band combo are design to meet simultaneous RX and TX between the FDD and SDL, there is no such problem but this feature may result in to relaxed requirements for these filters. This would happen if the switching time is negative, i.e. TX transmits before the RX has stopped receiving. Any finite switching time will mitigate this issue.

Observation 3: When RX for case 1 is available for reception, also TX is available without additional switching periods with regular transient periods but no sooner than that.

## 2.4	Other type combinations

WI outlines that other band combination can be added by means of basket WI. Some of the band combination in the motivating discussion [2] may need a different kind of framework both from RAN4 perspective and from RAN1 perspective.

An other kind of combination is where there are possibilities to combined RX filter. This kind of RF solution would simplify RAN1 design and it would have much less negative impact to the system and UE. For example band n12+n29 opens a possibility for a combined RX filter and the case description would change as in Figure 2 (a).

Figure 3. n12 + n29 for RX switching with advanced filter. Example of Type 2 configurations

This type of band combination can be operated with the same pattern as the type 1 (Figure 1, example combination in the WID [1])) but with rather great penalty to performance and optimised use would require a definition of case 3 as in Figure 3.

Observation 4: Type 2 combination would benefit from dedicated pattern design greatly

In addition, there were discussions on two FDD bands, such as n26+n106. If the pattern is defined based on FDD+SDL band, the Scell UL will be left unused or then we need other case description(s) and options are presented in Figure 3. Case 2 in Figure 3 is the reused Case from Type 1 combination. Reusing pattern design that was made for the Example band combination (Figure 1) will mean that the UL of the Scell in this combination would never be used.

Figure 4. Type 3 combination n26+n106 with possible new cases.

Observation 5: Reusing pattern design from Type 1 combination for Type 3 will mean 100 % loss of the Scell UL.

In conclusion, the process of adding new band combination under this feature under basket WI will result in to sub-optimal feature for some band combinations that different type than the Type 1 that is the example combination in the WID.

# Conclusion

We discussed the new RX switching WI and aspects and made the following five observations:

Observation 1: RX switching feature with loss of every other slot for FDD band means 3 dB loss of capacity and latency.

Observation 2: Benefit analysis has not been concluded in 3GPP for this feature.

Observation 3: When RX for case 1 is available for reception, also TX is available without additional switching periods with regular transient periods but no sooner than that.

Observation 4: Type 2 combination would benefit from dedicated pattern design greatly

Observation 5: Reusing pattern design from Type 1 combination for Type 3 will mean 100 % loss of the Scell UL.

And two proposals:

Proposal 1: CA band combination supporting this feature will not be combined with other bands to form higher order CA combinations

Proposal 2: Define RX switching time as a capability allowing also 0 usec and re-use TX switching feature switching period values 35, 140 and 210 usec .

# Reference

[1]	RP-243317, “New WID on low band carrier aggregation via switching”, Apple, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024