3GPP TSG-RAN WG4 Meeting #114	R4-2500533

Athens, Greece, 17th – 21st February 2025

Agenda item:			7.21.4

Source:	Moderator (Nokia)

Title:	Topic summary for [114][219] FS_NR_AIML_Mob

Document for:	Information

# Introduction

This summary thread addresses the issues related to AI/ML mobility SI in RAN4. The document is split into 3 high-level topics which are further split into sub-topics and issues.

Moderator’s recommendations are to discuss following issue, listed in the order of priority.

- Issue 1-1-1 Measurement event prediction use case

- Issue 1-1-2 Radio Link Failure (RLF) prediction use case

- Issue 2-1-4 Impacts of measurement error on prediction accuracy

- Issue 2-1-1a Relative RSRP Prediction Accuracy

- Issue 2-1-2 Ground truth definition for RSRP accuracy

- Issue 3-1-3 Considering multiple cells in the testing setup

- Issue 3-2-2 Principle of FR2 testing

- Issue 3-2-4 Testing aspects of inter-carrier scenarios

- Issue 2-1-3 Impacts on beam/cell level measurements

- Issue 2-1-9 Impact from observation and prediction windows

- Issue 3-2-1 Testing goal

# Topic #1: General Aspects

This section addresses general aspects and priorities of different scenarios and sub use-cases identified in the draft TR 38.744.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500232 | Apple | General aspects on AI/ML for mobility in NR Observation 1: RRM measurement predictions provide several advantages, including reducing measurement gaps, increasing the periodicity of SMTC windows, minimizing scheduling and measurement restrictions, decreasing measurement delays, and shortening measurement gap lengths Observation 2: Further discussion is needed in RAN4 regarding future requirements for RLF design: - Should the indication be reported from the UE to the network, or only from the UE's lower layers to higher layers? - If a direct or indirect indication is sent from the UE to the network: - To be determined (FFS): How should such an RLF indication be triggered to the network? - For a direct indication, should the UE report a predicted RLF to the network when all measured and/or predicted downlink radio link quality values on all configured RLM-RS resources fall below a threshold (Qout)? - For an indirect indication, should the UE report all predicted SINR values with corresponding RS indices to the network when the measured and/or predicted downlink radio link quality on all configured RLM-RS resources is worse than Qout? Alternatively, should the UE indicate the predicted SINR with the relevant RS indices as long as at least one configured RLM-RS resource shows measured and/or predicted downlink radio link quality? Proposal 1: RAN4 should evaluate the combined benefits of RRM measurement predictions across multiple domains (such as spatial beam/cell, frequency, and temporal) and dimensions (across various cells and frequencies). Additionally, a working plan should be established to investigate scenarios that maximize these potential advantages. Proposal 2: For measurement event prediction, leverage AI/ML for temporal prediction of {TX, RX} Beam Pair Changes to optimize RRM measurement efficiency. To minimize RRM measurement overhead, the AI/ML model will predict the timing of beam pair changes, enabling the system to conserve resources by focusing measurements only during periods when beam changes are predicted. This approach eliminates the necessity for constant beam monitoring. |
| R4-2501961 | Nokia | - On General Aspects of AIML Mobility Observation 1: RAN2 started the evaluation of measurement event prediction use case in RAN2#129 meeting. Proposal 1: RAN4 should postpone the start of the discussion on measurement event prediction use case to RAN4#114bis meeting Observation 2: RAN2 started the evaluation of RLF prediction use case in RAN2#129bis meeting. Proposal 2: RAN4 should postpone the start of the discussion on RLF prediction use case to RAN4#115 meeting. |
| R4-2501759 | Ericsson | General discussion on AI/ML for mobility Observation 1: Focus/scope of setting the requirement should not be on the details of the AI/ML model deployed to support mobility functionality. Observation 2: Measurements defined for legacy L3-based mobility are used as model input for the prediction of RRM measurement for mobility. Proposal 1: Requirements and side-conditions for legacy L3-based mobility measurements are applicable to the measurements performed by the UE for RRM measurement prediction for mobility purposes. Observation 3: Impact of the error in the measurements used as input for the AI/ML model for RRM measurement prediction on the accuracy of predicted measurements/event needs to be evaluated. Proposal 2: RAN4 to send an LS to RAN2 to at least consider impact of error on measurement (for example due to interference from another cell, inaccuracy of the measurement etc.) used for prediction on the accuracy of the predicted measurement/event. |
| R4-2501233 | Huawei, HiSilicon | Discussion on general aspects in AIML mobility Proposal 1: RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress. Proposal 2: RAN4 to consider all three sub-use cases until RAN2 gives the priority. Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy. Pros and Cons need to be analyzed. - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4. - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4. Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset. - Option 1: Use the channel model and the sampling rules. - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner |
| R4-2501339 | vivo | Discussion on general aspects in AIML mobility Proposal 1: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. For scenario 6, the study can be considered after enough progress is made in AI/ML beam management. Proposal 2: For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain Proposal 3: Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases |
| R4-2501099 | CMCC | Discussion on general aspects in AIML mobility Proposal 1: it is proposed to consider all three sub-use cases. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about priorities for AI/ML Mobility scenarios and sub-use cases are summarized

#### Issue 1-1-1: Measurement event prediction use case

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 should postpone the start of the discussion on measurement event prediction use case to RAN4#114bis meeting.

  - Proposal 2 (Apple): For measurement event prediction, leverage AI/ML for temporal prediction of {TX, RX} Beam Pair Changes to optimize RRM measurement efficiency. To minimize RRM measurement overhead, the AI/ML model will predict the timing of beam pair changes, enabling the system to conserve resources by focusing measurements only during periods when beam changes are predicted. This approach eliminates the necessity for constant beam monitoring.

  - Proposal 3 (vivo): For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain.

- Recommended WF:

  - RAN4 should postpone the start of the discussion on measurement event prediction use case to RAN4#114bis meeting.

#### Issue 1-1-2: Radio Link Failure (RLF) prediction use case

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 should postpone the start of the discussion on RLF prediction use case to RAN4#115 meeting.

  - Proposal 2 (Ericsson): RAN4 discussion on RLF prediction requirement shall start after sufficient progress has been made in RAN2.

- Recommended WF:

  - RAN4 to discuss above proposals

#### Issue 1-1-3: Scenario priorities

- Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Proposals and Observations:

  - Proposal 1 (Huawei): RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress.

  - Proposal 2 (vivo): RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. For scenario 6, the study can be considered after enough progress is made in AI/ML beam management.

  - Proposal 3 (Apple): RAN4 should evaluate the combined benefits of RRM measurement predictions across multiple domains (such as spatial beam/cell, frequency, and temporal) and dimensions (across various cells and frequencies). Additionally, a working plan should be established to investigate scenarios that maximize these potential advantages.

  - Proposal 4 (Samsung): RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

- Recommended WF:

  - Companies to discuss above proposals as candidate options.

#### Issue 1-1-4: Sub-use-case priorities

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model) At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Proposals and Observations:

  - Proposal 1 (Huawei): RAN4 to consider all three sub-use cases until RAN2 gives the priority.

  - Proposal 2 (vivo): Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases.

  - Proposal 3 (CMCC): it is proposed to consider all three sub-use cases.

  - Proposal 4 (QC): RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3.

    - FFS on whether and how to differentiate Sub-use case 2 and 3, e.g. one of or both sub-use case can be considered in defining RRM core and performance requirements.

  - Proposal 5 (Ericsson): RAN4 to prioritize sub case 2, where cell level measurements are used for the cell level RRM measurement prediction, for the requirement discussion for the temporal prediction cases.

  - Proposal 6 (Samsung): RAN4 to consider the Sub-use-case 2 as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction

- Recommended WF:

  - RAN4 to consider all three sub-use cases in case of no de-prioritization in RAN2.

# Topic #2: RAN4 requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500594 | NTT DOCOMO, INC | Discussions on RAN4 requirement impacts of AIML for mobility Observation 1: RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition. Proposal 1: RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML. Proposal 2: For PSS/SSS sync, - In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results. - In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases. Proposal 3: For SSB time index acquisition is need more discussions for all the scenarios. |
| R4-2501133 | OPPO | Study of impacts on RAN4 requirements for AI mobility Observation 1: For Sub-use case 1/2/3, differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction, the entire process or a portion thereof being replaced by AI/ML modules or solutions. For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms. Proposal 1: For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3-level measurements at point C. Proposal 2: RAN4 to evaluate and decide whether to define separate or unified requirements for different sub-use cases during WI stage. Proposal 3: RAN4 to use the following definition for measurement prediction accuracy: - Absolute L3 Predicted RSRP Accuracy is defined as the RSRP difference between reported predicted L3-RSRP and the ground truth of L3-RSRP of the same cell at point C, where ground truth is defined as the actual L3-RSRP measurement results based on legacy measurement procedure. - Relative L3 Predicted RSRP Accuracy can be defined as the RSRP difference between any two reported predicted L3-RSRP results. Proposal 4: The above definitions apply to both intra-frequency and inter-frequency measurement prediction. Proposal 5: RAN4 to consider certain channel model, beam consolidation configuration and L3 filter configuration as possible side conditions when defining the requirements. Proposal 6: RAN4 to consider the impact of UE speed, PW length, MRRT or skipping patterns on intra-frequency measurement prediction accuracy requirements. Proposal 7: RAN4 to consider the impact of correlation coefficient between two frequency layers or cluster approach on inter-frequency measurement prediction accuracy requirements. Observation 2: To avoid redundant work of assessments, no need to introduce measurement error evaluations in the research of RAN4 AI mobility study at this stage as the similar work is performing in the AI/ML WI. Observation 3: Initiating evaluations on measurement error is not the most critical task within the current 0.5 TU framework in RAN4. Proposal 8: RAN4 to focus on the possible requirements to be defined and the testability discussion for AI mobility during SI stage. Proposal 9: Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 scope in WI stage. Proposal 10: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length. Proposal 11: RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values. Proposal 12: RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation. Proposal 13: RAN4 to investigate the possibility of updates on scheduling restrictions, measurement capabilities or definition of known conditions due to measurement prediction. |
| R4-2501760 | Ericsson | On requirements for AI/ML based mobility Observation 1: Evaluation of beam level RRM measurement prediction for mobility has been de-prioritized in RAN2. Proposal 1: RAN4 to not study the impact of beam level measurement prediction (i.e., L1 beam-level (Point A1) and L3 beam-level (Point E)) on RAN4 requirement unless agreed to be considered for AI/ML based mobility by RAN2. Observation 2: The duration of OW or the number of measurements performed inside the OW has an impact on the accuracy of measurements predicted by the UE to be valid for the prediction instances inside the PW for temporal prediction cases. Observation 3: Prediction accuracy in temporal prediction cases is highest in sub case 2. Proposal 2: RAN4 to study relationship between the OW length and PW length and the impact of the number of measurements performed within the OW on the accuracy of the predicted measurements for the instances inside the PW for temporal prediction cases. Proposal 3: RAN4 to prioritize sub case 2, where cell level measurements are used for the cell level RRM measurement prediction, for the requirement discussion for the temporal prediction cases. Proposal 4: RAN4 to study the impact of measurement skipping within the OW on RAN4 requirement for temporal prediction case-B. Observation 4: Higher the correlation coefficient between the carriers of the input and output cells, higher is the accuracy of RRM measurements predicted for the output cell in frequency prediction case (scenario 3 in prioritized scenario). Observation 5: Clustering approach (measurement from multiple cells is used for RRM measurement prediction of an output cell) provides higher prediction accuracy in comparison to the case where the measurement from single cell in one frequency is used as an input to predict the RRM measurement for an output cell in another frequency. Proposal 5: RAN4 to study the impact of frequency relationship between the measured and predicted cell on the performance of frequency domain prediction of RRM measurement for mobility. Proposal 6: RAN4 to study requirement aspects for the following cases: - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. Observation 6: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the event prediction case. Proposal 7: RAN4 to study reporting delay requirement for predicted event triggered reporting. Proposal 8: Performance requirement for direct event prediction needs to be defined for event prediction. Proposal 9: RAN4 discussion on RLF prediction requirement shall start after sufficient progress has been made in RAN2. Proposal 10: Legacy definition for absolute RSRP accuracy and relative RSRP accuracy is adopted for AI/ML based mobility. Proposal 11: RAN4 to consider both absolute and relative RSRP accuracy requirements for AI/ML based mobility. |
| R4-2500696 | ZTE Corporation, Sanechips | Discussion impacts on requirements of AI/ML mobility Proposal 1: RAN4 should discuss the RRM impact on indirect measurement event prediction and direct measurement event prediction. Proposal 2: Measurement error should be considered into measurement event prediction, which is similar as RRM measurement prediction. Proposal 3: RAN4 should consider both serving cell and neighbor cell measurment prediction for intra-cell prediction. Specifically, serving cell (input) to predict the serving cell (output) and neigbour cell i (input) to predict the neigbour cell i (output). Proposal 4: Due to L1-RSRP is for beam management, it’s fine for us not to consider point A1 in AI Mob. Observation1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. And according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4). Proposal 5：RAN4 to deprioritize studying the impacts on requirements for L3 beam-level prediction. Proposal 6: If L3 filtered beam level prediction cases are lower priority, the definition of prediction accuracy for point E will correspondingly be given low priority. Proposal 7: For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test. Proposal 8: For FR2, the definiation of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP). Proposal 9: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances. Observation 2: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. Proposal 10: RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility. Proposal 11: The methodology of adding measurement error to AI mob should be similar to AI BM. To avoid redundant discussions, the simulation results of AI BM in RAN4 should be utilized as much as possible. Proposal 12: Measurement errors will affect the accuracy of predictions, and the larger the BB error, the greater the impact on prediction performance. |
| R4-2501100 | CMCC | Discussion on impacts on RAN4 requirements for AI/ML for mobility Proposal 1: Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - For FR2, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP) - For FR1, ground truth of L3-RSRP is the transmitted power, which is obtained as the legacy way of conducted test Proposal 2: except L3 cell level RSRP (Point C) prediction, it is proposed that RAN4 to consider L1 beam-level (Point A1). And it is proposed that L3 beam-level (Point E) can be deprioritized based on RAN2 agreements. Proposal 3: if it is agreed that RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event, at least how to model measurement error (e.g. distribution of baseband/RF error) need to be provided to RAN2. Proposal 4: it is proposed to discuss whether to introduce RAN4 requirements about MRRT and MRRS. Proposal 5: for RRM measurement prediction, the measurement delay requirements need to be defined/updated - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements |
| R4-2501234 | Huawei, HiSilicon | Discussion on impacts of RAN4 requirements in AI/ML mobility Proposal1: RAN4 to start the discussion on scenario 2, 3 and 4 with high priority. Scenario 6 (FR2 to FR2 intra-frequency spatial domain for the 1st goal) can be carried out after sufficient progress are achieved on high priority scenarios. Proposal 2: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed. Proposal 3: For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified. Proposal 4: For intra-frequency temporal domain prediction case A, the RSRP prediction accuracy would be specified. Proposal 5: For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT with limited PW length may need to be specified. Proposal 6: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified. Proposal 7: The processing time of inference function may need to be considered in measurement reporting delay. Proposal 8: RAN4 can deprioritize the sub-use cases of L3 filtered beam-level measurement predication (Point E). Proposal 9: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C. Proposal 10: RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases. Proposal 11: In test, to define L3 RSRP prediction accuracy, - the ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. - the ground truth of L3 RPRP in FR1 is FFS, whether it is reported L3 RSRP measurement under sufficient high SNR or the transmitted RSRP directly from TE (conducted test). Proposal 12: The absolute prediction accuracy for both intra-frequency and inter-frequency shall be prioritized, the relative perdition accuracy can be considered after more progress of measurement event prediction is achieved. Observation 1: Measurement impairment may impact the prediction accuracy. Observation 2: The RSRP with measurement impairment can be obtained in reality, rather than the ideal RSRP. Observation 3: In RAN2 assumption, the ideal RSRP is assumed both for model input and the label of model output. Proposal 13: RAN4 to discuss the measurement impairment considered for verifying the prediction accuracy enabled by AI. Proposal 14: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified. Proposal 15: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements. Proposal 16: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface. |
| R4-2500233 | Apple | Study of impacts on RAN4 requirements for AIML Mobility in NR Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A - Proposal 1: RAN4 will need to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E) Proposal 2: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C) Proposal 3: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR. Proposal 4: The proposed cell-level RSRP prediction accuracy is defined as follows: - Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy. - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test) Proposal 5: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps. Proposal 6: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells. Proposal 7: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. - Proposal 8: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. Proposal 9: RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility. |
| R4-2502219 | Qualcomm Incorporated | Definition of ground truth for AI/ML-based RRM measurement prediction Observation 1: It is not reasonable to assume that the same L3-RSRP measurement result at Point C in Figure 9.2.4-1 of TS38.300 should be observed across all UEs, particularly when the channel evolves over time and the measurement reference signals are contaminated due to low SNR, UE mobility, inter-cell/inter-sector interference, etc. Observation 2: It is not reasonable to assume that the predicted strongest neighbor cell would have as high SNR as the predicted strongest beam from the serving cell. Considering the geographical location of the UE, where the AI/ML-based neighbor cell’s RSRP prediction is of interest, is where the UE is not close to the serving cell but near the cell boundary, even the serving cell’s SNR is not expected to be high enough Observation 3: Considering that the models are not likely to be trained solely based on synthetic data generated by simulators, both the model input corresponding to the observation window and the ground truth of the model outputs corresponding to the prediction window will essentially be data sets generated based on actual UE measurement results, not so-called error-free ideal measurement results. Observation 4: The ground truth being the actual measurement result is also in line with the way NW and UE monitor the performance of the AI/ML-based RRM prediction. The effective KPI for performance monitoring is the difference between the prediction result and the actual measurement result on the performance monitoring reference signals, as there is no practical way of defining so-called error-free ideal measurement results. Observation 5: The UE’s L3-RSRP measurement accuracy performance will be ensured by the current requirements and test framework. Thus, the UE’s reported measurement results in more dynamic environments can serve as the benchmark for AI/ML-based L3-RSRP prediction accuracy performance requirements. - For testing, the following two-step test may thoroughly verify AI/ML-based L3-RSRP prediction performance: - Existing L3-RSRP Measurement Accuracy: - It is tested under static channel conditions (i.e., no RSRP variation during the entire test run), hence there is no ambiguity in the definition of the ground truth (or reference) RSRP. - With this, it can be claimed that the reported RSRP from the UE can serve as the ground truth for AI/ML-based L3-RSRP prediction in the second step below. - New Test Case for AI/ML-based L3-RSRP Prediction under Time-varying RSRP Conditions: - UE can be tested under multiple different RSRP variation profiles. - X% of the data set can be used for AI/ML model training. Note that it does not mean online training. - The rest of the data set can be used for AI/ML-based L3-RSRP prediction testing. - At time instance T0, UE reports AI/ML-based L3-RSRP prediction results based on previous observations. The reports correspond to future time instances. - UE and NW will be in sync in terms of how far into the future the measurement prediction results correspond to. - The accuracy/error can be defined as the difference between 'reported prediction results' and 'reported actual measurement results' corresponding to the same or closely timed instances. - The above is expected to allow UE to collect actual measurement results on/offline (measurement errors and UE implementation specifics are inherited in the data set) for model training and testing. Proposal 1: RAN4 to consider the following options for the definition of ground truth of L3-RSRP for the predicted absolute L3-RSRP accuracy (reported predicted L3-RSRP – ground truth of L3-RSRP): - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP) - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient. - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc. - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition (UE-reported L3-RSRP with noise) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms. - [Testability: to avoid UE from cheating the test, TE can create another cell having the same radio condition to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction] - The definition of the ground truth does not differentiate FR1 and FR2. - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free) - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough. - [Testability: another round of tests would be needed for the L3-RSRP measurement report] - The definition of the ground truth does not differentiate FR1 and FR2. Performance metrics for AI/ML-based RRM measurement prediction Observation 6: The relative cell level strength metric is important for event-based L3-RSRP reports and conditional handovers, as they are widely used in the field for measurement gap configuration, handover preparation, and handover execution decisions. Among multiple event types, the most commonly used event type is Event-A3, which is defined based on the relative cell level strength between the serving cell and a neighbor cell. Proposal 2: RAN4 to consider a relative L3-RSRP as one of the performance metrics for AI/ML-based L3-RSRP prediction with the following definition: - Accuracy of predicted relative L3-RSRP = Reported predicted relative L3-RSRP - Ground truth of relative L3-RSRP, where the ground truth of relative L3-RSRP is defined as follows: - For intra-frequency L3-RSRP, - Difference between ‘the ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from one cell’ and the ‘ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from another cell on the same frequency’, or - Difference between the ground truths of two L3-RSRPs defined for the predicted absolute L3-RSRP accuracy from the same cell - For inter-frequency, - Difference between ‘the ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from one cell on a frequency’ and the ‘ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from another cell on a different frequency in the same FR Observation 7: AI/ML-based RRM prediction is not expected to entirely replace actual UE measurements on neighbor cells, particularly during handover, due to the required downlink timing and frequency synchronization procedure with the target cell. If the AI/ML model can accurately predict the N strongest neighbor cells, the serving cell may reconfigure the measurement objects accordingly, allowing the UE to perform actual measurements on those cells. Observation 8: The RAN2 agreement also hints that the network would not solely rely on AI/ML-based RRM prediction results but would require the report of actual measurement results when deciding UE mobility. Observation 9: Based on system level simulation campaign conducted in RAN2 under a noise-free environment, AI/ML-based L3-RSRP prediction does not yet show decent accuracy. Proposal 3: RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction: - Top-1 (%) : The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’ - Top-K/1 (%) : The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’ - Top-1/K (%) : The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’ - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP. Performance metrics for AI/ML-based Measurement event prediction Proposal 4: RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction: - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction. - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP. - The above two metrics may jointly serve as a performance assessment metric for event prediction, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2: - Whether the AI model is for indirect or direct event prediction. - What metrics are to be reported to the serving cell. Sub-use-cases Proposal 5: RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3. - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - FFS on whether and how to differentiate Sub-use case 2 and 3, e.g. one of or both sub-use case can be considered in defining RRM core and performance requirements RRM prediction performance and impact of measurement error Proposal 6: RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g. - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach. - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction. - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction. - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions. Potential impact on RRM requirements Proposal 7: RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g. - Common aspects: - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc. - Measurement and scheduling restrictions in the prediction window. - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation. - Side conditions such as requirement applicability rules. - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements. - Scenario 2: FR1 to FR1 intra-frequency temporal domain case B, Intra-cell - Uniform measurement approach for cells in the same frequency layer - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell - CSSF - MG - Scenario 4: FR2 to FR2 intra-frequency temporal domain case A, Intra-cell - Uniform measurement approach for cells in the same frequency layer |
| R4-2500619 | Xiaomi | Discussion on impacts on RAN4 requirement for AI mobility Observation 1: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side. Observation 2: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side. Observation 3: Beam consolidation will have impact on deriving ideal cell level L3-RSRP from TE side. If beam consolidation is not considered, cell-level L3-RSRP at point C equals to filtered L1-RSRP at point A1. Observation 4: if TE will calculate ideal cell-level L3-RSRP, several issues need to be solved and some aspects will have limitation on UE implementation. Proposal 1: If the TE is capable of calculating the ideal cell-level L3-RSRP, RAN4 should discuss the specific conditions and information required for the TE to accurately determine this ideal value. Proposal 2: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Proposal 3: RAN4 should clarify the definition of relative RSRP prediction accuracy before determining whether to include it as a requirement. Proposal 4: Observation window and prediction window defined in RAN2 will have RRM impact: - Observation window will have impact on measurement duration - Prediction window will have impact on scheduling restriction Observation 5: Observation window and prediction length is dependent on different L1/L3 filtering method. Observation 6: Observation window and prediction length is dependent on different sub-cases. Observation 7: Observation window and prediction length is dependent on different patterns. Proposal 5: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window. Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period. Proposal 6: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period. |
| R4-2501674 | Samsung | Discussion on AI mobility general aspects Observation 1: The intention of scenarios 3 is to reduce the measurement overhead, one of the references would be measure all time instances based on the MG configuration Observation 2: For temoral domain prediction, the prediction accuracy obtained based on sliding and non-sliding filtering cases are different under the same simulation assumptions Observation 3: The existing RSRP accuracy contains measurement error, which is made up of RF calibration error and baseband measurement error. The RF calibaration on FR1 and FR2 may be different Proposal 1: RAN4 to consider the Sub-use-case 2 as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction Proposal 2: - RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction - RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization Proposal 3: - If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C Proposal 4: RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility Proposal 5: For scenario 3, to accommodate the existing accuracy definition, it is plausible to define relative RSRP accuracy requirement besides the absolute RSRP accuracy requirement Proposal 6: If to consider absolute and relative RSRP prediction accuracy for scenario 3, the accuracy definitions should be further discussed by taking at least the following into account: - Reference/baseline measurements related to 1st goal (measurement overhead reduction) - Difference of RSRP measured and RSRP predicted from different cells on different frequencies - Cell-specific and/or cluster prediction approaches Proposal 7: RAN4 to study accuracy requirements of predicted cell level RSRP for the proposed sub-use-case (s) and evaluation scenario (s), suggest to consider the following together - Sliding and/or non-sliding filtering cases - Total number of samples - SNR Side conditions Proposal 8: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error. - If additional simulations are needed, suggest to do the required simulation in WI phase Proposal 9: Suggest to wait for more progress/evaluation outcomes in AI/ML BM (especially for FR2 temporal domain prediction) to study the impact of measurement error on the prediction accuracy requirements for AI mobility Proposal 10: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction Proposal 11: For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for temporal domain prediction case A Proposal 12: For the RRM impact on prediction window, at least the following may need to be discussed - Scheduling restriction Measurement latency for MO (s), MO (s) reduction |
| R4-2500890 | MediaTek Inc. | Discussion on RRM requirements of AI mobility Proposal 1: For RRM measurement prediction, only use cell level RSRP as the performance metric. Proposal 2: For RRM measurement prediction, make down-selection from the following two options: - Option 1 - Absolute RSRP and relative RSRP accuracy is defined as legacy, i.e.: - absolute RSRP accuracy = reported result – ground truth - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2) - extra margin should be added on top of the defined accuracy requirements in the test considering that transmitted power at TE may not be used as the ground truth directly. - Option 2 - Absolute RSRP and relative RSRP accuracy is defined based on the test setup, i.e.: - absolute RSRP accuracy = reported result – “ground truth” - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (“ground truth” of RSRP of cell 1 – “ground truth” of RSRP of cell 2) - “ground truth” is the referenced RSRP to compare with in the test. Proposal 3: For scenario 2 and 3, set single SSB beam and not to use L3 filtering in the test. Proposal 4: For scenario 2 and 3, the “ground truth”, i.e., referenced RSRP to compare with in the test, can be derived based on the transmitted power at TE and the assumed L1 filtering. Proposal 5: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated cell level RSRP. Proposal 6: For scenario 3 of RRM measurement prediction, define the absolute and/or relative accuracy requirements of predicted/generated cell level RSRP. Observation 1: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases. Proposal 7: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown. Proposal 8: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring. Proposal 9: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility. Proposal 10: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2. Proposal 11: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved. Proposal 12: Scenario 4 has no impact on measurement delay requirements. |
| R4-2501962 | Nokia | On RRM Requirement Impacts of AIML Mobility Proposal 1: RAN4 may consider UE reported L3 RSRP measurement as the approximate of ideal measurement of L3-RSRP and the ground truth for FR2 L3 prediction. Proposal 2: RAN4 should consider that the test equipment in FR1 has the information of ground truth regarding L3-RSRP (transmit power). Proposal 3: RAN4 to consider both absolute and relative accuracy for measurement prediction. Observation 1: Depending on the scenario, the UE may be required to compare the predicted RSRP of one cell or beam with either the measured or predicted RSRP of another cell or beam. Proposal 4: RAN4 to define relative accuracy for the comparison between the predicted RSRP of one cell with either the measured or predicted RSRP-level of another cell or same cell. Observation 2: Definition and requirements for predicted relative accuracy need to be defined and aligned with the established definitions and requirements for legacy relative measurement accuracy specified in TS 38.133. Observation 3: It is expected Case-B temporal domain prediction to comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. Proposal 5: For at least temporal domain AI/ML mobility measurement prediction, RAN4 to define relative accuracy as “reported SS-RSRP level of one cell compared to another reported SS-RSRP level from another cell or the same cell, where the reported level is predicted or measured or a combination of both predicted and measured samples”. Proposal 6: RAN4 may study a different relative accuracy definition for frequency domain prediction aligning with definitions of RAN4 AI/ML for beam management. One option is to define relative accuracy as ground truth SS-RSRP of (the best) predicted cell compared to the ground truth of the highest SS-RSRP of another cell or the same cell. Observation 4: In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM. Observation 5: Beam-level L3 predictions are deprioritized in RAN2 FS_NR_AIML_Mob - Release 19. Proposal 7: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E). Observation 6: The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement. Observation 7: According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy. Proposal 8: RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2). Observation 8: RAN4 can discuss how to capture and model the measurement error with the following options. - Option 1: Both RF and baseband errors follow a Gaussian distribution. - Option 2: Baseband and RF errors follow different distributions. - Baseband error is modeled as: - Option A: Gaussian distribution - Option B: link-level simulation results - RF error is modeled as: - Option A: uniform distribution - Option B: Gaussian distribution - Option C: truncated Gaussian distribution - Option D: fixed value - How to apply RF error: - Option A: Independent for each TX beam - Option B: Same for each Tx beam - Option C: Independent for each module/panel - Option D: Same for each module/panel - Option 3: Other options are not precluded. Proposal 9: RAN4 should agree that both RF and baseband errors follow a Gaussian distribution (Option 1). Proposal 10: RAN4 should send an LS to RAN2, suggesting that RAN2 considers measurement errors in its evaluation. Proposal 11: RAN4 should investigate requirements related to LCM procedures (in particular model activation, deactivation, switching and fallback operation), aligned with RAN2 progress on this topic. Proposal 12: LCM framework agreed in AIML air interface work item will be used as baseline for any further enhancement for AIML mobility in RAN4. Proposal 13: RAN4 should investigate impacting factors on the selection of observation and prediction windows for RAN4 requirements and test cases specification. Observation 9: The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions. Proposal 14: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios. |
| R4-2501340 | Vivo | Discussion on impacts on RAN4 requirements for AI mobility Observation 1: For temporal domain case B, considering the measurement in the prediction window may be skipped directly, the reported RSRP measurement for the corresponding time instance may not be available as the ground truth of L3 RSRP to be considered Proposal 1: For testing, the ground truth definition of L3-RSRP for FR1 and FR2 can be further studied based on the following candidate Options: - Option 1: To define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR - Option 2: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2 Observation 2: For the prediction case on frequency domain, the discussion and simulation of RAN2 is still limited to the scenario of single/multiple cells as input and a single cell as output. Proposal 2: RAN4 to first consider absolute RSRP accuracy requirements. And for the relative RSRP accuracy, it can be further considered if the necessary scenarios are supported Proposal 3: RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI . |
| R4-2501040 | CATT | Discussion on impacts of AIML mobility on RRM requirements Proposal 1: RAN4 to postpone the discussion on the definition of ground truth in FR2. Proposal 2: The definition of ground truth in FR1 can be the ideal L3 filtered cell-level measurement results for both temporal and frequency domain predictions. Proposal 3: RAN4 not to consider studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E to align with RAN2 prioritization. Proposal 4: RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues. |
| R4-2501694 | Tejas Network Limited | Study of impacts on RAN4 requirements for AIML Mobility in NR - Observation 1: Sub-use Case 1 helps reduce measurements, but prediction inaccuracies may degrade the generated L3 cell-level measurements and handover performance. - Observation 2: Sub-use Case 2 and 3 help improve handover performance through reliable predictions. - Observation 3: Intra-frequency temporal domain Case A has been categorized as low priority. - Observation 4: larger gap between consecutive observation windows and the absence of overlap between the prediction and observation windows make Intra-frequency temporal domain Case B, Example 1, prediction less reliable. Observation 5: Predicting multiple inter-frequency measurements can be challenging due to varying channel conditions across different frequencies - Proposal 1: RAN4 should prioritize the study and performance evaluation of Sub-use case 2. - Proposal 2: Prioritize intra frequency temporal domain Case B example 2, for study and performance evaluation. Proposal 3: RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction. - Proposal 4: For FR1 conducted tests, the ground truth should be derived from the test equipment. Proposal 5: RAN4 to consider both absolute and relative RSRP accuracy requirements for study. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about the RRM requirements for AI/ML Mobility use cases are summarized.

#### Issue 2-1-1a: Relative RSRP Prediction Accuracy

- Proposals and Observations:

  - Proposal 1 (Oppo): Relative L3 Predicted RSRP Accuracy can be defined as the RSRP difference between any two reported predicted L3-RSRP results.

  - Proposal 2 (Ericsson):

    - RAN4 to consider both absolute and relative RSRP accuracy requirements for AI/ML based mobility.

    - Legacy definition for absolute RSRP accuracy and relative RSRP accuracy is adopted for AI/ML based mobility.

  - Proposal 3 (Huawei):  The absolute prediction accuracy for both intra-frequency and inter-frequency shall be prioritized, the relative perdition accuracy can be considered after more progress of measurement event prediction is achieved.

  - Proposal 4 (Qualcomm): RAN4 to consider a relative L3-RSRP as one of the performance metrics for AI/ML-based L3-RSRP prediction with the following definition:

    - Accuracy of predicted relative L3-RSRP = Reported predicted relative L3-RSRP - Ground truth of relative L3-RSRP, where the ground truth of relative L3-RSRP is defined as follows:

      - For intra-frequency L3-RSRP,

        - Difference between ‘the ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from one cell’ and the ‘ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from another cell on the same frequency’, or

        - Difference between the ground truths of two L3-RSRPs defined for the predicted absolute L3-RSRP accuracy from the same cell

      - For inter-frequency,

        - Difference between ‘the ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from one cell on a frequency’ and the ‘ground truth of L3-RSRP defined for the predicted absolute L3-RSRP accuracy from another cell on a different frequency in the same FR

  - Proposal 5 (Xiaomi): RAN4 should clarify the definition of relative RSRP prediction accuracy before determining whether to include it as a requirement.

  - Proposal 6 (Samsung):

- For scenario 3, to accommodate the existing accuracy definition, it is plausible to define relative RSRP accuracy requirement besides the absolute RSRP accuracy requirement

- If to consider absolute and relative RSRP prediction accuracy for scenario 3, the accuracy definitions should be further discussed by taking at least the following into account:

    - Reference/baseline measurements related to 1st goal (measurement overhead reduction)

    - Difference of RSRP measured and RSRP predicted from different cells on different frequencies

    - Cell-specific and/or cluster prediction approaches

  - Proposal 7 (MediaTek):

    - Option1: relative RSRP accuracy is defined as legacy, i.e.:

      - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2)

      - extra margin should be added on top of the defined accuracy requirements in the test considering that transmitted power at TE may not be used as the ground truth directly.

    - Option 2: relative RSRP accuracy is defined based on the test setup, i.e.:

      - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (“ground truth” of RSRP of cell 1 – “ground truth” of RSRP of cell 2)

      - “ground truth” is the referenced RSRP to compare with in the test.

  - Proposal 8 (Nokia):

    - For at least temporal domain AI/ML mobility measurement prediction, RAN4 to define relative accuracy as “reported SS-RSRP level of one cell compared to another reported SS-RSRP level from another cell or the same cell, where the reported level is predicted or measured or a combination of both predicted and measured samples”.

    - RAN4 may study a different relative accuracy definition for frequency domain prediction aligning with definitions of RAN4 AI/ML for beam management.  One option is to define relative accuracy as ground truth SS-RSRP of (the best) predicted cell compared to the ground truth of the highest SS-RSRP of another cell or the same cell.

  - Proposal 9 (vivo): RAN4 to first consider absolute RSRP accuracy requirements. And for the relative RSRP accuracy, it can be further considered if the necessary scenarios are supported

- Candidate options:

  - Option 1: relative RSRP accuracy is defined as.:

    - relative RSRP = reported RSRP of cell 1 – reported RSRP of cell 2

  - Option 2: relative RSRP accuracy is defined as legacy, i.e.:

    - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2)

- Recommended WF

- Discuss above candidate Options.

- FFS: cell 1 and cell 2 are inter-frequency or intra-frequency.

- FFS: whether extra margin should be added on top of the defined accuracy requirements in the test

- FFS: whether ground truth can be reported or ideal (Follow issue 2.1.2)

- FFS: whether the reported RSRP can be predicted or measured or a combination of both predicted and measured samples

#### Issue 2-1-1b: Strongest Cell Prediction accuracy

- Proposals and Observations:

  - Proposal 1 (Qualcomm): RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction:

    - Top-1 (%) : The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’

    - Top-K/1 (%) : The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’

    - Top-1/K (%) : The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’

    - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB

    - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP.

- Recommended WF

  - Discuss the above proposal.

#### Issue 2-1-2: Ground Truth Definition for RSRP accuracy

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

    - For FR2, the definition of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP).

  - Proposal 2 (Huawei):

    - The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

    - The ground truth of L3 RPRP in FR1 is FFS, whether it is reported L3 RSRP measurement under sufficient high SNR or the transmitted RSRP directly from TE (conducted test).

  - Proposal 3 (Apple):

    - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

    - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

  - Proposal 4 (Qualcomm):

    - Option 1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 2-A: Ground truth based on measurement report from the UE under the SAME radio condition (UE-reported L3-RSRP with noise)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

        - [Testability: to avoid UE from cheating the test, TE can create another cell having the same radio condition to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction]

      - The definition of the ground truth does not differentiate FR1 and FR2.

    - Option 2-B: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

      - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

        - [Testability: another round of tests would be needed for the L3-RSRP measurement report]

      - The definition of the ground truth does not differentiate FR1 and FR2.

  - Proposal 5 (Xiaomi):

    - If the TE is capable of calculating the ideal cell-level L3-RSRP, RAN4 should discuss the specific conditions and information required for the TE to accurately determine this ideal value.

    - If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports.

  - Proposal 6 (MediaTek):

    - For scenario 2 and 3, the “ground truth”, i.e., referenced RSRP to compare with in the test, can be derived based on the transmitted power at TE and the assumed L1 filtering.

  - Proposal 7 (Nokia):

    - RAN4 may consider UE reported L3 RSRP measurement as the approximate of ideal measurement of L3-RSRP and the ground truth for FR2 L3 prediction

    - RAN4 should consider that the test equipment in FR1 has the information of ground truth regarding L3-RSRP (transmit power).

  - Proposal 8 (vivo): For testing, the ground truth definition of L3-RSRP for FR1 and FR2 can be further studied based on the following candidate Options:

    - Option 1: To define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

    - Option 2: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

  - Proposal 9 (CATT):

    - The definition of ground truth in FR1 can be the ideal L3 filtered cell-level measurement results for both temporal and frequency domain predictions

    - RAN4 to postpone the discussion on the definition of ground truth in FR2.

  - Proposal 10 (Tejas Network Limited)

    - For FR1 conducted tests, the ground truth should be derived from the test equipment.

  - Proposal 11 (OPPO)

    - ground truth is defined as the actual L3-RSRP measurement results based on legacy measurement procedure for both FR1 and FR2.

  - Proposal 12 （CMCC）

    - For FR2, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP)

    - For FR1, ground truth of L3-RSRP is the transmitted power, which is obtained as the legacy way of conducted test

  - Proposal 13 (Samsung): Two directions can be considered to define ideal RSRP

    - 	Direction 1: Settled by TE. The ideal RSRP is of the transmit power

    - 	Direction 2: Settled by measurement report. The ideal RSRP can be derived based on the RSRP measurements reported by UE for the noise-free or very high SNR conditions

For the both directions above, emulation of noise-free/high SNR test conditions and assumption of single signal source are supported

- Candidate options:

  - Option 1: The definition of the ground truth may differentiate between FR1 and FR2.

  - Option 2: The definition of the ground truth does not differentiate between FR1 and FR2.

  - Option 1-FR1:

    - Option 1-1-FR1: For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test.

      - FFS: If certain high SNR is required and what may be the SNR level required?

      - FFS: The power level at which SS-RSRP measurement is conducted.

    - Option 1-2-FR1: The ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

    - Option 1-3-FR1: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

    - Option 1-4-FR1: The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X

      - FFS: If certain high SNR is required and what may be the SNR level required?

      - FFS: Further study factors impacting Y, such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc

    - Option 1-5-FR1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

  - Option 1-FR2:

    - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

    - Option 1-2-FR2: The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X

      - FFS: If certain high SNR is required and what may be the SNR level required?

      - FFS: Further study factors impacting Y, such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc

    - Option 1-3-FR2: : Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

      - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

      - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

    - Option 1-4-FR2: RAN4 to postpone the discussion on the definition of ground truth in FR2.

  - Option 2-1: Ground truth based on expected measurement result (spec-defined expected L3-RSRP)

    - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE and the configured filter coefficient.

    - FFS on whether and how to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc.

  - Option 2-2: Ground truth based on measurement report from the UE under the SAME radio condition (UE-reported L3-RSRP with noise)

    - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

      - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

      - The radio condition such as S(I)NR remains the same during the target inference time instance and the report instance, i.e., from X – Y ms to X + Y ms.

      - [Testability: to avoid UE from cheating the test, TE can create another cell having the same radio condition to the extent possible (e.g., large-scale fading) as the target cell for the L3-RSRP prediction]

    - The definition of the ground truth does not differentiate FR1 and FR2.

  - Option 2-3: Ground truth based on measurement report from the UE under the IDEAL radio condition (UE-reported L3-RSRP with noise-free)

    - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

      - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

      - The radio condition such as S(I)NR for the measurement report is set such that the measurement result at the UE can be accurate enough.

      - [Testability: another round of tests would be needed for the L3-RSRP measurement report]

    - The definition of the ground truth does not differentiate FR1 and FR2.

- Recommended WF

  - First Discuss Option 1 and Option 2.

  - If Option 1

    - Discuss Option 1-FR1 and Option 1-FR2

  - If Option 2

    - Discuss Option 2-1, Option 2-2 and Option 2-3.

  - If no agreement on Option 1 and Option 2, the moderator suggests to start the discussion with Option 1-FR1 and Option 1-FR2.

#### Issue 2-1-3: Impacts on beam/cell level measurements

- Background:

The following agreement was achieved in RAN4#112bis meeting:

Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

The figure below from TS 38.300 (Figure 9.2.4-1) should be considered as a reference for this discussion.

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - Due to L1-RSRP is for beam management, it’s fine for us not to consider point A1 in AI Mob.

    - RAN4 to deprioritize studying the impacts on requirements for L3 beam-level prediction.

    - If L3 filtered beam level prediction cases are lower priority, the definition of prediction accuracy for point E will correspondingly be given low priority.

  - Proposal 2 (Huawei):

    - RAN4 can deprioritize the sub-use cases of L3 filtered beam-level measurement predication (Point E).

    - For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C.

    - RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

  - Proposal 3 (Apple):

    - RAN4 will need to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E)

    - For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C)

  - Proposal 4 (Samsung):

    - RAN4 to prioritize studying the impacts on requirements for L3 cell level RSRP (Point C) prediction

    - RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction, aligning with RAN2 prioritization

    - If it is proposed to consider L1 beam level prediction, RAN4 to study the impacts on requirements for L1 beam level RSRP (Point A1) prediction

      - FFS: Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

    - RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

  - Proposal 5 (MediaTek):

    - For RRM measurement prediction, only use cell level RSRP as the performance metric.

  - Proposal 6 (Nokia):

    - RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

  - Proposal 7 (Oppo):

    - For all three sub-use cases, RAN4 to define prediction accuracy requirements and testing for L3-level measurements at point C.

  - Proposal 8 (Ericsson):

    - RAN4 to not study the impact of beam level measurement prediction (i.e., L1 beam-level (Point A1) and L3 beam-level (Point E)) on RAN4 requirement unless agreed to be considered for AI/ML based mobility by RAN2.

  - Proposal 9 (CMCC):

    - except L3 cell level RSRP (Point C) prediction, it is proposed that RAN4 to consider L1 beam-level (Point A1). And it is proposed that L3 beam-level (Point E) can be deprioritized based on RAN2 agreements.

- Candidate options:

  - Option 1: Do not consider L3 beam-level (Point E) and L1 beam-level (Point A1) measurement prediction

  - Option 2: Consider L3 beam-level (Point E) measurement prediction as second priority

  - Option 3: Consider L1 beam-level (Point A1) measurement prediction as second priority

  - Option 4: RAN4 should consider studying the impacts on requirements for L3 beam-level (Point E)

  - Option 5: RAN4 should consider studying the impacts on requirements for L1 beam-level (Point A1)

- Recommended WF

  - Discuss different options listed above.

#### Issue 2-1-4: Impacts of measurement error on prediction accuracy

- Background

Following candidate options were identified in RAN4#113 meeting’s moderator’s summary.

- Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - The methodology of adding measurement error to AI mob should be similar to AI BM. To avoid redundant discussions, the simulation results of AI BM in RAN4 should be utilized as much as possible.

    - Measurement errors will affect the accuracy of predictions, and the larger the BB error, the greater the impact on prediction performance.

  - Proposal 2 (Huawei):

    - RAN4 to discuss the measurement impairment considered for verifying the prediction accuracy enabled by AI.

  - Proposal 3 (Apple):

    - RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility.

  - Proposal 4 (Qualcomm):

      - RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g.

        - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach.

        - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction.

        - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction.

        - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions.

  - Proposal 5 (Nokia):

    - RAN4 should send an LS to RAN2, suggesting that RAN2 considers measurement errors in its evaluation

    - RAN4 should agree that both RF and baseband errors follow a Gaussian distribution

  - Proposal 6 (vivo):

    - RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI .

  - Proposal 7 (Oppo):

    - Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 scope in WI stage.

  - Proposal 8 (CMCC):

    - if it is agreed that RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event, at least how to model measurement error (e.g. distribution of baseband/RF error) need to be provided to RAN2

  - Proposal 9 (Samsung):

    - RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error.

      - If additional simulations are needed, suggest to do the required simulation in WI phase

    - Suggest to wait for more progress/evaluation outcomes in AI/ML BM (especially for FR2 temporal domain prediction) to study the impact of measurement error on the prediction accuracy requirements for AI mobility

  - Proposal 10 (Ericsson):

    - RAN4 to send an LS to RAN2 to at least consider impact of error on measurement (for example due to interference from another cell, inaccuracy of the measurement etc.) used for prediction on the accuracy of the predicted measurement/event.

- Candidate options:

  - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase

  - Option 2: RAN4 should study the impact of measurement error on the prediction accuracy during WI phase

  - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4)

  - Option 4:  RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

    - RAN4 to discuss and identify specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility.

    - Option 4-a: Both RF and baseband errors follow a Gaussian distribution.

    - Option 4-b: Baseband and RF errors follow different distributions.

      - Baseband error is modeled as:

        - Option A: Gaussian distribution

        - Option B: link-level simulation results

      - RF error is modeled as:

        - Option A: uniform distribution

        - Option B: Gaussian distribution

        - Option C: truncated Gaussian distribution

        - Option D: fixed value

      - How to apply RF error:

        - Option A: Independent for each TX beam

        - Option B: Same for each Tx beam

        - Option C: Independent for each module/panel

        - Option D: Same for each module/panel

    - Option 4-c: Others

- Recommended WF

  - First Discuss Option 1 to Option 4.

  - If Option 4 is agreed

    - Discuss Option 4-a, Option 4-b and Option 4-c.

#### Issue 2-1-5: Inter-frequency prediction scenario

- Proposals and Observations:

  - Proposal 1 (Huawei):

    - For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified.

  - Proposal 2 (Apple):

    - To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps.

  - Proposal 3 (Qualcomm):

    - RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g.

      - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell

        - CSSF

        - MG

  - Proposal 4 (Xiaomi):

    - For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 5 (MediaTek):

    - For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

  - Proposal 6 (Nokia):

    - RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

  - Proposal 7 (Tejas Network Limited)

    - RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction.

  - Proposal 8 (OPPO):

    - RAN4 to consider the impact of correlation coefficient between two frequency layers or cluster approach on inter-frequency measurement prediction accuracy requirements

    - RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation

  - Proposal 9 (Ericsson):

    - RAN4 to study the impact of frequency relationship between the measured and predicted cell on the performance of frequency domain prediction of RRM measurement for mobility.

    - RAN4 to study requirement aspects for the following cases

      - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

      - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - Proposal 10 (Samsung): RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

- Recommended WF

  - RAN4 to study the impacts on RSRP prediction accuracy for inter-frequency prediction scenario considering the following aspects:

    - the side condition of frequency prediction (e.g., EPRE difference)

    - the impact of correlation coefficient between two frequency layers

    - impact of cluster approach, e.g.,

      - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

      - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - RAN4 to study the impacts on measurement period requirements for inter-frequency prediction scenario considering the following aspects:

    - the feasibility of predicting measurement results for multiple cells per frequency layer.

    - the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps.

    - Impacts on CSSF

    - Impacts on MG

    - gap sharing mechanism among different inter-frequency layers when more than one inter-frequency layers are involved.

  - Other aspects for inter-frequency prediction scenario

    - RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

    - RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction.

#### Issue 2-1-6: LCM related requirements

- Proposals and Observations:

  - Proposal 1 (Apple):

    - RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility

  - Proposal 2 (Nokia):

    - RAN4 should investigate requirements related to LCM procedures (in particular model activation, deactivation, switching and fallback operation), aligned with RAN2 progress on this topic.

    - LCM framework agreed in AIML air interface work item will be used as baseline for any further enhancement for AIML mobility in RAN4.

  - Proposal 3 (CATT):

    - RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues.

  - Proposal 4 (Huawei):

    - If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

    - If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

    - Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface.

- Recommended WF Candidate options / tentative agreements:

  - Option 1: RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues.

  - Option 2: RAN4 should start discussion LCM related issues in parallel of discussion on performance metrics.

- Recommended WF

  - LCM related discussions should be held in RAN4 potentially at a later stage. RAN2 needs to progress on this topic.

#### Issue 2-1-7: Impact on Core requirements

- Background

- Following candidate options were identified in RAN4#112bis meeting’s moderator’s summary.

- Candidate options: - Option 1: For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2 - Option 2: For inter-frequency inter-cell frequency domain prediction, RAN4 to study the side condition of frequency prediction (e.g., EPRE difference). - Option 3: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility - Option 4: RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility. - Option 5: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case A considering the impact of OW or PW length.

    - RAN4 to discuss the measurement period requirements for intra-frequency measurement temporal case B considering the impact of skipped measurement instances, e.g., based on MRRT values.

  - Proposal 2 (Ericsson):

    - RAN4 to study the impact of measurement skipping within the OW on RAN4 requirement for temporal prediction case-B.

  - Proposal 3 (ZTE):

    - RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility.

  - Proposal 4 (CMCC):

    - for RRM measurement prediction, the measurement delay requirements need to be defined/updated

      - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

      - For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements

  - Proposal 5 (Huawei):

    - The processing time of inference function may need to be considered in measurement reporting delay.

  - Proposal 6 (Apple):

    - Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

  - Proposal 7 (Qualcomm):

    - RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g.

      - Common aspects:

        - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc.

        - Measurement and scheduling restrictions in the prediction window.

        - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation.

        - Side conditions such as requirement applicability rules.

        - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements.

      - Scenario 2: FR1 to FR1 intra-frequency temporal domain case B, Intra-cell

        - Uniform measurement approach for cells in the same frequency layer

      - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell

        - CSSF

        - MG

      - Scenario 4: FR2 to FR2 intra-frequency temporal domain case A, Intra-cell

        - Uniform measurement approach for cells in the same frequency layer

  - Proposal 8 (Xiaomi):

    - Observation window and prediction window defined in RAN2 will have RRM impact:

      - Observation window will have impact on measurement duration

      - Prediction window will have impact on scheduling restriction

  - Proposal 9 (Samsung):

    - RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction.

  - Proposal 10 (MediaTek):

    - Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility

    - Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

- Recommended WF

  - RAN4 should wait for further progress in RAN2 before starting discussion on core requirements in RAN4.

#### Issue 2-1-8: Impacts on cell identification

- Proposals and Observations:

  - Proposal 1 (NTT Docomo):

    - RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML.

  - Proposal 2 (NTT Docomo):

    - For PSS/SSS sync,

      - In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results.

      - In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases.

  - Proposal 3 (Qualcomm):

    - RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g.

      - Common aspects:

        - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation.

- Recommended WF

  - Discuss above listed proposals.

#### Issue 2-1-9: Impacts from observation and prediction windows

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - RAN4 to consider the impact of UE speed, PW length, MRRT or skipping patterns on intra-frequency measurement prediction accuracy requirements.

  - Proposal 2 (Ericsson):

    - RAN4 to study relationship between the OW length and PW length and the impact of the number of measurements performed within the OW on the accuracy of the predicted measurements for the instances inside the PW for temporal prediction cases.

  - Proposal 3 (Huawei):

    - For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

    - For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT with limited PW length may need to be specified.

  - Proposal 4 (Apple):

    - Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

  - Proposal 5 (Xiaomi):

    - For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

  - Proposal 6 (Samsung):

    - For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for temporal domain prediction case A.

    - For the RRM impact on prediction window, at least the following may need to be discussed

      - Scheduling restriction

      - Measurement latency for MO (s), MO (s) reduction

  - Proposal 7 (Nokia):

    - RAN4 should investigate impacting factors on the selection of observation and prediction windows for RAN4 requirements and test cases specification.

- Candidate options:

  - Option 1: RAN4 to consider the impact of PW length on prediction accuracy requirements for temporal predictions.

  - Option 2: RAN4 to consider the impact of OW length on prediction accuracy requirements for temporal predictions.

  - Option 3: RAN4 to consider the impact of OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - Option 4: RAN4 to not consider the impact of OW and PW lengths on prediction accuracy requirements for temporal predictions.

- Recommended WF

  - Discuss above listed candidate options.

#### Issue 2-1-10: RAN4 requirements for Measurement event prediction use case

- Proposals and Observations:

  - Proposal 1 (Ericsson):

    - RAN4 to study reporting delay requirement for predicted event triggered reporting.

    - Performance requirement for direct event prediction needs to be defined for event prediction

  - Proposal 2 (Qualcomm):

    - RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction:

      - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction.

      - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP.

      - The above two metrics may jointly serve as a performance assessment metric for event prediction, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2:

        - Whether the AI model is for indirect or direct event prediction.

        - What metrics are to be reported to the serving cell

- Recommended WF

  - Since RAN2 hasn’t progressed sufficiently on measurement even prediction use case, RAN4 should postpone the start of the discussion on measurement event prediction use case to RAN4#114bis meeting.

# Topic #3: Testability and interoperability

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2501235 | Huawei, HiSilicon | Discussion on testability and interoperability issues in AIML mobility Proposal 1: For testing goal, the principle of defining the minimum performance can be discussed based on the following two options: - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement. - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters. - FFS: The value of MRRT. - FFS: Other parameters and the values, e.g., the prediction window. Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction. Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap. Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account. Observation 2: Both cell level prediction and L3 filtered beam level prediction sub-use cases are identified in RAN2, and L3 filtered beam level prediction sub-use cases are lower priority. Proposal 3: RAN4 to discussion the testability of Case 1, Case 2 and Case 3. Proposal 4: Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results), Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM. Proposal 5: RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results). |
| R4-2502072 | Nokia | Testability and Interoperability Issues for AIML Mobility Proposal 1: RAN4 should define test setups considering L1 or L3 measurements from serving and neighbouring cells with related configuration (inputs, prediction horizon). Observation 1: RAN4 should consider the additional aspects for testing (compared to the legacy test) in AI/ML mobility such as L3 filtering, channel model, maximum number of cells, Tx beams and the number of AoAs. Proposal 2: The legacy test setup may be considered as baseline for intra cell scenarios (i.e. scenarios 2 and 4 with sub-use case 2). RAN4 should consider higher number of cells for inter-frequency inter-cell scenario (i.e. scenario 3). - Proposal 3: For FR1, RAN4 should emulate radio channel variations by considering fading channels such as CDL or TDL to test and evaluate the prediction accuracy of AIML models. Proposal 4: For FR2, the testing setup and channel model in AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing. Observation 2: Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup. Observation 3: Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E. Proposal 5: RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases. Proposal 6: RAN4 should consider Options 1 and 2 to study how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test. Proposal 7: RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI. Proposal 8: RAN4 should wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4. |
| R4-2501341 | Vivo | Discussion on testability and interoperability for AI mobility Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion. Observation 2: There are no testability issues for the conducted testing in scenario 2 and 3. Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system. Proposal 1: RAN4 to consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell. Proposal 2: RAN4 to consider using AWGN/TDL channel model as the baseline for testing. Proposal 3: RAN4 to consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power. Proposal 4: RAN4 to take the potential testability setups for the 3 scenarios into consideration as the following: scenario number; Sub-scenario; Frequency Range; Test method; Channel model (as start point); Min cell number; Min probe number; SNR; UE trajectory. 2; 1; FR1; Conducted; AWGN/TDL; 1 serving cell; N/A; legacy SNR; Change doppler and SNR. 2; 2 (1 serving cell, 1 neighbour cell). 3; 1; FR1; Conducted; AWGN/TDL; 2 (1 serving cell, 1 neighbour cell); N/A; legacy SNR; Change doppler and SNR. 2; 3 (2 co-located neighbour cell, 1 serving cell). 4; 1; FR2; OTA; AWGN/TDL; 1 serving cell; 1; Post SNR of the strongest beam pair; Change doppler, SNR, UE rotation, and change probe power. 2; 2 (1 serving cell, 1 neighbour cell); 2. Observation 4: A simplified RAN4 test system would skip Tx beams and the input dimension mismatch issue may happen in the lab. Proposal 5: RAN4 to consider evaluating the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance. |
| R4-2500231 | Apple | On Testability and Interoperability Issues for AIML Mobility in NR Observation 1: The primary challenge in achieving generalization for AI/ML models in AI/ML mobility is ensuring that a model trained with specific cell or inter-cell characteristics can perform effectively across different cells and inter-cell environments. A critical question is whether an AI/ML model trained in one deployment scenario can be directly applied to a different scenario without retraining. Addressing this challenge is essential, as it could reduce the need for multiple AI/ML models, thereby enhancing adaptability and robustness in diverse and realistic radio conditions (SNR). Proposal 1: RAN4 should define performance metrics/KPIs and clarify the objectives, testing properties, and methodologies for evaluating AI/ML functionalities across each mobility use case and sub-use case addressing both Goal 1 (measurement reduction) and Goal 2 (mobility optimization): - 1st goal: To assess the trade-off between measurement reduction and measurement prediction accuracy and its effect to mobility performance with legacy as a baseline with MRRT, SNR as side conditions - 2nd goal: To determine if the AI/ML model achieves measurable improvements in handover (HO) performance with PW, OW, SNR as side conditions Proposal 2: For temporal domain prediction, to investigate whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes. Proposal 3: Define multiple test iterations, each designed to reflect unique, time-varying conditions such as reception power and SNR influenced by UE trajectory, to ensure coverage of potential conditions in AI/ML-assisted mobility scenarios Proposal 4: For Rel-19 AI/ML mobility, it is essential for RAN4 to investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers. Proposal 5: RAN4 should investigate the side conditions for RSRP prediction pertinent to AI/ML mobility, along with network assistance information required to guarantee generalization and effective operation across multiple target cells and different frequency layers. Additionally, it should leverage the discussions in AI/ML beam management regarding the signaling of associated IDs to ensure consistency between primary and secondary carrier components, thereby enhancing generalization capabilities. - Proposal 6: RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility. |
| R4-2500697 | ZTE Corporation | Discussion on the Interoperability and testability aspects Observation 1: One side model is considered in AI mobility. For one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happened between UE and NW mainly depended on signalling. Observation 2: Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition. Proposal 1: For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test. Proposal 2: For FR2, the definition of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP). Proposal 3: The testability discussion of sub-use-case 1 (L1-L1) and sub-use-case 3 (L1-L3) should wait the progress of the testability study in AI-BM. And RAN4 can start the discussion on sub-use-case 2 (L3-L3). Proposal 4: For testing on AI mob, prefer to use AWGN/TDL channel model as baseline. Observation 3: the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility. Proposal 5: RAN4 should discuss how to reflect randomness and variability in the testing process, such as reception power and SNR influenced by mobility. Observation 4: Genralization performance is important for prediction, RAN2 thinks that AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found, and RAN4 should discuss whether/how to define and test the generalization performance of AI/ML model. Proposal 5: Generalization discussion in RAN4 can be started if sufficient progress in RAN2 or NR_AIML_Air WI. Proposal 6: From the perspective of test, core part and performance part requirements should be considered, i.e. measurement delay and measurement accuracy. Proposal 7: RAN4 to discuss numbers of Tx beam/SSBs to be configured and the number of cell to be measured for AI/ML mobility. |
| R4-2500891 | MediaTek Inc. | Discussion on testability and interoperability of AI mobility Proposal 1: Further check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility. Proposal 2: In FR1, computer simulations alike test setup can be adopted for AI mobility test. The following figure can be used to determine the power transmitted at TE on each cell, i.e., TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. |
| R4-2502220 | Qualcomm Incorporated | Testability and data consistency Proposal 1: The following measurement points should all be included in the scope of the testability study: - L3 cell level RSRP (Point C) - L3 beam-level (Point E) - L1 beam-level (Point A1) Proposal 2: RAN4 to further study the impact of time-varying channels and SNR due to fading and UE mobility on the definition of ground truth L3-RSRP. Proposal 3: RAN4 to study how and to what extent data consistency during training and inference, and between serving cell configuration (e.g. TCI state update, TA updates, etc) and dynamic radio environment, needs to be ensured. |
| R4-2500628 | Xiaomi | Discussion on Study of testability and interoperability for AI/ML mobility Proposal 1: For testing in FR1 RRM measurement prediction, the configuration of 1 Tx and TDL channel model can be considered. Proposal 2: For testing in FR1 RRM measurement prediction, RAN4 can assume that the actual ground truth is available at TE. Proposal 3: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied. Proposal 4: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline. |
| R4-2501041 | CATT | Discussion on testability and interoperability issues for AIML mobility Proposal 1: For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS. Proposal 2: For scenario 4, RAN4 further discusses how to define HO performance metric. Proposal 3: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs. Proposal 4: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes, which makes ground truth known to TE. The range of TX power can be FFS. Proposal 5: AWGN or TDL channel can be used in FR1 conducted test. Proposal 6: The following values can be a baseline for the number of configured cells in FR1 HO performance test: - [9] cells for intra-frequency predictions; - [5] cells for inter-frequency predictions; - And values of carrier frequencies can be determined later. Proposal 7: For measurement reduction test, less cells can be set up in test, e.g., [3] cells. Proposal 8: RAN4 to postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM. Proposal 9: Colocation can be reflected by a minor RSRP difference between two carriers. |
| R4-2501134 | OPPO | Study of testability and interoperability for AI mobility Proposal 1: RAN4 to discuss how to model the time-varying characteristics for temporal prediction (e.g., according to UE trajectories or changing the channel model with a stable transmission power). Proposal 2: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused as baseline, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it. Proposal 3: For inter-carrier scenario (inter-frequency inter-cell prediction), the following aspects need to be discussed: - how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted. - how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship. - channel modelling used for inter-cell prediction Proposal 4: RAN4 to consider including L3 filter coefficient as part of the testing side conditions. Proposal 5: RAN4 to consider how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements. Proposal 6: For intra and inter frequency measurement prediction, RAN4 to use legacy testing setup as baseline and include the AI specific parameters if needed. Proposal 7: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used. Proposal 8: The methodology of generalization discussion in NR_AIML_Air WI can be used as starting point for AI mobility. Proposal 9: Wait for more progress from RAN2 on the generalization for AI mobility. |
| R4-2501101 | CMCC | Discussion on testability and interoperability for AI/ML for mobility Proposal 1: for test on intra-frequency temporal domain predictiontes (scenario 2 and 4), the test setup for legacy mobility can be used as baseline but with modification, since test setup for legacy mobility is based on two cells while AI/ML test targets for intra-cell. Proposal 2: for test on inter-frequency frequency domain prediction (scenario 3), it is proposed to discuss whether leagcy test setup based on two cells can be used as baseline for cluster based approach, since RAN2 agree to focus on frequency domain for cluster based approach. Proposal 3: for FR1 AI/ML mobility, it is proposed that the ground truth is the transmit power settled by TE. Proposal 4: for AI/ML Mobility testing in FR2, it is proposed to take the outcome of AI/ML BM use case as baseline. |
| R4-2501675 | Samsung | General discussion on AI mobility regarding testability and interoperability Observation 1: For FR2 testing, on the exact AoA to be used, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage Proposal 1: RAN4 to consider the following testability aspects for AI mobility - Consideration of serving and neighbour cells - Multi-cell setup and number of cells to measure - The legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline Proposal 2: Two directions can be considered to define ideal RSRP. - Direction 1: Settled by TE. The ideal RSRP is of the transmit power. - Direction 2: Settled by measurement report. The ideal RSRP can be derived based on the RSRP measurements reported by UE for the noise-free or very high SNR conditions. For the both directions above, emulation of noise-free/high SNR test conditions and assumption of single signal source are supported Proposal 3: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition. Proposal 4: For FR2 testing in AI/ML mobility, at least the following aspects can be studied: - Emulation of the propagation conditions - AWGN and TDL - Emulation of channel models - The number of emulated transmission reception points (TRxP) - Emulation of noise and EPRE - AoA setup - Rx beam direction assumption Proposal 5: For FR2 testing in AI/ML mobility, multiple beam environment could be emulated Proposal 6: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline - RAN4 to discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study Proposal 7: For FR2 testing in AI mobility, if the test is to mostly verify the RRM RSRP prediction performance, RAN4 to study the feasibility of testing in beam peak direction on the exact AoA to be used Proposal 8: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements - Reuse the principle/methodologies on generalization achieved in AI/ML for NR air interface as much as possible |
| R4-2501761 | Ericsson | On testability issues related to AI/ML for mobility Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility. Proposal 2: The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet: - requirement for the measurements performed inside the OW, - requirement for the measurements predicted for the time instances inside the PW. Proposal 3: The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE: - meet the requirements for measured cell, and - meet the requirements for the target cell. Proposal 4: Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3. - Cell 1 in the test can be considered as the cell for measurement. - Cell 2 in the test can be considered as the target cell for measurement prediction. Proposal 5: For testing in FR1, ground truth of the predicted L3-RSRP value is calculated for the conducted testing setup in place for testing. Proposal 6: Consider OTA testing as baseline option for testing in FR2. Proposal 7: Ground truth value of L3-RSRP measurement in FR2 is the measurement performed by the UE under a sufficiently high side condition under the assumption that the resulting value is close to the true value. RAN4 to further discuss side condition to derive ground truth value for predicted L3-RSRP. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: New Testability aspects

Sub-topic description:

In this sub-topic the company proposals about the testability aspects for AI/ML Mobility use cases are summarized.

#### Issue 3-1-1: Testing Setup in FR1

- Proposals and Observations:

  - Proposal 1 (Huawei):

    - Discuss the testability of Case 1, Case 2 and Case 3

      - Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results), Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM.

      - Reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results)

  - Proposal 2 (Nokia):

    - Legacy test setup may be considered as baseline for intra cell scenarios (i.e. scenarios 2 and 4 with sub-use case 2).

    - Emulate radio channel variations by considering fading channels such as CDL or TDL to test and evaluate the prediction accuracy of AIML models.

    - Consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

  - Proposal 3 (Vivo):

    - Consider using AWGN/TDL channel model as the baseline for testing

    - Consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turntable, and change probe power

    - Consider the potential testability setups in FR1:

| Scenario number | Sub-scenario | Freq Range | Test method | Channel model (as start point) | Min cell number | Min. probe number | SNR | UE trajectory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | FR1 | Conducted | AWGN/ TDL | 1 serving cell | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbor cell) |  |  |  |
| 3 | 1 | FR1 | Conducted | AWGN/ TDL | 2 (1 serving cell, 1 neighbor cell) | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 3 (2 co-located neighbor cell, 1 serving cell) |  |  |  |

  - Proposal 4 (Apple):

    - Define multiple test iterations, each designed to reflect unique, time-varying conditions such as reception power and SNR influenced by UE trajectory, to ensure coverage of potential conditions in AI/ML-assisted mobility scenarios

  - Proposal 5 (ZTE):

    - Use AWGN/TDL channel model as baseline.

    - Discuss how to reflect randomness and variability in the testing process, such as reception power and SNR influenced by mobility

    - Testability discussion of sub-use-case 1 (L1-L1) and sub-use-case 3 (L1-L3) should wait the progress of the testability study in AI-BM. And RAN4 can start the discussion on sub-use-case 2 (L3-L3).

    - Consider core and performance part requirements, i.e. measurement delay and measurement accuracy from the perspective of test

    - Discuss numbers of Tx beam/SSBs to be configured and the number of cell to be measured for AI/ML mobility

  - Proposal 6 (MediaTek):

    - Adopt computer simulations alike test setup for AI mobility test in FR1, e.g., TE can transmit the signals of cell 1, 2, and 3 with time-variance following a certain channel model.

  - Proposal 7 (Xiaomi):

    - For testing in FR1 RRM measurement prediction, the configuration of 1 Tx and TDL channel model can be considered.

  - Proposal 8 (CATT):

    - RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

    - AWGN or TDL channel can be used in FR1 conducted test.

  - Proposal 9 (Oppo):

    - Discuss how to model the time-varying characteristics for temporal prediction (e.g., according to UE trajectories or changing the channel model with a stable transmission power).

    - Include L3 filter coefficient as part of the testing side conditions.

    - Discuss how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements

    - For intra and inter frequency measurement prediction, RAN4 to use legacy testing setup as baseline and include the AI specific parameters if needed

  - Proposal 10 (CMCC):

    - For scenario 2 and 4, the test setup for legacy mobility can be used as baseline but with modification, since test setup for legacy mobility is based on two cells while AI/ML test targets for intra-cell.

    - For scenario 3, discuss whether legacy test setup based on two cells can be used as baseline for cluster based approach, since RAN2 agree to focus on frequency domain for cluster based approach.

  - Proposal 11 (Samsung):

    - Define ideal RSRP as settled by TE (ideal RSRP is of the transmit power) )

    - Emulate noise-free/high SNR test conditions and assume single signal source

  - Proposal 12 (Ericsson):

    - Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

      - Cell 1 in the test can be considered as the cell for measurement.

      - Cell 2 in the test can be considered as the target cell for measurement prediction

    - Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility.

- Candidate Options:

    - Option 1: Study how legacy testing setup for L3 measurements can be reused

    - Option 2: Consider multi-beam scenarios

    - Option 3: Discuss the channel model for testing: AWGN, TDL, and/or CDL

- Recommended WF

    - To discuss above listed candidate options.

#### Issue 3-1-2: Prediction consistency in time domain

- Proposals and Observations:

  - Proposal 1(Apple):

    - Investigate temporal domain prediction by considering whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes

  - Proposal 2(MediaTek):

    - Check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility

  - Proposal 3(Qualcomm):

    - Study how and to what extent data consistency during training and inference, and between serving cell configuration (e.g. TCI state update, TA updates, etc) and dynamic radio environment, needs to be ensured.

  - Proposal 4(Nokia):

    - Consider Options 1 and 2 to study how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test.

  - Proposal 5(CATT):

    - TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes. The range of TX power can be FFS.

  - Proposal 6(OPPO):

Discuss how to model the time-varying characteristics for temporal prediction (e.g., according to UE trajectories or changing the channel model with a stable transmission power).

- Candidate Options:

    - Option 1: Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation

    - Option 2: Discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories

    - Option 3: Model time-variant RSRP changes by randomly adjusting TE’s transmitting power of SSBs from all cells

- Recommended WF

    - To discuss above listed candidate options.

#### Issue 3-1-3: Considering multiple cells in the testing setup

- Proposals and Observations:

  - Proposal 1 (Xiaomi):

    - For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied

  - Proposal 2 (CATT):

    - Later determine values of carrier frequencies and the number of configured cells in FR1 HO performance test is:

      - [9] cells for intra-frequency predictions;

      - [5] cells for inter-frequency predictions;

    - For measurement reduction test, less cells can be set up in test, e.g., [3] cells

  - Proposal 3 (Samsung):

    - Consider serving and neighbor cells

    - Multi-cell setup and number of cells to measure: the legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline

  - Proposal 4 (Apple):

    - Investigate the side conditions for RSRP prediction pertinent to AI/ML mobility, along with network assistance information required to guarantee generalization and effective operation across multiple target cells and different frequency layers.

  - Proposal 5 (Nokia):

    - Consider higher number of cells for inter-frequency inter-cell scenario (i.e. scenario 3).

    - Define test setups considering L1 or L3 measurements from serving and neighboring cells with related configuration (inputs, prediction horizon).

- Candidate Options

    - Option 1: Study multi-cell setup in the test  for inter-frequency predictions

    - Option 2: Consider at least two cells: serving cell and another/targe cell for inter-cell measurement prediction

    - Option 3: Specify that [9] cells for intra-frequency predictions; [5] cells for inter-frequency predictions. For measurement reduction test, less cells can be set up in test, e.g., [3] cells.

    - Option 4: Consider any side condition for multiple cells

- Recommended WF

    - To discuss above listed candidate options.

#### Issue 3-1-4: Ground Truth derivation in FR1

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - The ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

  - Proposal 2 (CMCC):

    - The ground truth is the transmit power settled by TE.

  - Proposal 3(CATT):

    - ground truth is known to TE when TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes

  - Proposal 4(ZTE):

    - For FR1, the transmitted power can be used as baseline for the definition on ground truth of L3-RSRP, which is obtained as the legacy way of conducted test

  - Proposal 5(Xiaomi):

    - For testing in FR1 RRM measurement prediction, RAN4 can assume that the actual ground truth is available at TE.

  - Proposal 6(Samsung):

    - For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition.

      - FFS: The power level at which SS-RSRP measurement is conducted

      - FFS: The exact SNR level emulated

  - Proposal 7 (Ericsson):

    - In FR1, ground truth of the predicted L3-RSRP value is calculated for the conducted testing setup in place for testing.

  - Proposal 8 (Qualcomm):

    - Study the impact of time-varying channels and SNR due to fading and UE mobility on the definition of ground truth L3-RSRP

- Candidate Options

    - Option 1: the ground truth is the transmit power settled by TE, i.e., ground truth is known by the TE.

    - Options 2: actual measurement report based on legacy procedure is used to derive the ground truth

    - Option 3: study the impact of time-varying channels and SNR on the ground-truth definition

- Recommended WF

    - This issue can follow Issue 2-1-2.

### Sub-topic 3-2: Testing goals and related aspects

Sub-topic description:

In this sub-topic the company proposals about the testability and interoperability aspects for AI/ML Mobility use cases are summarized.

#### Issue 3-2-1: Testing goal

Background:

Following agreement was reached in RAN4#113 meeting:

- Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

- Proposals and Observations:

  - Proposal 1 (Huawei):

    - For testing goal, the principle of defining the minimum performance can be discussed to verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement or within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

      - FFS: The value of MRRT and other parameters, e.g., the prediction window.

    - Measurement prediction requirements consider the gain derived from power consumption reduction and measurement gap reduction

  - Proposal 2 (Vivo):

    - Evaluate the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance

  - Proposal 3 (Apple):

    - Define performance metrics/KPIs by clarifying the objectives, testing properties, and methodologies for evaluating AI/ML functionalities across each mobility use case and sub-use case addressing both Goal 1 (measurement reduction) and Goal 2 (mobility optimization)

      - 1st goal: To assess the trade-off between measurement reduction and measurement prediction accuracy and its effect to mobility performance with legacy as a baseline with MRRT, SNR as side conditions

      - 2nd goal: To determine if the AI/ML model achieves measurable improvements in handover (HO) performance with PW, OW, SNR as side conditions

  - Proposal 4 (Ericsson):

    - The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

      - requirement for the measurements performed inside the OW,

      - requirement for the measurements predicted for the time instances inside the PW.

  - Proposal 5 (CATT):

    - For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

    - For scenario 4, RAN4 further discusses how to define HO performance metric.

- Candidate Options:

  - Option 1: Update in the agreed definition of testing goal in RAN4#113 is needed

  - Option 2: No update in the agreed definition of testing goal in RAN4#113 is needed.

  - In case of Option 1, discussed above listed proposals as candidate Options.

- Recommended WF

  - Since RAN4 agreed in RAN4#113 that testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved. This definition is same as agreed for NR_AIML_air WI. Therefore, it should be first discussed if any update in the definition is required.

#### Issue 3-2-2: Principle of FR2 testing

- Proposals and Observations:

  - Proposal 1 (Ericsson):

    - Consider OTA testing as baseline option for testing in FR2

    - Ground truth value of L3-RSRP measurement in FR2 is the measurement performed by the UE under a sufficiently high side condition under the assumption that the resulting value is close to the true value. RAN4 to further discuss side condition to derive ground truth value for predicted L3-RSRP.

  - Proposal 2 (CMCC):

    - Take the outcome of AI/ML BM use case as baseline for FR2

  - Proposal 3(Xiaomi):

    - For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

  - Proposal 4(OPPO):

    - For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused as baseline, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

  - Proposal 5 (ZTE):

    - For FR2, the definition of ground truth on R19 AI BM can be reused, that is, ground truth of L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP)

  - Proposal 6 (Nokia):

    - For FR2, the testing setup and channel model in AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing.

    - Consider the additional aspects for testing (compared to the legacy test) in AI/ML mobility such as L3 filtering, channel model, maximum number of cells, Tx beams and the number of AoAs.

  - Proposal 7 (Vivo):

    - Consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

    - Consider the potential testability setups in FR2:

| Scenario number | Sub-scenario | Freq Range | Test method | Channel model (as start point) | Min cell number | Min. probe number | SNR | UE trajectory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1 | FR2 | OTA | AWGN/ TDL | 1 serving cell | 1 | Post SNR of the strongest beam pair | Change doppler, SNR, UE rotation, and change probe power |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbor cell) | 2 |  |  |

  - Proposal 8 (CATT):

    - Postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM.

  - Proposal 9 (Samsung): For FR2 testing in AI/ML mobility, at least the following aspects can be studied:

    - Emulation of the propagation conditions

    - Emulation of channel models

    - The number of emulated transmission reception points (TRxP)

    - Emulation of noise and EPRE

    - AoA setup

    - Rx beam direction assumption

  - Proposal 10 (Samsung): For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline

    - RAN4 to discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study

  - Proposal 11 (Samsung): For FR2 testing in AI mobility, if the test is to mostly verify the RRM RSRP prediction performance, RAN4 to study the feasibility of testing in beam peak direction on the exact AoA to be used

- Recommended WF

  - For AI/ML Mobility testing in FR2, use testing setup under consideration for AI/ML BM use case as a baseline.

#### Issue 3-2-3: Testing of Generalization aspects

- Proposals and Observations:

  - Proposal 1 (Apple):

    - Investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers

    - Investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers

  - Proposal 2 (ZTE):

    - Start to discuss generalization in RAN4 if sufficient progress in RAN2 or NR_AIML_Air WI.

  - Proposal 3 (Samsung):

    - Study generalization related tests/requirements

    - Reuse the principle/methodologies on generalization achieved in AI/ML for NR air interface as much as possible

  - Proposal 4 (Apple):

    - Leverage the discussions in AI/ML beam management regarding the signaling of associated IDs to ensure consistency between primary and secondary carrier components, thereby enhancing generalization capabilities.

  - Proposal 5 (OPPO):

    - The methodology of generalization discussion in NR_AIML_Air WI can be used as starting point for AI mobility.

    - Wait for more progress from RAN2 on the generalization for AI mobility

  - Proposal 6 (Nokia):

    - Wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4

- Candidate options:

  - Option 1: Discuss whether to start to discuss generalization in RAN4 or wait for more progress from RAN2

  - Option 2: Use the generalization in NR_AIML_Air WI as a starting point when it is possible to reuse

- Recommended WF

  - Discuss the options listed above.

#### Issue 3-2-4: Testing aspects of inter-carrier scenarios

- Proposals and Observations:

  - Proposal 1 (Nokia):

    - Study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI

  - Proposal 2 (CATT):

    - Colocation can be reflected by a minor RSRP difference between two carriers.

  - Proposal 3 (OPPO):

    - Discuss how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted.

    - Discuss how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship.

    - Discuss channel modelling used for inter-cell prediction

  - Proposal 4 (Ericsson):

    - The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE:

      - meet the requirements for measured cell, and

      - meet the requirements for the target cell.

- Recommended WF

  - Above listed proposals need to be further discussed.