3GPP TSG-RAN WG4 Meeting # 114					       			       			R4-2500538

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.25.3

Source:	Moderator (Huawei)

Title:	Topic summary for [114][224] Netw_Energy_NR_enh_Part2

Document for:	Information

# Introduction

This topic summary includes RRM core requirements for Rel-19 network energy saving (7.25.2.2).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 2-1-1: RRM requirements impacts for OD-SIB1

Issue 1-2-1: Applicable L1 requirements for SSB adaptation

Issue 1-2-2: Transition requirements applicability

Issue 1-2-3: Transition requirements – details

Issue 1-4-1: SCell activation requirement for SSB adaptation

# Topic #1: Adaptation of common signal/channel transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500057 | Qualcomm Technologies Ireland | Observation 1: RAN1 confirmed that SSB adaptation means adaptation of burst periodicity. There is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. It was confirmed that there is no change in transmitted SSB before and after SSB adaptation. Proposal 1: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged. Proposal 2: RAN4 should not define requirements for SSB adaptation for PCell Observation 2: No information has yet been received from RAN2 regarding the impact on SMTC. Proposal 3: RAN4 should postpone discussions about impacts on L3 requirements and focus on L1 requirements. Observation 3: In our understanding the RAN4 agreement means that during the transition period, i.e., direct after the SSB burst periodicity has been changed, the requirements apply with a periodicity of max{P1, P2} instead of P1. That means the requirement is potentially relaxed. Observation 4: The transition period can be defined as Tfirst_SSB + meas_period(P2). Tfirst_SSB is the time till the first SSB with the new period P2 is received. meas_period(P2) is the measurement period based on the new periodicity P2. Proposal 4: RAN4 should analyze whether Tfirst_SSB + meas_period(P2) can serve as transition period for all requirements. RAN4 should also discuss whether a general definition of the transition time is sufficient for ich relaxed requirements apply without defining new test cases. |
| R4-2500389 | MediaTek inc. | Proposal 1: Due to UL-WUS transmission for OD-SIB1, RAN4 further discuss the impact on paging interruption requirement in cell re-selection. One possible approach is to extend the interruption time to include the UL WUS transmission. |
| R4-2500457 | Apple | Proposal 1: RAN4 work of this objective is based on legacy SSB burst periodicity (5ms to 160ms). Proposal 2: RAN4 work of this objective shall focus on SSB burst periodicity adaptation. This can be revisited if RAN1 reaches further agreement. Proposal 3: applicable L1 requirements in SSB adaptation shall include the following L1 operations for serving cell: - L1-RSRP for serving cell - L1-SINR - BFD - CBD Proposal 4: not consider L1 impact of SSB adaptation in mTRP and LTM scenario. |
| R4-2500636 | Xiaomi | Observation 1: In case of M=1 and P=1 (no gap sharing), only single SSB need to be considered within a successful measurement period. That is L1 measurement requirements at SSB transition defined by the equation (1) is not applicable. Proposal 1: L1 measurement requirement at transition agreed could not applicable for L1 measurement when M=1 and P=1. Observation 2: the transition interval between gNB signaling on the SSB adaptation and next latest SSB could significantly impact on L1/L3 measurement in case of adaptation SSB. Proposal 2: For L1/L3 measurement under SSB adaption, the start point of measurement period shall be clarified. Proposal 2a: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. Observation 3: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message can be reused for L1/L3 measurement Observation 4: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW. Proposal 3: L3 measurement requirements with the adaptation SSB can be FFS upon LS reply on whether there is impact on SMTC. |
| R4-2500773 | LG Electronics Inc. | Proposal 1: Defer L3 measurement requirement pending on RAN2 progress Proposal 2: RAN4 to preclude PCell from the scenario Proposal 3: Apply indicated period of SSB adaptation for following L1 measurement requirement - L1-RSRP - L1-SINR - L1 measurement during SCell activation - TCI state switching delay - Beam failure detection, Candidate beam detection |
| R4-2500849 | CMCC | Proposal 1: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that: - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 Proposal 2: The L1 transition requirements will be applied to BFD, L1-RSRP and L1-SINR measurement of serving cell. Proposal 3: No requirement for the case that SSB adaptation during SCell activation. Proposal 4: Legacy requirement can be reused for the case that SSB adaptation before SCell activation. Proposal 5: UE shall use the adapted SSB for SCell activation for the case of SSB adaptation happens at the same time as SCell activation. Proposal 6: No requirement for the case that SSB adaptation during TCI state switching. |
| R4-2500908 | ZTE Corporation, Sanechips | Proposal 1: According to the LS reply and latest progress in RAN1, focus on the SSB adaptation in SCell with the assumption of same frequency and same QCL. Proposal 2: The transition requirements for L1 measurement are applicable for L1-RSRP, L1-SINR, BFD, CBD, and TCI state switching measurements. Proposal 3: For the L3 measurement impact, RAN4 continues to wait for RAN2 LS reply and meanwhile internally discuss the potential L3 measurement impact with SMTC configuration update or not. |
| R4-2501018 | CATT | Proposal 1: Regarding the impact on L3 measurement requirements due to SSB adaptation, RAN4 continue to wait for RAN2's response on the SMTC configuration. Proposal 2: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. Observation 1: There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) in RAN1#119, which means that RAN1 will not continue to discuss the issue. Proposal 3: RAN4 will not to define the SSB adaptation requirement for PCell in Rel-19. |
| R4-2501127 | OPPO | Observation 1: Any SMTC impact should be considered for OD-SSB and SSB adaptation together. Proposal 1: SMTC adaptation is not expected from UE’s perspective due to dynamic SSB adaptation or OD-SSB activation/deactivation. |
| R4-2501221 | Huawei, HiSilicon | Observation 1: Based on RAN1 progress so far, following scenario are supported: - SSB adaption in SCell - Time domain: SSB burst periodicity can be adapted - Frequency domain: same frequency location before and after SSB adaption - QCL: Same SSB index are QCLed when actually transmitted SSBs within a burst before and after SSB are same Proposal 1: RAN4 take following assumption for further discussion, which can be further updated accordingly with further RAN1 progress. - SSB adaption in SCell - Time domain: SSB burst periodicity can be adapted - Frequency domain: same frequency location before and after SSB adaption - QCL: Same SSB index are QCLed when actually transmitted SSBs within a burst before and after SSB are same Proposal 2: Only define transition requirements of SSB adaption for BFD/CBD. For SSB adaption for L1-RSRP/L1-SINR, RAN4 to considering following options: - Option 1: UE to combine the SSB before and after adaption - Option 2: UE to restart measurement using the SSB periodicity after adaptation from the time of adaptation. Proposal 3: For SSB adaptation in L3 measurement, RAN4 assumes that UE is still expected to perform measurement based on SMTC. RAN4 to wait for RAN2 LS reply to decide whether there is RAN4 requirements impacts (e.g. whether to define requirements at transition). Observation 2: Based on latest progress on SSB adaption scenario, OD-SSB and SSB adaption share a lot common characteristics (e.g. Only for SCell, and at least SSB periodicity adaption is supported) Proposal 4: Similar as OD-SSB agreements, UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. - Detailed triggering on SSB adaptation depends on further RAN1 discussion. - FFS whether to consider SSB adaption happens during SCell activation, which can be aligned with the progress in OD-SSB. Observation 3: In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption. |
| R4-2501305 | Ericsson | Proposal 1: RAN4 not to define the SSB adaptation requirement for PCell in Rel-19. Proposal 2: RAN4 to only focus on SSB periodicity adaptation. Proposal 3: RAN4 to wait RAN2’s LS response about SMTC adaptation in L3 measurement. Proposal 4: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. Proposal 5: RAN4 to define L1 based SSB adaptation requirement for BFD, CBD and L1-RSRP. |
| R4-2501336 | China Telecom | Proposal 1: No requirements when SSB adaptation happens during SCell activation. Proposal 2: No requirements when SSB adaptation happens during TCI state switching. |
| R4-2501602 | vivo | Proposal 1 RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs CBD. Proposal 2 RAN4 NOT to discuss the SSB adaptation impact on BFD, since the SCell BFD requirements are only applicable when the RSs are periodic CSI-RSs. RAN4 may further discuss impacts on RLM/BFD requirements if RAN1 agrees SSB adaptation on SpCell. Proposal 3 The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 postpone corresponding discussion until RAN1/2 make conclusions. Proposal 4 RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. Proposal 5 For impacts to L3 measurements, RAN4 to wait for RAN2 conclusion regarding whether there is impact to SMTC due to SSB adaptation. |
| R4-2501676 | Samsung | Observation 1: Based on RAN1 agreements on SSB adaptation: - PCell (connected mode) are not supported for adaptation mechanisms of SSB in time-domain - Frequency domain adaptation (e.g., inter-frequency) is precluded in RAN1 Observation 2: Due to the mis-match of adapted SSB and SMTC window, if the adapted SSB triggered by MAC-CE/DCI does not fall into the configured SMTC window, the UE cannot perform L3 measurement based on the adapted SSBs Observation 3: Based on RAN2 discussions, the methods to optimize SMTC configurations for NES can be: - NES dedicated Measurement Object (e.g., MeasObjectNES) with/without new MeasId - Separate/new optional IE for NES configured within the existing MeasObjectNR - Separate/New serving cell configurarions for NES configured within the existing ServingCellConfig/ServingCellConfigCommon - New optional IE configured in SCellConfig to support Scell supporting NES indication Proposal 1: - There is no need to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in Pcell - There is no need to discuss the RRM requirements for the scenarios of frequency domain adaptation Proposal 2: The measurement period is the maximum of the period corresponding to the SSB before and after adaptation - Applicability: SSB adaptation happens during one measurement period for L3 cell-level measurements and L1-RSRP measurement based on more SSB samples (i.e., M=3) Proposal 3: To guarantee the measurement accuracy at transition - UE can to perform measurement based on the SSB/SMTC resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation Proposal 4: RAN4 to discuss how to specify time periods for intra-frequency SCell identification and measurment, and inter-frequency SCell identification and measurment after receiving RAN2 reply LS on SMTC configuration mechanism (s) for NES Proposal 5: For adaptation of SSB transmissions, suggest RAN4 deprioritize the discussion on RRM requirement impact on deactivated SCell measurement/detection - If deactivated SCell measurement/detection is needed based on SSB sources for adaptation, RAN4 to discuss whether the principle defined for OD-SSB can be reused - Proposal 6: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode Proposal 7: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception Proposal 8: For FR2 unknown SCell activation, if there is no active FR2 serving cell supporting SSB adaptation in the band, it can be assumed that UE to use SSB resources for adaptation to active the SCell (w.r.t. obtain AGC) instead of SMTC Proposal 9: When UE receive SCell activation command and SSB adaptation indication command at the same time, for FR2 unknown SCell activation, the activation delay can be shorten by reducing the AGC settling time, cell detection time, and L1 measurement time based on dense SSB periodicity for adaptation Proposal 10: If UE receive SSB adaptation indication command before receiving SCell activation command, RAN4 to discuss whether to/how to define the known cell condition for FR1/FR2 for NES Proposal 11: For Rel-19 NES-capable UE’s SCell - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation |
| R4-2502240 | Nokia, Nokia Shanghai Bell | Scenarios for SSB adaptation Proposal 1: RAN4 to not consider RRM impacts from SSB adaptation for PCell based on current RAN1 agreement. Proposal 2: RAN4 to consider RRM impacts from SSB adaptation for NCD-SSB in SCell based on current RAN1 agreement and to wait on further RAN1 progress on SSB adaptation for CD-SSB in SCell. Proposal 3: RAN4 to identify RRM impacts for SSB adaptation in time domain for SCell and NCD-SSB and limited to SSB periodicity adaptation on the same frequency location. Proposal 4: RAN4 to study RRM impacts due to applying SSB adaptation for OD-SSB Case-1. Proposal 5: RAN4 to specify requirements for SSB adaptation on OD-SSB Case-1 under the SSB adaptation objective. Proposal 6: RAN4 to maintain the specified range of SSB burst periodicities (5ms to 160ms) for SSB periodicity adaptation. Layer 3 measurement impact Proposal 7: RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and NR intra-frequency time index delay, as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell. Layer 1 measurement impact Proposal 8: RAN4 to not consider the L1 measurement impact from SSB periodicity adaptation to RLM. Proposal 9: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation to measurement delay for BFD, CBD, L1-RSRP and L1-SINR for active SCells. Transition requirements Proposal 10: RAN4 to consider transition requirements for L1-RSRP, L1-SINR, BFD and CBD requirements. Proposal 11: RAN4 to consider L3 transition requirements due to SSB periodicity adaptation for single always-on SSB and for OD-SSB Case-1, respectively. SCell activation requirements Proposal 12: RAN4 to not consider SSB periodicity adaptation during SCell activation. TCI switching requirements Proposal 13: RAN4 to not consider an impact from SSB periodicity adaptation to TCI state switching delay. SSB adaptation delay requirements Proposal 14: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1/RAN2, to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 SSB adaptation – Scenario

Issue 1-1-1: Scenario clarification based on RAN1 progress

Background:

RAN4 agreements in RAN4#112bis R4-2416860 Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) RAN1#118 bis Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact RAN4 agreements in RAN4#113 R4-2420242 Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. RAN1#119 Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

- Proposals

  - Option 1: RAN4 should not define requirements for SSB adaptation for PCell. (QC, LG, CATT, Ericsson, Samsung, Nokia)

  - Option 2: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged. (QC)

  - Option 3: RAN4 work of this objective shall focus on SSB burst periodicity adaptation. This can be revisited if RAN1 reaches further agreement.(Apple, Ericsson)

  - Option 4: no need to discuss the RRM requirements for the scenarios of frequency domain adaptation. (Samsung)

  - Option 5: According to the LS reply and latest progress in RAN1, focus on the SSB adaptation in SCell with the assumption of same frequency and same QCL. (ZTE)

  - Option 6: RAN4 to identify RRM impacts for SSB adaptation in time domain for SCell and NCD-SSB and limited to SSB periodicity adaptation on the same frequency location. (Nokia)

- Moderator:

Based on moderator’s understanding, agreements in last RAN4 meeting still hold. In addition, following could be agreed per RAN1 latest agreements.

- Recommended WF:

  - Further clarification on the scope of RRM impact for SSB adaption:

    - No RRM impacts for SSB adaptation requirement for PCell:

    - Frequency location is same before and after SSB adaptation.

    - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation

  - Note: Above could be further updated based on further RAN1 progress if any.

Issue 1-1-2: Scenario clarification- SSB periodicity

- Proposals

  - Option 1: RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements (Apple, Nokia)

- Recommended WF:

  - Postpone the discussion and wait for RAN1 progress.

Issue 1-1-3: Scenario clarification-  Others

- Proposals

  - Option 1: not consider L1 impact of SSB adaptation in mTRP and LTM scenario (if any). (Apple)

  - Option 2: The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 postpone corresponding discussion until RAN1/2 make conclusions

- Recommended WF: Discuss above proposals

### Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-1: Applicable L1 requirements for SSB adaptation

RAN4#112bis R4-2416860 Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

- Proposals

  - Option 1: L1-RSRP (Apple, LGE, CMCC, Ericsson, Nokia, ZTE, Samsung)

    - Option 1a: For serving Cell (Apple, CMCC)

  - Option 2: L1-SINR (Apple, LGE, CMCC, Nokia, ZTE, Samsung)

    - Option 2a: For serving Cell (CMCC)

  - Option 3:	BFD (Apple, LGE, CMCC, Huawei, Nokia, ZTE)

    - No: Vivo

  - Option 4: CBD (Apple, LGE, Huawei, Ericsson, Nokia, ZTE)

    - No: Vivo

  - Option 5: L1 measurement during SCell activation (LGE)

  - Option 6: TCI state switching (ZTE)

  - Option 7: RLM

    - No: Nokia

- Moderator:

  - This issue is to discuss general impact due to SSB adaption. The details on whether and how to define requirements for transition period, and the details on requirements after transition period (if defined) will be discussed in other issues.

  - Impact on TCI/SCell activation/L3 intra-f and inter-f measurement will be discussed in other issues.

  - Recommended WF:

    - Applicable L1 requirements for SSB adaptation:

      - No impact:

        - RLM

        - FFS: BFD

      - No requirements if SSB adaption happens:

        - FFS: CBD

      - Define requirements if SSB adaption happens:

        - L1-RSRP for serving cell

        - L1-SINR

        - FFS: CBD

        - FFS: BFD

Issue 1-2-2: Transition requirements applicability

RAN4#113 agreement Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements.

- Proposals

  - Option 1: CBD: (ZTE, Huawei, Nokia)

  - Option 2: BFD: (CMCC, ZTE, Huawei, Nokia)

  - Option 3: L1-RSRP: (CMCC, ZTE, Nokia)

    - Option 3a: Not apply. UE to combine the SSB before and after adaption. (Huawei)

    - Option 3b: Not apply UE to restart measurement using the SSB periodicity after adaptation from the time of adaptation. (Huawei)

  - Option 4: L1-SINR: (CMCC, ZTE, Nokia)

    - Option 4a: Not apply. UE to combine the SSB before and after adaption. (Huawei)

    - Option 4b: Not apply. UE to restart measurement using the SSB periodicity after adaptation from the time of adaptation. (Huawei)

- Moderator:

- Recommended WF:

  - Define requirements at transition for BFD and CBD which is subject to the conclusion of issue 1-2-1.

  - Discuss whether to define requirement at transition for L1-RSRP/L1-SINR and what the understanding if no requirements at transition.

Issue 1-2-3: Transition requirements – details

- Proposals

  - Option 1: RAN4 should analyze whether Tfirst_SSB + meas_period(P2) can serve as transition period for all requirements. RAN4 should also discuss whether a general definition of the transition time is sufficient for ich relaxed requirements apply without defining new test cases.  (QC)

  - Option 2: L1 measurement requirement at transition agreed could not applicable for L1 measurement when M=1 and P=1. (Xiaomi)

  - Option 3: Applicability: SSB adaptation happens during one measurement period for L3 cell-level measurements and L1-RSRP measurement based on more SSB samples (i.e., M=3)  (Samsung)

  - Option 4: For L1/L3 measurement under SSB adaption, the start point of measurement period shall be clarified. (Xiaomi)

- Recommended WF:

  - Discuss the starting/ending point of transition period.

    - (Moderator): The transition period starts from when SSB adaption occurs, and the uncertainty time Tfirst_SSB till the first SSB is considered, and the transition period is fined as:

      - Option 1: Tfirst_SSB + maximum of the measurement/evaluation period. (agreements in last meeting)

      - Option 2: Tfirst_SSB + measurement/evaluation period of SSB after adapation

  - Discuss whether to differentiate requirements at transition w.r.t value of M and P.

Issue 1-2-4: L1 requirements impact after/without transition

- Proposals

  - Option 1 (Measurement priority with relation to DRX): For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode.  (Samsung)

  - Option 2 (Measurement behaviour at transition): UE can to perform measurement based on the SSB/SMTC resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation (Samsung)

- Recommended WF:

  - Discuss above proposals.

### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation – general

- Proposals

  - Option 1: Wait for L2 inputs on whether there is impact on SMTC. (QC, Xiaomi, LGE, CATT, Huawei, Ericsson, Vivo, Samsung)

  - Option 2: RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and NR intra-frequency time index delay, as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell. (Nokia)

  - Option 3: SMTC adaptation is not expected from UE’s perspective due to dynamic SSB adaptation or OD-SSB activation/deactivation. (Oppo)

  - Option 4: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi)

  - Option 5: For adaptation of SSB transmissions, suggest RAN4 deprioritize the discussion on RRM requirement impact on deactivated SCell measurement/detection (Samsung)

    - If deactivated SCell measurement/detection is needed based on SSB sources for adaptation, RAN4 to discuss whether the principle defined for OD-SSB can be reused

- Recommended WF:

  - Wait for LS reply on whether there is impact on SMTC.

  - Discuss whether to define requirements at transition for L3 measurement base on LS reply.

### Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation

- Proposals

  - Option 1: SSB adaptation during SCell activation:

    - Option 1a: No requirements (CMCC, Ericsson, CTC, Vivo, Samsung, Nokia)

    - Option 1b: No impacts (CATT)

    - Option 1c: FFS which can be aligned with the progress in OD-SSB.

  - Option 2: SSB adaptation before SCell activation:

    - Option 2a: Reuse legacy requirements. (CMCC)

    - Option 2b: UE shall use SSB after adaption for SCell (Huawei)

    - Option 2c: RAN4 to discuss whether to/how to define the known cell condition. (Samsung)

  - Option 3: SSB adaptation happens at the same time as SCell activation:

    - Option 3a: UE shall use the adapted SSB for SCell activation. (CMCC, Huawei, Samsung)

      - Option 3a-1: For FR2 unknown SCell activation, if there is no active FR2 serving cell supporting SSB adaptation in the band, it can be assumed that UE to use SSB resources for adaptation to active the SCell (w.r.t. obtain AGC) instead of SMTC. (Samsung)

  - Option 4: RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation (Samsung).

- Recommended WF:

  - No requirements when SSB adaptation happens during SCell activation.

  - SSB adaptation happens before SCell activation:

    - Option 2a: Reuse legacy requirements.

    - Option 2b: UE shall use SSB after adaption for SCell

    - Option 2c: RAN4 to discuss whether to/how to define the known cell condition.

  - SSB adaptation happens at the same time as SCell activation:

    - Option 3a: UE shall use the adapted SSB for SCell activation.

Issue 1-4-2: TCI state switching requirements for SSB adaptation

- Proposals

  - Option 1: No requirement for the case that SSB adaptation during TCI state switching. (CMCC, CATT, Ericsson, CTC, Vivo, Nokia)

  - Option 2: Apply indicated period of SSB adaptation for TCI state switching delay. (LGE)

    - Option 2a: Transition requirements apply for TCI state switching. (ZTE)

- Recommended WF:

  - No requirements when SSB adaptation happens during TCI state switching.

Issue 1-4-3: Other requirements impacts by SSB adaptation

- Proposals

  - Option 1: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1/RAN2, to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay. (Nokia)

  - Option 2: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi)

- Recommended WF:

  - Discuss above proposal

# Topic #2: OD-SIB1

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2500389 | MediaTek inc. | Proposal 1: Due to UL-WUS transmission for OD-SIB1, RAN4 further discuss the impact on paging interruption requirement in cell re-selection. One possible approach is to extend the interruption time to include the UL WUS transmission. |
| R4-2500849 | CMCC | Proposal 1: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that: - reference cell is the NES cell - NTA for UL-WUS is defined as 0 - is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133 |
| R4-2501600 | vivo | Observation 1 OD-SIB1 related procedure only impacts UE behaviour in RRC idle/inactive states. Observation 2 UE performs cell re-selection to NES cell with OD-SIB1 enabled, when UL WUS triggering condition is met, i.e. UL WUS triggering condition is the same as cell re-selection condition and OD-SIB1 acquisition shall be regarded as a special case of cell re-selection. Proposal 1 RAN4 to further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure. Proposal 2 For R19 OD-SIB1 related procedure, clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional relaxation or additional applicability restrictions (e.g. SNR lower bound) on Te requirements is needed. Proposal 3 Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements. Proposal 4 RAN4 to further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 RRM requirements impacts for OD-SIB1

Issue 2-1-1: RRM requirements impacts for OD-SIB1

RAN4#112 R4-2413885 Issue 2-1-1: RRM impact of OD-SIB1 - Agreement - Postpone RAN4 work until any RRM impact is identified.

- Proposals

  - Option 1: RAN4 to further discuss the potential RRM requirement impacts on paging interruption for R19 OD-SIB1 related procedure. (Vivo)

    - Option 1a: Clarify in TS 38.133 4.2.2.6 that, time for UL WUS msg1 transmission and time for msg2 reception are components of paging interruption for OD-SIB1 cell re-selection requirements. (Vivo)

    - Option 1b: Due to UL-WUS transmission for OD-SIB1, RAN4 further discuss the impact on paging interruption requirement in cell re-selection. One possible approach is to extend the interruption time to include the UL WUS transmission. (MTK)

  - Option 2: For R19 OD-SIB1 related procedure, clarify DL timing reference of UL-WUS in TS 38.133. RAN4 may further discuss whether additional relaxation or additional applicability restrictions (e.g. SNR lower bound) on Te requirements is needed. (Vivo)

    - Option 2a: For the UL timing requirement of UL-WUS, the DL timing reference of UL-WUS shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc, that: (CMCC)

      - reference cell is the NES cell

      - NTA for UL-WUS is defined as 0

      - is the n-TimingAdvanceOffset from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

  - Option 3: RAN4 to further discuss the RRM requirements on minimum WUS msg1 attempts during each paging cycle, as part of paging interruption requirements. (Vivo)

- Recommended WF:

  - Since this is no dedicated agenda for OD-SIB1 in this meeting, moderator suggests to treat this issue online for guidance.