3GPP TSG-RAN WG4 Meeting #114							                                      	R4-2500541

Athens, GR, 17th-21st February 2025

Agenda item:			7.27.3

Source:	China Telecom

Title:	Topic summary for [114][227] NR_Mob_Ph4_Part2

Document for:	Information

Introduction

This summary includes the proposals from companies on RRM requirements for conditional LTM. The relevant objectives in the latest WID and RAN2’s conclusions are duplicated here for information:

Objectives on conditional Intra-CU LTM in WID - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

RAN2’s conclusions in last meeting 1. The triggering condition of conditional LTM can be based on L3 measurement. 2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. 3. The L1 execution condition of a candidate cell is associated to only one triggering event. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 5. To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. 7. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. 8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. 9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. 10. Candidate cell TA is maintained by a new timer. 11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-1: The applicable scenarios for conditional LTM

Issue 1-1-2: Measurement requirements on conditional LTM

Issue 1-2-1: UE based TA measurement

Issue 1-4-2: The starting point of Subsequent conditional LTM delay

Issue 1-3-2: The starting point of conditional LTM delay

Issue 1-3-1: General aspects of Conditional LTM delay

Issue 1-3-3: The components of conditional LTM delay

Issue 1-3-8: Tinterrupt

Issue 1-3-7: TCLTM_execution / TLTM_execution

Moderator’s recommendation is also provided under issue.

Topic #1: RRM discussion

Companies’ contributions summary

| TDoc | Source | Proposals |
| --- | --- | --- |
| R4-2501423 | China Telecom | Proposal 1: Support L3 measurement based conditional LTM for FR1. Proposal 2: Define RRM requirements for UE based TA measurement. Proposal 3: The starting point of initial conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. Proposal 4: The starting point of subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. Proposal 5: DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. Where: - TRRC is the RRC procedure delay. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. - Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts. - TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. - TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch. Proposal 6：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions. |
| R4-2500630 | Xiaomi | Proposal 1: RAN4 to define requirements for supporting L3 measurement based conditional LTM for FR1. Proposal 2: CHO requirements framework can be used as baseline for Conditional LTM. Proposal 3: The starting point of conditional LTM delay is from the end of the last TTI containing the RRC command implying conditional handover. Proposal 4: RAN4 to define the conditional LTM delay as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution, where - TRRC is the RRC procedure delay, - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch, - Tmeasure can be the same as one measurement period of L1-RSRP measurements for neighbor cell, or legacy L3 measurements, - Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell. Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU, where the TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU can refer to clause 6.3.1.3, - TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, including both RACH based and RACH less cases, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. |
| R4-2501344 | vivo | Proposal 1: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells Proposal 2: RAN4 to define requirements for supporting L3 measurement based conditional LTM for FR1 Proposal 3: RAN4 to use CHO delay requirements framework as a baseline when defining conditional LTM delay requirements, the timeline of CLTM cell delay requirement is defined as follows. The definition and corresponding value of each delay component can be further determined. DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-interrupt + TCLTM_execution Proposal 4: The starting point of conditional LTM delay is the time the UE receives an RRC message implying conditional LTM Proposal 5: For the part of TEvent_DU and part of CLTM (TCLTM-interrupt), RAN4 to wait for more details on procedure designs from RAN2 group Proposal 6: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU. Proposal 7: RAN4 to define RRM requirements for UE-based TA measurement in CLTM Proposal 8: For subsequent conditional LTM, the starting point of the delay (for both RACH-based and RACH-less based) is when UE transmits RRCReconfigurationcomplete message for the previous PCell. Apart from this, the delay requirement on conditional LTM can be reused to define the remaining components of subsequent CLTM. |
| R4-2501022 | CATT | Observation 1: RAN2 has agreed to support L3 measurement based conditional LTM. Proposal 1: RAN4 to follow RAN2’s agreement to support L3 measurement based conditional LTM for FR1. Proposal 2: Before RAN4 reaches a conclusion on whether to define the delay requirements for conditional LTM and subsequent conditional LTM together, RAN4 should prioritize discussing the delay requirements for conditional LTM here. Proposal 3: For the starting point of conditional LTM delay, the legacy definition in R18 LTM can be reused. - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. Proposal 4: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. Proposal 5: The legacy requirments for L3 CHO can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. Proposal 6: RAN4 need to wait for RAN1/RAN2 more progress and further study the LTM related features based on the above requirments. Proposal 7: At least TRRC will not be considered in subsequent conditional LTM delay. Proposal 8: The following general description in proposal 1 is ok for us: - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. Observation 2: The detailed measurement period is related to whether L3 based measurement is considered. Proposal 9: If RAN4 also agree to define the related requirement for L3 measurement based conditional LTM, Tmeasure can refer to proposal 1a. Proposal 10: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. Proposal 11: For Tinterrupt in Rel-19 conditional LTM Cell Switch delay, excluding TLTM-RRC-processing, other components defined in the interruption time for Rel-18 LTM Cell Switch delay can be a baseline, including TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU. Observation 3: Both UE based TA measurement mechanism and PDCCH ordered early TA acquisition are supported for conditional intra-CU LTM in RAN2. Observation 4: Compared with R18 LTM, the UE based TA measurement mechanism in CLTM is more suitable for UE, because UE can start measuring at any time and have more autonomy. Proposal 12: RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM. |
| R4-2501085 | CMCC | Proposal 1: it is proposed to define measurement requirements for L1 measurement on deactivated SCC. Proposal 2: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements for UE based TA adjustment. Proposal 3: for conditional LTM, the starting point is the time when UE receives RRC command triggering conditional LTM. Proposal 4: for conditional LTM, cell switch delay is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TRRC_delay is the RRC procedure delay. - TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurement time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. Proposal 5: for subsequent conditional LTM, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch. Proposal 6: for subsequent conditional LTM, cell switch delay is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurement time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. TLTM-interrupt is the interruption time as specified in Rel-18. |
| R4-2502223 | Qualcomm Incorporated | Proposal 1: RAN4 to define RRM requirements for UE based TA measurement as follows: - Tx timing for the target cell = current Tx timing + 2 x (downlink frame boundary of reference cell – downlink frame boundary of the target cell), where the definition of ‘timing reference cell’ is the reference cell defined in the clause of ‘7.1 UE transmit timing’ in TS38.133. - FFS on the side conditions in terms of SNR, timeline for the downlink frame boundary estimation, etc. |
| R4-2500459 | Apple | Proposal 1: in the agreed applicable scenario of “PCell switch to an LTM candidate cell that is a serving SCell in MCG”, clarify that if CLTM is triggered by L1 measurement (e.g. event LTM3) the scope is limited to active SCell. Proposal 2: to align with RAN2, RAN4 shall also support L3 measurement based conditional LTM. Observation 1: RAN2 already agreed new design on how to handle TA for C-LTM. Therefore, C-LTM can work well without UE based TA measurement. Observation 2: UE based TA measurement has been widely discussed in R18. Eventually RAN4 realized that it is very challenging to define corresponding RAN4 requirements. Observation 3: UE based TA measurement is a R18 optional feature. It is not clearly in scope of RAN4 R19 requirements. Proposal 3: Do not define RRM requirements for UE based TA measurement. Proposal 4: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements. Proposal 4: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements. Proposal 5: The starting point is the end of the last TTI containing the RRC command for conditional LTM. Proposal 6: The ending point could be same as that defined in existing LTM cell switch delay requirements, i.e. time the UE transmits the first UL message on the target cell, which already cover both RACH based and RACH-less. Proposal 7: conditional LTM cell switch delay is defined as: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch. Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes an LTM cell switch to a target cell and interruption time starts: - if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18. - if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19. - if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement. TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms]. TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch. Proposal 8: Assuming subsequent CLTM is to be supported by RAN2, RAN4 requirements for subsequent CLTM can be defined using CLTM requirements as baseline with the following updates: - The starting point of subsequent CLTM is the time UE transmits RRCReconfigurationcomplete message for the previous CLTM. - Note: RAN2 agreed that the LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM. - TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM. Proposal 9: the starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. Detailed requirements can be derived based on CLTM with the exception that starting point needs to be updated. |
| R4-2500887 | MediaTek Inc. | Proposal 1: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM. Observation 1: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB. Proposal 2: Not to define the requirements for UE based TA measurement in RAN4 in R19. |
| R4-2501232 | Huawei, HiSilicon | Proposal 1: In FR1 it is feasible to support L3 measurement based C-LTM. Proposal 2: Conditional LTM delay can take legacy CHO delay as a reference and update Tinterrupt according to TLTM-interrupt defined in R18 LTM cell switch delay. Proposal 3: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM. Proposal 4: TEvent_DU can be reused in conditional LTM delay, as it can cover both cases: 1) UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation 2) neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met. Proposal 5: In C-LTM delay, Tmeas is the L1 measurement period - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. - FFS on CSI-RS based L3-RSRP measurement. Proposal 6: TCHO_execution can be reused in conditional LTM. Proposal 7: The term Tinterrupt may induce ambiguity when define conditional LTM delay, as interruptions can also happen in early PDCCH RACH procedure. Proposal 8: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM. Proposal 9: C-LTM delay can be depicted as DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU Proposal 10: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell. Proposal 11: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM. |
| R4-2500061 | Nokia | Proposal 1: Define CLTM requirements for the case when condition to trigger cell switch is L3 based. Observation 1: Defining new measurement requirements to support any missing scenarios within the CLTM objective is not mentioned in the WID, and hence should be avoided. Proposal 2: RAN4 not to define CLTM cell switch delay requirements for a deactivated SCell, when the condition to trigger cell switch is L1-based. Proposal 3: CLTM requirements to cover the case of target cell being a deactivated SCell, when the condition to trigger cell switch is based on L3 SSB measurements. Proposal 4: If RAN4 agrees to define requirements for UE based TA estimation in Rel-19, at least the issues listed below need to be taken into account. I. Any UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions. II. Existing transmit timing accuracy requirements shall apply. III. Which measurements are expected to be used when performing UE-based TA measurements. IV. How long the acquisition of the UE-based TA takes after candidate cell configuration, to the point where UE is expected to have estimated the TA. V. How to ensure the accuracy of the UE-estimated TA. VI. Requirements to cover not only CLTM but ideally also Rel-18 LTM. Proposal 5: No need to change existing PDCCH ordered RACH delay requirements based on current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed. Proposal 6: No need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed. Proposal 7: Wait for more RAN2 agreements regarding the starting point of the CLTM cell switch delay. Proposal 8: Wait for more RAN2 agreements before agreeing on the exact components of the CLTM cell switch delay. Proposal 9: Too early to agree whether TRRC is needed. Proposal 10: Wait for further RAN2 agreements before agreeing on the definition of Tevent_DU. Proposal 11: When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure. Proposal 12: When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO. Proposal 13: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay. Proposal 14: ASN.1 decoding and validity check is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay. Proposal 15: Fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay. Proposal 16: Wait for more RAN2 agreements before defining the requirements for subsequent CLTM. |
| R4-2501116 | Ericsson |  Proposal 1: The deactivated SCC is applicable for the scenario where Pcell switch to LTM candidate cell that is a serving Scell (deactivated SCC) from both procedure and requirement point of view and shall be supported for Rel-19 CLTM.  Proposal 2: The L3 measurement based conditional LTM shall be supported and RAN4 shall specify requirements accordingly both in FR1 and FR2.  Proposal 3: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.  Proposal 4: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .  Proposal 5: The 1st time CLTM delay can be defined as DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + +TEarly_process.  Proposal 6: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. |
| R4-2500704 | ZTE Corporation, Sanechips | Proposal 1: Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. Proposal 2: RAN4 should define RRM requirements for UE based TA measurement. Proposal 3: For conditional LTM delay, CHO requirements framework can be used as baseline. Proposal 4: before defining the starting point of conditional LTM delay, RAN4 should discuss whether to use the same requirements for conditional LTM and subsequent conditional LTM. - If yes, The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. - If no, the starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. Proposal 5: Use DCLTM = TRRC + TEvent_DU +Tmeasure + TCLTM_execution + TLTM-interrupt. as baseline and consider whether/how to capture the time for early candidate TCI State activation/deactivation and early TA acquisition. TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes LTM cell switch to a target cell and interruption time starts. TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch. Proposal 6: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated. Proposal 7: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. |

Open issues summary

General aspects on Conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#113 < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

- Candidate solutions:

- Proposal 1: If CLTM is triggered by L1 measurement (e.g. event LTM3), the scope is limited to active SCell. (Apple, vivo, Nokia)

- Proposal 2: If CLTM is triggered by L3 measurement, define the requirements to cover the case of target cell being a deactivated SCell. (Nokia)

- Proposal 3: The deactivated SCC is applicable and shall be supported for Rel-19 CLTM. (E///)

  - Proposal 2a: Define measurement requirements for L1 measurement on deactivated SCC. (CMCC)

- Recommended WF

  - For this issue, we could divide it into two cases which CLTM is triggered by L1 measurement and CLTM is triggered by L3 measurement.

For the case which CLTM is triggered by L1 measurement, some companies think deactivated SCC should be excluded, and some companies think not. SO, Further discussion is needed.

For the case which CLTM is triggered by L3 measurement, we think deactivated SCell could be included.

Issue 1-1-2: Measurement requirements on conditional LTM

RAN4#113 < Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting. RAN2#128 1. The triggering condition of conditional LTM can be based on L3 measurement. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- Candidate solutions:

  - Proposal 1: Define requirements for supporting L3 measurement based conditional LTM for FR1. (CTC, Xiaomi, vivo, CATT, Apple, Huawei, Nokia, ZTE)

  - Proposal 1a: The L3 measurement based conditional LTM shall be supported and RAN4 shall specify requirements accordingly both in FR1 and FR2. (E///)

- Recommended WF

  - Most companies support proposal 1. So, we the requirements for supporting think L3 measurement based conditional LTM for FR1 should be defined.

For FR2, whether defines requirements for supporting L3 measurement based conditional LTM needs further discussion. We think in Rel-18 LTM, the requirements for L3 measurement based LTM in FR2 were not defined, and we assume it is same in R19.

Early synchronization and TCI state activation

Issue 1-2-1: UE based TA measurement

RAN4#113 < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

- Candidate solutions:

- Proposal 1: Define RRM requirements for UE based TA measurement. (CTC, CMCC, QC, ZTE, vivo, CATT)

  - Proposal 1a: (QC)

    - Tx timing for the target cell = current Tx timing + 2 x (downlink frame boundary of reference cell – downlink frame boundary of the target cell), where the definition of ‘timing reference cell’ is the reference cell defined in the clause of ‘7.1 UE transmit timing’ in TS38.133.

    - FFS on the side conditions in terms of SNR, timeline for the downlink frame boundary estimation, etc.

  - Proposal 1b: If RAN4 agrees to define requirements for UE based TA estimation in Rel-19, at least the issues listed below need to be taken into account. (Nokia)

    - I.	Any UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.

    - II.	Existing transmit timing accuracy requirements shall apply.

    - III.Which measurements are expected to be used when performing UE-based TA measurements.

    - IV.How long the acquisition of the UE-based TA takes after candidate cell configuration, to the point where UE is expected to have estimated the TA.

    - V.How to ensure the accuracy of the UE-estimated TA.

    - VI.Requirements to cover not only CLTM but ideally also Rel-18 LTM.

- Proposal 2: Do not define RRM requirements for UE based TA measurement. (Apple, MTK, E///)

- Recommended WF

  - Discussion is needed.

Issue 1-2-2: PDCCH ordered RACH

RAN4#113 < Agreement > - PDCCH ordered RACH is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new PDCCH ordered RACH procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

- Candidate solutions:

- Proposal 1: No need to change existing PDCCH ordered RACH delay requirements based on current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed. (Nokia)

- Recommended WF

  - No need to change existing PDCCH ordered RACH delay requirements based on current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Issue 1-2-3: Early TCI state activation

RAN4#113 < Agreement > - Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

- Candidate solutions:

- Proposal 1: No need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed. (Nokia)

- Recommended WF

- No need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Conditional LTM delay

Issue 1-3-1: General aspects of Conditional LTM delay

- Candidate solutions:

- Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM. (Xiaomi, CTC, vivo, Apple, Huawei, ZTE)

  - Proposal 1a: Update Tinterrupt according to TLTM-interrupt defined in R18 LTM cell switch delay. (Huawei)

- Proposal 2: Before RAN4 reaches a conclusion on whether to define the delay requirements for conditional LTM and subsequent conditional LTM together, RAN4 should prioritize discussing the delay requirements for conditional LTM here. (CATT)

- Proposal 3: RAN4 specifies the 1st time CLTM and subsequent CLTM delay separately. (E///)

- Proposal 4: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM. (MTK)

- Recommended WF

  - CHO requirements framework can be used as baseline for Conditional LTM.

  - RAN4 specifies the initial CLTM and subsequent CLTM delay separately.

  - Proposal 4 needs further discussion.

Issue 1-3-2: The starting point of conditional LTM delay

- Candidate solutions:

- Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. (CTC, Xiaomi, CATT, Apple, ZTE)

- Proposal 2: The starting point of conditional LTM delay is the time the UE receives an RRC message implying conditional LTM. (vivo, CMCC, Huawei)

- Proposal 3: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. (ZTE)

- Proposal 4: Wait for RAN1/RAN2 more progress. (Nokia)

- Recommended WF

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Issue 1-3-3: The components of conditional LTM delay

- Candidate solutions:

- Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. (CTC)

    - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. (CTC)

- Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + … +TEarly_process. (E///)

- Proposal 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TCLTM_execution. (CTC, Xiaomi, vivo, CATT, CMCC, Apple, ZTE)

    - Proposal 3a: Whether/How to capture the time for early candidate TCI State activation/deactivation and early TA acquisition. (ZTE)

- Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (Huawei)

    - Proposal 4a: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM. (Huawei)

- Proposal 5: Wait for RAN1/RAN2 more progress. (CATT, Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-3-4: TRRC

- Candidate solutions:

- Proposal 1: TRRC is the RRC procedure delay. (CTC, Xiaomi, CMCC, Apple, ZTE)

- Proposal 2: Too early to agree whether TRRC is needed. (Nokia)

- Recommended WF

  - TRRC is the RRC procedure delay.

Issue 1-3-5: TEvent_DU

- Candidate solutions:

- Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. (CTC, Xiaomi, CMCC, Apple, ZTE, Huawei)

    - Proposal 1a: It can cover both cases: (Huawei)

- UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

- Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

- Proposal 2: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU. (vivo)

- Proposal 3: Wait for more RAN2 agreements. (vivo, Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-3-6: Tmeasure

- Candidate solutions:

- Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. (CTC, CMCC, Apple, ZTE)

    - Proposal 1a:

  - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. (Apple, Huawei, Xiaomi, CATT)

  - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. (Apple, Huawei, Xiaomi, [CATT])

  - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. (Apple, Huawei)

  - FFS on CSI-RS based L3-RSRP measurement. (Huawei)

    - Proposal 1b: (Nokia)

      - When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure.

      - When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO.

- Recommended WF

  - Discussion is needed.

Issue 1-3-7: TCLTM_execution / TLTM_execution

- Candidate solutions:

- Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. (CTC, Xiaomi, CATT, CMCC, Apple, Huawei, ZTE)

    - Proposal 1a: TLTM_execution can be up to 10ms. (CMCC, Apple)

- Proposal 2: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay. (Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-3-8: Tinterrupt

- Candidate solutions:

- Proposal 1: Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL transmission on the target cell. (CTC, Xiaomi, CMCC, Apple, ZTE)

- Proposal 2: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure. (Huawei)

- Proposal 3: Wait for more RAN2 agreements. (vivo)

- Recommended WF

  - Discussion is needed.

Issue 1-3-9: The components of Tinterrupt

- Candidate solutions:

  - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (CTC, Xiaomi, CMCC, Apple, ZTE)

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

  - Proposal 2: Excluding TLTM-RRC-processing, other components defined in the interruption time for Rel-18 LTM Cell Switch delay can be a baseline, including TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU. (CATT)

  - Proposal 3: ASN.1 decoding and validity check and fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay. (Nokia)

- Recommended WF

  - Discussion is needed.

Subsequent Conditional LTM

Issue 1-4-1: General aspects of Subsequent Conditional LTM delay

RAN4#112bis < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

- Candidate solutions:

- Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. (CTC, vivo, Apple, Huawei, ZTE)

- Proposal 2: Wait for more RAN2 agreements before defining the requirements for subsequent CLTM. (Nokia)

- Recommended WF

  - Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Issue 1-4-2: The starting point of Subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. (CTC, vivo, CATT, CMCC, Apple, Huawei, ZTE)

- Recommended WF

  - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Issue 1-4-3: The components of subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. (CMCC)

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (Apple)

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. (E///)

- Recommended WF

  - Discussion is needed.