# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500553

# Athens, Greece, 17th-21st Feb, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on receiver characteristics for LP-WUR

# Agenda Item:   7.26.2.3

# Document for:  Approval

- Introduction

In this contribution, we provided our views on receiver characteristics for LP-WUR.

- Discussion

## 2.1 Maximum input level

In last meeting, RAN4 discussed whether maximum input level for LP-WUR should be scaled-down and most companies thought it is feasible to reuse legacy value.

Issue 3-2-2: Maximum input level requirements Agreements: - For maximum input power level, further discuss - Option 1: use legacy -25dBm - Option 2: scaled to -26.5dBm. - Option 3: same as MR, FFS test signal

In legacy spec, -25dBm is defined as the maximum mean power received at UE antenna port, which applies for whole allocated RBs. Considering that RAN4 is to define minimum requirements, usually with the worst case assumption. For 5MHz channel bandwidth with 30kHz SCS, LP-WUS occupies all the allocated 11RBs, fully allocated. Therefore, we agree to use legacy -25dBm as maximum input level.

Proposal 1: We agree to use legacy -25dBm as maximum input level for LP-WUR.

## 2.2 IBB and OBB

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies specified for IBB and OBB. LP-WUR and MR may share the same antenna and baseband processor, which means that LP-WUR and MR will suffer the same level of interference. To make sure that MR can be waken up timely as long as MR can work, same interference level of IBB and OBB as MR should be assumed for LP-WUR.

Proposal 2: Same interference level of IBB and OBB as MR should be assumed for LP-WUR.

Similar to ACS test, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier.

Proposal 3: Consider the test parameters for IBB of LP-WUR in Table 1, Table 2 and the test parameters for OBB of LP-WUR in Table 3, Table 4. For these test parameters, the missed detection rate shall be ≤1%.

Table 1  In-band blocking parameters for LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| LP-WUS Power | dBm | LR REFSENS + 6 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 2  In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | [-56] | [-44] |
| Finterferer (offset to LP-WUS)1 | MHz | -minimum guard band2-FIoffset, case 1 and minimum guard band2+FIoffset, case 1 | ≤ -minimum guard band2-FIoffset, case 2 and ≥ minimum guard band2+FIoffset, case 2 |
| NOTE 1: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). NOTE 2: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |  |

Table 3  Out-of-band blocking parameters for LP-WUR

| WUS transmission bandwidth configuration (RB) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11RB | LR REFSENS + 6.0 dB |

Table 4  Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | [-44] | [-30] | [-15] |
| Finterferer (CW) | MHz | -60 < f - FDL_low < -15 or 15 < f - FDL_high < 60 | -85 < f - FDL_low ≤ -60 or 60 ≤ f -FDL_high < 85 | 1 ≤ f ≤ FDL_low -85 or FDL_high + 85 ≤ f ≤ 12750 |

## 2.3 Intermodulation Requirements

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively in TS38.101-1[2]. The missed detection rate shall be ≤1% under these assumptions. Similar to the discussion in Section 2.2, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier.

Proposal 4: Consider the test parameters for intermodulation of LP-WUR in Table 5. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Wide band intermodulation parameters for LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | LR REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | [-46] |
| PInterferer 2 (Modulated) | dBm | [-46] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset to LP-WUS)2 | MHz | -minimum guard band3-7.5 / minimum guard band3+7.5 |
| FInterferer 2 (Offset to LP-WUS)2 | MHz | -minimum guard band3-15 / minimum guard band3+15 |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

## 2.4 Spurious Response

Spurious response is also used to evaluate the receiver capability when there is presence of an unwanted CW interfering signal on the frequency where out-of-band blocking limit is not met. Spurious response requirement is needed for LP-WUR.

Proposal 5: Consider the test parameters for spurious response of LP-WUR in Table 6 and Table 7. For these test parameters, the missed detection rate shall be ≤1%.

Table 6  Spurious response parameters for LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
|  |  | 11RB |
| Power in LP-WUS transmission bandwidth configuration | dBm | LR REFSENS + 6 dB |

Table 7  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [-44] |
| FInterferer | MHz | Spurious response frequencies |

- Conclusion

Proposal 1: We agree to use legacy -25dBm as maximum input level for LP-WUR.

Proposal 2: Same interference level of IBB and OBB as MR can be assumed for LP-WUR.

Proposal 4: Consider the test parameters for IBB of LP-WUR in Table 1, Table 2 and the test parameters for OBB of LP-WUR in Table 3, Table 4. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 5: Consider the test parameters for intermodulation of LP-WUR in Table 5. For these test parameters, the missed detection rate shall be ≤1%.

Proposal 6: Consider the test parameters for spurious response of LP-WUR in Table 6 and Table 7. For these test parameters, the missed detection rate shall be ≤1%.

- Reference

- R4-2420394 WF on UE RF requirements for LP-WUS, vivo, RAN4#113, Orlando, USA, Dec 2024.

- TS38.101-1, v18.7.0