# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500555

# Athens, Greece, 17th-21st Feb, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2)

# Agenda Item:   7.26.1

# Document for:  Approval

- Introduction

According to the discussion after RAN4#113 meeting, we provided a text proposal for TR 38.774 on Clause 7.2.2 for the requirement of LP-WUS power boosting [1][2].

- Reference

- R4-2419802 WF for [113][308] NR_LPWUS, RAN4#113, Orlando, USA, Nov. 2024.

- R4-2416589 Way Forward for [112bis][308] NR_LPWUS, RAN4#112bis, Hefei, China, Oct. 2024.

- Text Proposal

<Start of Text Proposal>

### 7.2.2	LP-WUS power boosting

In RAN4 discussion, there are two approaches to define the LP-WUS power boosting requirements, i.e., traditional dynamic range and EPRE ratio. The definitions for these two approaches are:

- For traditional dynamic range, by using similar method of NB-IoT, the LP-WUS power boosting is defined as the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

For EPRE ratio, the LP-WUS power boosting is defined as the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over NR REs (the NR carrier excluding the LP-WUS REs).

The above two approaches are mathematically equivalent, and the specific analysis can be found in R4-2407547 and R4-2419482. There are both pros and cons for each approach. For example, with the concept of traditional dynamic range, manufacturers are allowed to declare different power boosting levels for different NR channel bandwidth, which provides flexibility in power boosting declaration. Further, this concept applies irrespective of RB allocations, i.e., even when there are no REs allocated for NR. With the concept of EPRE ratio, the power boosting level could be same irrespective of the different NR channel bandwidths, i.e., the same power boosting level may be declared for all supported NR channel bandwidths. In this case, the minimum supported channel bandwidth and the single boosting level should be declared at the same time. However, there must be NR REs allocated otherwise the EPRE of NR REs is zero, which means that this definition may not always work.

Besides, power boosting for LP-WUS REs in the case of small NR channel bandwidth such as 5MHz may cause large power de-boosting for the leftover NR REs, which will degrade the NR performance. This can be solved by proper power boosting level declared by the manufacturers.

According to the considerations above, RAN4 decides to adopt traditional dynamic range as the definition of LP-WUS power boosting. Similar with traditional way, different power boosting levels for LP-WUS may be needed for different NR channel bandwidths, and also it is possible to declare single power boosting value for all NR channel bandwidths, which depends on manufacturer declaration.

Moreover, the total transmission power is shared between NR OFDM signal and LP-WUS for the same carrier, and the declared rated carrier output power is not changed with LP-WUS power boosting.

<End of Text Proposal>