3GPP TSG-RAN WG4 Meeting #114	R4-2500607

Athens, Greece, 17th – 21st February 2025

# Source: 	ZTE Corporation, Sanechips

# Document for:	Approval

# Title: 	Discussion on coexistence study based on ITU regulations for NR NTN Ku band

# Agenda item:	7.11.2

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from system parameters perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

# 2 Discussion

In last few meetings, we almost defined all simulation assumptions for Ku band coexistence study. There is a detail about TN UE antenna pattern needs to be further checked.

- Sub-topic 2-9: TN BS/UE antenna patterns for 11 GHz Agree to use TN BS and UE antenna and beam forming pattern modelling in TS 38.921 for 11GHz carrier frequency for Ku band coexistence study - Sub-topic 2-10: TN BS antenna pattern for 14 GHz Agree to use TN BS antenna and beam forming pattern modelling in TS 38.922 for 14GHz carrier frequency for Ku band coexistence study. For TN UE, the isotropic antenna model will be used as mentioned in TS 38.922.

The above are agreements reached in last meeting [2]. For 11GHz, the TN UE antenna is assumed as isotropic radiation pattern antenna and no beamforming since UE in 11GHz may have a conducted interface with “FR1 like” requirements. While for 14GHz, it provides two UE antenna element pattern assumptions: FR1 like and FR2 like, which is as follows. To simplify the coexistence simulation, we propose to assume “FR1 like” antenna pattern with isotropic radiation pattern antenna and no beamforming for 14GHz.

Proposal 1: We propose to assume “FR1 like” antenna pattern with isotropic radiation pattern antenna and no beamforming for TN UE in 11GHz and 14GHz.

- 6.1.2.3.3 UE antenna element pattern - 6.1.2.3.3.1 FR1 like In this approach, a UE will most likely have a conducted interface with an assumed isotropic radiation pattern antenna with [0 or 5] dB receive diversity gain and no beamforming. - 6.1.2.3.3.2 FR2 like Table 6.1.2.3.3.2-1: UE antenna element pattern Parameter; Values. Antenna element vertical radiation pattern (dB); AE,V(θ″)=−min12θ″−90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB. Antenna element horizontal radiation pattern (dB); AE,H(ϕ″)=−min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB. Combining method for 3D antenna element pattern (dB); A″(θ″,ϕ″)=−min−AE,Vθ″+AE,Hϕ″,Am. Maximum directional gain of an antenna element GE,max; 5 dBi. (Mg, Ng, M, N, P); (1, 1, 2, 2, 2). (dv, dh); (0.5λ, 0.5λ). UE orientation; Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees. NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees..

In addition, based on the agreed simulation assumptions, we share the preliminary simulation result for scenario 1, in which we assume “FR1 like” antenna pattern for TN UE.

Figure 2.1: Preliminary simulation result for scenario 1

# 3 Conclusion

In this contribution, we shared some views on coexistence study for NTN Ku band and the proposal is made as follows:

Proposal 1: We propose to assume “FR1 like” antenna pattern with isotropic radiation pattern antenna and no beamforming for TN UE in 11GHz and 14GHz.

# 4 Reference

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)