3GPP TSG-RAN WG4 Meeting#114		R4-2500614

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.24.2.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on RF requirements for ambient IoT device 1

Document for:	Discussion

Introduction

In RAN4#113 meeting, in SI stage, conclusion in TR 38.769 about RF requirements is agreed. The WID [1] was approved. This contribution discussed RF requirements for device 1 in the WI stage as follows.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

Discussion

TX requirements

According to the conclusion in TR 38.769 in RAN4#113 meeting, TX requirements are potential need to be discussed or no need to define and TBD. These requirements need to be discussed in WI stage.

- Transmitter power

Backscattered modulated power

In TR, for device1, D2R backscattering is transmitted on a carrier wave provided externally, backscattering power and power backscattering are potentially need to be defined to ensure the coverage range between reader and device. The minimum backscattered modulated power can be specified for device1. The backscatter power is the amount of power reflected back from the device when a successful command sequence is completed at the device at a specific sensitivity level. The minimum backscatter modulated power is to ensure the minimum coverage range between reader and device. In the SI stage, the coverage target is 10m. According to the RAN1’s simulation conclusion about coverage, the results of D2R coverage basically are more than 10m. In this case, we can assume the minimum coverage between device and reader is 10m. From device to reader, including three parts of loss or gain, they are respectively freedom pathloss, shadow fading and antenna gain. The formula of freedom pathloss is L=33.63+21.9log10d3D+20log10fc, shadow fading is 1.346σSF=1.346*4=5.384, the antenna gain of device1 is 0dBi, the antenna gain of A-IoT BS is 6dBi, REFSENS of medium range BS (A-IoT BS) is -101.8dBm, the backscatter loss of OOK is 8~10dB for practically. Through the above calculation, we can get that the minimum backscatter is -37.796dBm. Taking into account a 3~4dB margin, we think the minimum backscatter modulated power is -34dBm could be a starting point.

Proposal 1: The minimum backscatter modulated power needs to be defined. -34dBm for the minimum backscatter modulated power could be a starting point.

Backscatter loss

The backscatter loss is the incident signal minus the backscattered signal. It depends on a mismatch between antenna impedance and load impedance. In theory, the backscatter loss for OOK is 6dB. However, considering the actual implementation, we need to leave 3~4 dB margin, hence, the backscatter loss is 10dB could be a starting point.

Proposal 2: The backscatter loss needs to be defined. The backscatter loss is 10dB could be a starting point.

- Output power dynamics

Output power dynamics include minimum output power, transmit off power, transmitter on/off time mask and power control. We agree with the conclusion for TR about this part. Output power dynamics requirements are unnecessary.

Proposal 3: Output power dynamics are unnecessary to define.

- Transmitted signal quality

Transmitted signal quality includes frequency error, carrier leakage, in-band emission and EVM. In the SI stage, frequency error, carrier leakage and in-band emission don’t need to be defined. However, some companies think that modulation quality requirements is potentially needed. In our understanding, device1 is a low-cost and simple device. Hence, as long as the power that device1 can transmitter and meet the threshold of power. As for the modulation quality, the that reader can guarantee it.

Proposal 4: There is no need to define the modulation quality requirements.

- Output RF spectrum emissions

Output RF spectrum emissions include occupied bandwidth, out of band emission (SEM, ACLR), spurious emission and transmit intermodulation. According to the conclusion in TR, occupied bandwidth requirement is potentially needed. The specific value is dependent on the physical design of D2R, which has not yet been finalized at this stage. In our understanding, we need to consider the guard band which is caused by SFO.

Proposal 5: Occupied bandwidth needs to be defined. The specific value is dependent on the channel bandwidth of D2R.

RX requirements

- Reference sensitivity

Reference sensitivity is strongly related to the coverage of R2D. According to the co-existence assumptions about device1 of reference sensitivity which is -36dBm. However, reference sensitivity may not be so good for some companies to give feedback, such as -30dBm. Of course, it depends on the actual situation. In addition, if the spectrum for R2D is DL spectrum, the transmitter of A-IoT BS could be 33dBm, but if the spectrum for R2D is UL spectrum, the transmitter of A-IoT BS could be 23dBm. In our understanding, whatever the spectrum is used to for R2D, they all need to meet the minimum coverage of R2D (i.e., 10m). Similarly to D2R, we can assume the minimum coverage between device and reader is 10m. From reader to device, including three parts of loss or gain, they are respectively freedom pathloss, shadow fading and antenna gain. The formula of freedom pathloss is L=33.63+21.9log10d3D+20log10fc, shadow fading is 1.346σSF=1.346*4=5.384, the antenna gain of device1 is 0dBi, the antenna gain of A-IoT BS is 6dBi.Through the above calculation, we can get that the reference sensitivity is -31dBm.Similarly, we need to take into account a 3~4dB margin. Hence, -31dBm plus 3dB is -28dBm. Based on all the above analysis, the reference sensitivity could be -28dBm as a starting point.

Proposal 6: The reference sensitivity needs to be defined. -28dBm for the reference sensitivity could be a starting point.

- Maximum input power

According to the conclusion in TR about maximum input power, the maximum input power needs to be discussed in WI stage. Maximum input power is defined as the maximum mean power received at the UE antenna port. The purpose of the definition is to prevent the whole link from being saturated, thus causing the deterioration of SNR and performance. According to the investigation for envelop detector, the dynamic range of envelop detector is about 45dB, and the reference sensitivity of device 1 and device2a is -36dBm and -46dBm. The transmission power of A-IoT BS is 33dBm, assumed for co-existence study. Which means that it may exceed the dynamic range of envelop detector. Hence, it is needed to define the maximum input power for device 1.

Proposal 7: Maximum input power is needed to define. The specific value needs to be further discussed.

- Interference rejection

The interference could be wide-band NR signal (e.g. in-band deployment) or CW signal (e.g. from unsynchronized reader/CW node) or D2R signal from other devices. According to the conclusion in TR about interference rejection, whether interference rejection should be defined needs to be discussed in WI stage. In our understanding, device 1 is a low-cost and simple device. We just need to make sure that this device can work normally in a limited range. Hence, there is no need to define interference rejection.

Proposal 8: Interference rejection is no needed to define.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: The minimum backscatter modulated power needs to be defined. -34dBm for the minimum backscatter modulated power could be a starting point.

Proposal 2: The backscatter loss needs to be defined. The backscatter loss is 10dB could be a starting point.

Proposal 3: Output power dynamics are unnecessary to define.

Proposal 4: There is no need to define the modulation quality requirements.

Proposal 5: Occupied bandwidth needs to be define. The specific value is dependent on the channel bandwidth of D2R.

Proposal 6: The reference sensitivity needs to be defined. -28dBm for the reference sensitivity could be a starting point.

Proposal 7: Maximum input power is needed to define. The specific value needs to be further discuss.

Proposal 8: Interference rejection is no needed to define.

Reference

- RP-243326, New Work Item: Solutions for Ambient IoT(Internet of Things) in NR, Huawei, TSG RAN-P meeting #106