3GPP TSG-RAN WG4 Meeting # 114	R4-2500622

# Athens, GR, 17th – 21th Feb., 2025

Source: 	Xiaomi

Title: 	Discussion on UE-initiated/event-driven beam management on Rel-19 MIMO phase 5

Agenda Item:	7.22.3.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the RRM requirement of UE-initiated/event-driven beam management for Rel-19 MIMO phase 5.

- Discussion

2.1 Measurement result in L1 report

Issue 1-5: Whether to update L1 report which is measured in L1 report? <Way forward > - Option 1: (Xiaomi) - It needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. - Option 2: (vivo) - RAN4 to specify the requirements to consider the condition which will trigger the event is remained fixed during the time UE performs L1 measurements. - Whether UE shall update the L1 report content based on latest L1 measurement results, or shall even cancel the report in some cases, is up to RAN1/2 discussion.

For Issue 1-5, the major impact is which L1 measurement result is put into the L1 report. In legacy L3 event triggered reporting, this behaviour is clarified in 38.331. Since this is L1 report, the procedure is better specified in physical layer. We prefer to clarify this issue in RAN4.

In legacy L3 event triggered reporting, according to 38.331, if L3 measurement result satisfies the event triggering condition, UE will initiate the message reporting procedure and submit the MeasurementReport message to lower layers for transmission. It means that the L3 measurement result which triggers reporting will be included in the L3 report.

### 5.5.4 Measurement report triggering #### 5.5.4.1 General 2> if the reportType is set to eventTriggered, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event): 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> if useT312 is set to true in reportConfig for this event: 4> if T310 for the corresponding SpCell is running; and 4> if T312 is not running for corresponding SpCell: 5> start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR; 3> initiate the measurement reporting procedure, as specified in 5.5.5; ### 5.5.5 Measurement reporting #### 5.5.5.1 General ……. 1> else if the UE is in NR-DC: 2> if the measurement configuration that triggered this measurement report is associated with the SCG: 3> if SRB3 is configured and the SCG is not deactivated: 4> submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends; 3> else: 4> submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3; 2> else: 3> submit the MeasurementReport message via SRB1 to lower layers for transmission, upon which the procedure ends; 1> else: 2> submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

Observation 1: For legacy L3 event triggered reporting, it is specified in 38.331 that the measurement result which triggers L3 reporting will be included in the L3 report.

For L1 event triggered reporting, different from L3, L1 measurement is much faster, it’s possible that UE can obtain several new L1 results between the time UE trigger L1 reporting and the time when UL resource arrives.

Fig.1 New L1 results

Similar as L3 event triggered reporting, it needs to specify which measurement result will be included in L1 reporting. Following the same logic of L3, L1 result which triggers L1 reporting will be included in L1 reporting.

Proposal 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

2.2 Measurement time

In last meeting, RAN1 send a LS to RAN4[1]:

Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

From the LS, it shows that for single CC case, at least RAN1 will support Alt-1, where periodicity of the current beam RS should be the same as that of the new beam RS(s). while Alt-1 may have some issue on CA case as different CC may have different SCS. RAN1 will further discuss for CA scenario and it’s also possible that the RS periodicity for current beam and new beam is different. we prefer to define a unified requirement for single CC and cross-CC case. Therefore, if multiple beams will be measured with different measurement period, total measurement time will be maximum measurement period between multiple beams.

Proposal 2: If multiple beams will be measured, total measurement time will be maximum measurement period between current beam and new beam.

2.3 Reporting delay

Issue 1-4: Reporting delay requirements <Way forward > - Option 1: (Qualcomm) - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0; - Option 2: (Nokia) - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event. - Option 3: (Xiaomi) - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams. - Option 4: (Samsung) - Need further RAN1 progress for RSs relationship between current beam and new beams. - Option 5: (CMCC) - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on - Option 6: (Apple) - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay - Option 7: ( MediaTek) - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period - Option 8: (Ericsson) - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel - RAN4 to discuss components and criteria for defining TTime to first UL channel.

We suggest to follow the legacy L3 event triggered reporting delay, i.e. The legacy L3 event triggered reporting delay is defined based on measurement delay and didn’t consider the extra delay caused by UL resource waiting time and L3 filtering time.

- The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when UE transmits a first PUCCH triggered by the event over the air interface. The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. Where Tmeasure is the measurement delay.

Proposal 3: For event triggered L1 reporting delay is defined as the time between an event that will trigger a L1 measurement report and the point when UE transmits a first PUCCH triggered by the event over the air interface. The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. Where Tmeasure is the measurement delay.

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward > - Option 1: (Nokia) - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. - Option 2: (Xiaomi, Samsung, Apple, Ericsson, MediaTek) - Reuse current beam sweeping factor for UE/event triggered beam reporting

We suggest to reuse the current assumption of beam sweeping and don’t introduce enhancement. Therefore, we support option 2.

Proposal 4: Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting.

2.4 L1-RSRP Accuracy requirement

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek) - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements. - Option 2: (Samsung) - Focus on core requirements firstly, FFS on accuracy requirements. - Option 3: (CMCC) - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account. - Option 4: (ZTE) - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.

For legacy L3 event triggered reporting, there is no new requirement for measurement accuracy. Similar, for L1 event triggered reporting, there is also no new requirement for measurement accuracy.

Proposal 5: No new accuracy requirements for L1 measurements are required.

2.5 Reduction of beam application delay

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

In last meeting, RAN1 agrees to further study the reduction of beam application delay, the majority options will skip extra SSB reception for synchronization. This part may have impact on the MAC CE based TCI activation delay if target TCI state is not in the active TCI state list. RAN4 may need to consider the impact according to RAN1 progress.

Proposal 6: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: For legacy L3 event triggered reporting, it is specified in 38.331 that the measurement result which triggers L3 reporting will be included in the L3 report.

Proposal 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

Proposal 2: If multiple beams will be measured, total measurement time will be maximum measurement period between current beam and new beam.

Proposal 3: For event triggered L1 reporting delay is defined as the time between an event that will trigger a L1 measurement report and the point when UE transmits a first PUCCH triggered by the event over the air interface. The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. Where Tmeasure is the measurement delay.

Proposal 4: Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting.

Proposal 5: No new accuracy requirements for L1 measurements are required.

Proposal 6: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Reference

[1] R1-2410913, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, ZTE