3GPP TSG-RAN WG4 Meeting #114			                                          R4-2500627

Athens, Greece, 17th – 21st Feb., 2025

Agenda Item:	7.14.2.2

Source:	Xiaomi

Title:			Discussion on OTA test for FR1 NTN devices

Document for:	Discussion

- Introduction

The test methodology and radiated performance metric for FR1 NTN devices has been extensively discussed in previous meetings. The latest progress was captured in the WF [1] as follow. This contribution continues to provide our views on these remaining issues.

### Sub-topic 3-1 UE type and usage scenarios for NTN (NR-NTN and IoT-NTN) Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric Agreements: - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric Issue 3-1-4: non-Handheld IoT-NTN performance metric Agreements: - FFS on partial sphere metrics, such as the radiated power and receive sensitivity within a certain angular range, should be performance metric for non-handheld IoT NTN devices. - This can be considered as a package with issue 3-1-3. Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN) Agreements: - For UEs support both GSO and NGSO, single set of performance metric is required for GSO and NGSO handheld UEs. - For UE support only GSO or NGSO, FFS performance metric should be the same.

- Discussion

According to the agreed WF, it was agreed that full sphere TRP and TRS are considered as the performance metric for IoT-NTN and NR-NTN handheld UEs supporting scenarios 1. The agreement mainly targets for handheld devices to ensure that spherical coverage is independent on device orientation. However, for those UEs supporting scenario 2 only, FFS on the performance metric. From our perspective, in scenario 2, the user is more likely to hold the UE in a certain position and in order to guarantee the UL coverage and antenna efficiency, it is reasonable to consider the directional antenna in real implementation to concentrate energy in a certain direction. RAN4 should consider the partial sphere metrics within a certain angular range which is declared by OEMs as the radiated performance metric for IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only. And for non-handheld IoT-NTN devices, the similar reason should be considered since they typically have a certain physical installation form.

Proposal 1: RAN4 consider the partial sphere metrics within a certain angular range which is declared by OEMs as the radiated performance metric for IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only.

Proposal 2: RAN4 consider the partial sphere metrics within a certain angular range which is declared by OEMs as the radiated performance metric for IoT-NTN non-handheld UEs.

The other issue is whether performance metric should be the same for GEO and NGEO for handheld UEs. RAN4 agreed that single set of performance metric is required for GSO and NGSO handheld UEs supporting both GSO and NGSO. And FFS the performance metric for UE support only GSO or NGSO. From UE implementation point of view, if UE supports a certain satellite band, the unified antenna design will be adopted for either GSO or NGSO, in addition, as the orientation of the handheld UE is arbitrary, so even if the handheld UE only supports the GSO or NGSO scenario, its spherical coverage is still required to be the same.

Proposal 3: For UEs support only GSO or NGSO, single set of performance metric is defined for handheld UEs.

3	Conclusions

In this paper, we provide our views on developing test methodology and radiated performance metric for FR1 NTN devices, Proposal 1: RAN4 consider the partial sphere metrics within a certain angular range which is declared by OEMs as the radiated performance metric for IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only.

Proposal 2: RAN4 consider the partial sphere metrics within a certain angular range which is declared by OEMs as the radiated performance metric for IoT-NTN non-handheld UEs.

Proposal 3: For UEs support only GSO or NGSO, single set of performance metric is defined for handheld UEs.

4 References

[1] R4-2419808, WF for [113][325] TRP_TRS_Ph3, Vivo