3GPP TSG-RAN WG4 Meeting #114				                                R4-2500633

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.10.3.4

Title: 	Discussion on RRM requirements for less than 5MHz NTN

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for enhanced requirements and test methodology for NR and IoT NTN was agreed and one of objectives for less than 5MHz for NTN is self-contained as below.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

And in the last RAN4 meeting, the agreements for less than 5MHz for NTN was achieved in [2, R4-2416851]. Therefore, in this discussion we provided further considerations on the open issues for less than 5MHz NTN.

- Discussion

Up to RAN4#113, the main open issues for these RRM requirements are [2]

- requirements for SSB time index detection

Issue 1-1-2: Measurement(SSB index detection requirements) Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

Usually, RAN4’s requirements is based on the worst case including the lowest SINR side condition. It is true that UE may work under the different SINR. But it doesn’t make sense to define the multiple sets of requirements under the different SINR condition which will significantly increase UE testing efforts.

Observation 1: It’s unnecessary to define the other requirements based on SINR besides the lowest one.

Regarding to the same SSB punctured used and same SINR side conditions, for less than 5MHz NTN, the performance degradation due to the less PSS/SSS/PBCH could be same as these in case TN less than 5MHz. Therefore, we can suggest that:

Proposal 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by [4] and [3] more samples for intra-frequency and inter-frequency cases respectively.

- Conclusion

In this contribution, serval issues related to the RRM requirements under less than 5MHz NTN in Rel19 are discussed. The observations and proposals can be summarized as:

Observation 1: It’s unnecessary to define the other requirements based on SINR besides the lowest one.

Proposal 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by [4] and [3] more samples for intra-frequency and inter-frequency cases respectively.

- References

- RP-240857

- R4-2416851

- TS38.133 v18.7.0

- TS38.211 v18.3.0