# 3GPP TSG-RAN WG4 Meeting #114	R4-2500643

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.11.4

Source: 	Xiaomi

Title: 	VSAT Types for Ku band

Document for:	Discussion

1		Introduction

For Rel-19 NTN Ku Band WI, RAN#106 updated the WID to include two sets of bands with FR1-like numerology and FR2-like numerology. It’s also agreed to add Mobile VSAT with NGSO into the scope.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible, specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

In this contribution, we discussed candidate VSAT types with associated RF requirements.

2		Discussion

Target terminal types for Ku Band(s) operation is same as Ka band as VSAT. Refer to existing TS 38.101-5, 5 types of VSAT are specified in the specifications based on the assumption of supporting satellite types (GSO only, both GSO and NGSO), antenna types in terms of beam steering capacity (mechanically beam steering and electronical beam steering) and terminal mobility (fixed VSAT and mobile VSAT).

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

According to the agreed WID, RAN4 target to leverage existing VSAT requirements and VSAT type definition as much as possible meanwhile RAN-P also agreed to introduce mobile VSAT supporting both NGSO scenario and GSO scenario based on existing Type 5 requirements. In our view, VSAT types were introduced based on regulation difference, and RF requirements dependency.  RAN4 shall strive to harmonize device types to minimize specification work, and market fragmentation.

From UE capability perspective, separate UE capability IEs are introduced for VSAT on supporting satellite types, mobility (fixed or mobile ) and antenna type (electronic or mechanical steering antenna).

From RF requirements perspective, the major difference between mobile VSAT supporting both NGSO and GSO and supporting NGSO only are Minimum Peak EIRP and EIS.  The requirements for device type supporting NGSO only are more relaxed that UE supporting GSO.  Based on the WID, in Rel-19 we only focus on mobile VSAT supporting both NGSO and GSO scenarios based on existing Mobile VSAT Type 5 supporting NGSO.

Proposal 1: For Ku band(s) UE RF requirements, focus on VSAT Type 2, Type 4 and Type 5.

Proposal 2: Extend existing VSAT Type 5 to support mobile VSAT with both GSO and NGSO scenarios.

RAN-P also agreed to introduce two sets of bands with FR1-like or FR2-like numerologies. In existing RAN4 requirements, VSAT RF requirements only specified for FR2-NTN bands, RAN4 shall strive to specify common RF requirements for both FR1-NTN and FR2-NTN Ku band(s) which target for VSAT terminal types with radiated requirements only.

In table below, we provide preliminary analysis whether common RF requirements can be specified for both FR1-NTN and FR2-NTN Ku-bands.

| RF requirements | Analysis on numerologies dependency | Conclusion (FR1 and FR2) |
| --- | --- | --- |
| 9.2 Transmitted power | TRP and EIRP requirements specified based on regulation and co-existence study and VSAT types which agnostic to numerologies | Common requirements VSAT types and regulation dependency |
| 9.3 Output power dynamics | FR1 or FR2 dependency on transmit on/off time mask. In general, 10us transient period applied for FR1 and 5us transient period. | More discussion on 9.3.3 transmits on/off time mask required - FR1-NTN Ku band(s) reusing FR1 requirements - FR2-NTN Ku band(s) reusing FR2 requirements No update on 9.3.1 Minimum output power, 9.3.3 transmit off power and 9.3.4 Power control |
| 9.4 Transmitter signal quality | Core requirements is independent to numerologies For measurement methodology specified in Annex i.e., measurement window and length are CHBW and SCS specific. | Common on core requirements No update on core requirements |
| 9.5 Output RF spectrum emission | Occupied channel bandwidth is bandwidth specific Measurement bandwidth for ACLR is channel bandwidth specific, which also pending co-existence study. OOB boundary is channel bandwidth specific. On-axis spurious requirement pending on regulation and operating range. | Occupied channel bandwidth, ACLR measurement bandwidth, and OOB boundary are channel bandwidth specific. |
| 10.3 OTA reference sensitivity level | Existing requirements based on minimum CHBW 50MHz specified. Need to be scaled with CHBW and SCS. | Update required considering FR1 CHBW and SCS |
| 10.4 Maximum input level | CHBW and FRC needs to be update | CHBW and FRC reference needs to be update |
| 10.5 ACS & 10.6 blocking | Channel bandwidth and interference offset pending on CHBW ACS value pending on co-existence study | CHBW and FRC reference needs to be update |
| 10.7 Spurious emission | Not applicable | Not applicable |
| 10.8 Receiver antenna off-axis performance | Regulation and frequency range dependent | Common requirements |

3		Conclusion

In this contribution, views on UE RF requirements for Ku band were provided.

Proposal 1: For Ku band(s) UE RF requirements, focus on VSAT Type 2, Type 4 and Type 5.

Proposal 2: Extend existing VSAT Type 5 to support both GSO and NGSO scenarios.

Proposal 3: Taking below table as starting point for numerologies dependency for RF requirements:

| RF requirements | Numerologies dependency (FR1 and FR2) |
| --- | --- |
| 9.2 Transmitted power | Independent |
| 9.3 Output power dynamics | More discussion on 9.3.3 transmits on/off time mask required - FR1-NTN Ku band(s) reusing FR1 requirements - FR2-NTN Ku band(s) reusing FR2 requirements No update on 9.3.1 Minimum output power, 9.3.3 transmit off power and 9.3.4 Power control |
| 9.4 Transmitter signal quality | Numerology independent for core requirements |
| 9.5 Output RF spectrum emission | Occupied channel bandwidth, ACLR measurement bandwidth, and OOB boundary are channel bandwidth specific. |
| 10.3 OTA reference sensitivity level | Update required considering FR1 CHBW and SCS |
| 10.4 Maximum input level | CHBW and FRC reference needs to be update |
| 10.5 ACS & 10.6 blocking | CHBW and FRC reference needs to be update |
| 10.7 Spurious emission | Not applicable |
| 10.8 Receiver antenna off-axis performance | Numerology independent |

Reference

[1] RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT