# 3GPP TSG-RAN WG4 Meeting #114	R4-2500650

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.6.2

Source: 	Xiaomi

Title: 						Methods for reducing the number of UE Rx chains

Document for:	Discussion

- Introduction

Rel-19 agreeded a study item to study the feasibility and requirements impact on using shared Rx chain to support mutilple fragemented carriers in single band under intra-band NC CA.

In previous RAN4 meeting, the reference archirecture and target scope has been clarified and agreed as following:

#### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

In last RAN4 meeting, following agreements were reached for topic 2 ”Methods for reducing the number of UE Rx chains”:

# Main session online agreements #### Issue 2-1: Discussion on baseband assumptions Agreement: - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI - 15KHz SCS is used as the starting point - Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR. # Ad hoc meeting agreements #### Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied - Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2 #### Issue 2-6: Discussion on semi-statically switch hardware resources and fallback considerations When one Rx RF chain cannot perform anymore: - UE can fallback to single carrier

In this contribution, views on remaining open issues were provided.

- Discussion

Issue 2-1: Discussion on baseband assumptions

The traditional reference UE architecture for intra-band NC CA is partially shared Rx chain i.e., antenna, LNA with separate PLL/mixer, analog LPF, ADC and separate digital BB processing (FFT, digital filtering).

Both Rx analog components including analog LPF, ADC and digital BB processing (FFT operation and digitail BB filtering) have contribute to handle power imbalance between wanted carriers, the in-band blocker:

  - 25dBc PSD imbalance between wanted carriers allowed for intra-band NC CA (refer to Non-collocated NC CA with separate ADC)

  - ~ 30dBc, ~40dBc, ~50dBc PSD imbalance between wanted signal and interference on 1st , 2nd and 3rd  adjacent channels allowed (refer to selectivity and in-band blocking requirements)

Observation 1: Both analog RF components (LPF, ADC) and digital BB processing will impact UE capacity of in-gap interference rejection and maximum tolerated power imbalance between wanted carriers.

Separate FFT operation as per sub-block basis can be helpful to attenuate interference with in the gap if interference is orthogonal to wanted carrier. If the orthogonality can’t be guaranteed, then no much different between single wide FFT and separate FFT operation across sub-blocks on rejection in-gap interference.  Multiple FFT is still requited to handle non-orthogonality case between wanted carriers with mixed numerologies, unaligned grid on RB allocation and or wider separation exceed UE FFT size.

Proposal 1: The condition of single FFT operation as following:

- Aligned numerologies/SCS between wanted carriers

- The maximum separation of between lower edge of lowest CC and upper edge of highest CC not exceed maximum UE channel bandwidth under corresponding numerology/SCS e.g. 50MHz for 15kHz SCS

- The carrier spacing among any combination of 2 wanted carriers shall be multiple times of SCS e.g., for bands with 100kHz channel raster, carrier spacing shall be multiple times of 300kHz

Another issue needs to clarify what’s the assumption of digital BB filtering i.e. whether single wideband digital BB LBF can be considered or not. In our view, in order to attenuate the in-gap interference after ADC, digital BB filtering shall de designed to accommodate the interference under fully shared Rx chain mode otherwise receiver performance will be degraded.

Proposal 2: Digital BB filtering shall be considered as per sub-block basis to reject/attenuate the in-gap interference.

Issue 2-2: Discussion on 2Rx applicability

On 2Rx applicability, in last meeting two options discussed as captured in moderator summary as following:

- Recommended WF - Discuss if the case of the fragmented carrier operation in the bands n7 and n41 could relieve the UE, only when configured in fragmented carrier operation in these bands, of the requirement to support 4Rx. - Option 1: UE is allowed to only support 2Rx when UE is configured DL NCCA in the mandatory 4Rx band when configured in fragmented carrier operation - Option 2: UE shall still support 4Rx when UE is configured DL NCCA in the mandatory 4Rx band when configured in fragmented carrier operation

Firstly, we need to clarify companies’ understanding on existing 4Rx mandatory applicable rules in the specifications.

Refer to TS 38.101-1 as captured below:

## 7.2 Diversity characteristics The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination. Unless otherwise stated, the following applicability rules apply, - For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for four and eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports. - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, the UE shall be verified with eight Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with eight Rx antenna ports unless the UE does not support eight Rx ports for band(s) in a band combination in which case those band(s) shall be verified with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports. - The above rules apply for all clauses except for clause 7.9. A Redcap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. Clause 7 requirements for four Rx antenna ports do not apply to a RedCap UE. If aUE indicates intraBandNR-CA-non-collocated-r18, both Rx power imbalance requirements as specified in clause 7.10A and Rx requirements in clauses 7.3 – 7.9 shall be verified.

From above general statement in existing TS 38.101-1, our understanding on the bands which 4Rx is mandatory, this 4Rx mandatory applied for both standalone single CC or CA.

Observation 2: 4Rx mandatory is applied for both standalone single CC or CA as captured in TS 38.101-1.

Refer to UE capability in terms of MIMO layers (no dedicated signaling on Rx antenna ports) in TR 38.823/TS 38.306

RAN2 capability signaling on PDSCH MIMO layer IE is specified as per CC per band per band combination basis with most flexibility.

Observation 3: RAN2 PDSCH MIMO layer capability IE is specified as per CC per band per band combination basis.

This SI target to enable single Rx RF chain to support multiple fragmented carriers with benefits of supporting high order MIMO layers or high order CA combinations. Reducing number of MIMO layers per CC can be another way to support more fragmented carriers with limited number of Rx chains.

The story of intra-band NC CA is different which target to enable intra-band NC CA by reduced MIMO layers per CC under the limitation Rx chains.

Theoretically, supporting more fragmented carriers can be achieved by shared RF chain and/or reduced MIMO layers/RX chain per carrier. However, this is out of the scope of this SI for reducing MIMO layers to achieve more fragmented carriers or high modulation orders and actually the existing mechanism/UE capability signaling already allow such operation.

Proposal 3: UE shall maintain 4Rx mandatory applicable rule as existing specification under intra-band NC CA regardless when operating in fragmented operation.

Issue 2-3: Study on power spectral density difference between carriers of co-located adjacent channel operators from UE side

This issue related issue 2-5 also, when decide the triggering condition to enable ”one RF Rx chain” we need to guranteen the Rx performance with acceptable degradtion as  corresponding upper bound of interference level and also needs to consider the possibility of power spectral density in the deployment scenario for evaluation purpose.

From ADC perspective, not only PSD impact the perfomance, and the absolute interference level will impact the ADC dynamic range, quantization resolution given with shared Rx RF chain not possible to attenuate the adjacent channel interference before ADC.

Proposal 4: Consider 6dBc to 25dBc PSD between wanted carriers and in-gap interference for evaluation the performance impact purpose.

Issue 2-4: Signaling related

On signalling aspect, our preference we shall take this feature based on existing band combination structure with addtional necessary indications.

When UE access to the network, network will inquire necessary information from UE and if UE supported fragemented carriers feature and network have such fragemented operation deployment scenario which may further inquire the supporting indications.

Under single RF Rx chain operation condition with limited Rx RF chain; UE may increase MIMO layers on supported carriers or extend supporting high order band combinations with more carriers pending on RF capacity and baseband processing capacity.

Proposal 5 : Supporting alternative 1 ” Legacy band combination reporting scheme + Rx sharing capability indication”.

- The indication can be per band per band combinations.

Proposal 6: UE shall allow to indicate separate MIMO layer capacity under single Rx chain operation condition and fallback normal condition on 4Rx optional bands.

Issue 2-5: Triggering condition to enable ”One RF Rx chain”

In our view, RAN4 shall evaluate the tolerable  in-gap interference on absolute power level and PSD similar as existing selectivity and in-band blocking requirements with acceptable performance degradtion. And then separate requirements can be specified in the specification under certain configured condition to replace existing requirements.  On how to decide to enable ”one RF Rx chain“ or fall-back to normal mode, this can be left to UE implemention as well as UE can guranteen the Rx performance.

Proposal 7: RAN4 shall evaluate the tolerable in-gap interference in terms of absolute power level and PSD

- These values can be considered as reference to introduce new selectivity/in-band blocking requirements during WI phase if any

On evaluation criteria, we sugguest to consider similar thing as SBFD interference evaluation, using receiver desensitization as evaluation metric [1dB] receiver desensitization can be considered as starting point.

Proposal 8: Using receiver desensitization as evaluation criteria for” One RF Rx chain” on/off condition.

- X dB receiver desensitization, X= [1dB] as starting point

- Conclusion

In this contribution, views on remaining open issues were provided.

Observation 1: Both analog RF components (LPF, ADC) and digital BB processing will impact UE capacity of in-gap interference rejection and maximum tolerated power imbalance between wanted carriers.

Proposal 1: The condition of single FFT operation as following:

  - Aligned numerologies/SCS between wanted carriers

  - The maximum separation of between lower edge of lowest CC and upper edge of highest CC not exceed maximum UE channel bandwidth under corresponding numerology/SCS e.g. 50MHz for 15kHz SCS

  - The carrier spacing among any combination of 2 wanted carriers shall be multiple times of SCS e.g., for bands with 100kHz channel raster, carrier spacing shall be multiple times of 300kHz

Proposal 2: Digital BB filtering shall be considered as per sub-block basis to reject/attenuate the in-gap interference.

Observation 2: 4Rx mandatory is applied for both standalone single CC or CA as captured in TS 38.101-1.

Observation 3: RAN2 PDSCH MIMO layer capability IE is specified as per CC per band per band combination basis.

Proposal 3: UE shall maintain 4Rx mandatory applicable rule as existing specification under intra-band NC CA regardless when operating in fragmented operation.

Proposal 4: Consider 6dBc to 25dBc PSD between wanted carriers and in-gap interference for evaluation the performance impact purpose.

Proposal 5 : Supporting alternative 1 ” Legacy band combination reporting scheme + Rx sharing capability indication”.

  - The indication can be per band per band combinations.

Proposal 6: UE shall allow to indicate separate MIMO layer capacity under single Rx chain operation condition and fallback normal condition on 4Rx optional bands.

Proposal 7: RAN4 shall evaluate the tolerable in-gap interference in terms of absolute power level and PSD

  - These values can be considered as reference to introduce new selectivity/in-band blocking requirements during WI phase if any

Proposal 8: Using receiver desensitization as evaluation criteria for” One RF Rx chain” on/off condition.

  - X dB receiver desensitization, X= [1dB] as starting point

Reference

[1] R4-2420337, WF on NR FR1 DL fragmented carriers study, MTK