3GPP TSG-RAN WG4 Meeting # 114 	R4-2500729

Athens, GR, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.101-1 on MPR applicability for single carrier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For UE UL bandwidth with extension, the MPR reduction could take effect. And the corresponding emission requirements should also apply in a new way. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For UE UL bandwidth with extension, add the corresponding extension bandwidth & RB information, the new inner region and the new emission requirements applying mechanism in 38.101-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR reduction with extended BW could not be applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.4.2.3, 6.5.2.2, 6.5.2.4.1, 6.5.3.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change1 >>

### 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-2, Table 6.2.2-1, Table 6.2.2-4b and Table 6.2.2-5, respectively for channel bandwidths  ≤ 100 MHz.  For UE power class 1.5 with 2Tx, the allowed maximum power reduction  (MPR) is defined in Table 6.2D.2-2 and Table 6.2D.2-3 in accordance with the indicated modifiedMPR-Behaviour specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. For UE power class 1.5 with 4 Tx, the allowed maximum power reduction is defined in Table 6.2D.2-4, 6.2D.2-5. When A UE that indicates PC1.5 for a given band is limited to PC2 by the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply. Unless otherwise specified, ‘pi/2 BPSK’ refers to both variants of pi/2 BPSK referenced in table 6.2.2-1.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth > 4% for TDD bands or > 3% for FDD bands, the ∆MPR is defined in Table 6.2.2-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high)

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 02 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 2: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, thereference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Table 6.2.2-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 | 30 MHz | 0.5 |
| n40 and n97 | Power class 3 Power class 2 | 30 MHz 40 MHz | 1 |
| n71 | Power class 3 Power class 2 | 25 MHz 30 MHz 35 MHz | 0.5 |

Table 6.2.2-4 Void

Table 6.2.2-4a Void

Table 6.2.2-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | (NOTE 1) | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | (NOTE 1) | 0 | 0 |
|  | QPSK | (NOTE 1) | ≤ 1 | 0 |
|  | 16 QAM | (NOTE 1) | ≤ 2 | ≤ 1 |
|  | 64 QAM | (NOTE 1) | ≤ 2.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 4.5 |  |
| CP-OFDM | QPSK | (NOTE 1) | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | (NOTE 1) | ≤ 3 | ≤ 2 |
|  | 64 QAM | (NOTE 1) | ≤ 3.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 6.5 |  |
| NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB. |  |  |  |  |

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel LCRB ≤ 2 RBs, except for PC1 UE supporting other bands than n14.

And for PC1 UE supporting other bands than n14 RB allocation is an Edge RB allocation if

LCRB≤LCRB,edge AND ( RBstart≤RBstart,edge OR RBstart≥NRB-RBstart,edge-LCRB ),

where

LCRB,edge=6, CBW<50 MHz12, CBW≥50 MHz.

For CBW≥70 MHz with DFT-S-OFDM waveform and pi/2-BPSK, QPSK, or 16-QAM modulation, RBstart,edge=1. Otherwise, RBstart,edge=0.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

When ΔPPowerBoost is a positive value and the UE supports powerBoosting-pi2BPSK-QPSK-r18

-	An enhanced power inner allocation region within the inner region is defined so any waveform it contains satisfies the following condition:

RBStart,Low + P1  ≤  RBStart  ≤  RBStart,High - P1

Where P1 = MIN{12,CEIL(2+NRB/25)}

-	When a UE that supports powerBoosting-pi2BPSK-QPSK-r18 but does not support powerBoosting-pi2BPSK-QPSK-Modified-r18 an RB allocation that belongs to the inner region but is outside the enhanced power inner region, the applicable MPR from Table 6.2.2-x is increased by the value of ΔPPowerBoost.

If the UE supports [MPR improvement with BW extension-r19], and when IE [BWExtension -LowerSide-Rel19] and [BWExtension -HigherSide-Rel19] are both set to 1, all the RB allocations are Inner RB allocation.

If the UE supports [MPR improvement with BW extension-r19], and when IE [BWExtension -LowerSide-Rel19] is indicated, the RB allocation is an Inner RB allocation if the following conditions are met:

RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(2/3 *NRB)

If the UE supports [MPR improvement with BW extension-r19], and when IE [BWExtension -HigherSide-Rel19] is indicated, the RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart  ≤ NRB – LCRB, and

LCRB  ≤  ceil(2/3 *NRB)

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap ) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in clause 6.1.2.2 of TS 38.214 [10]. For UE that indicates support for almostContiguousCP-OFDM-UL, the almost contiguous signals in power class 1.5, 2 and 3, the allowed maximum power reduction defined in Table 6.2.2-2, Table 6.2.2-1 and Tables 6.2D.2-1 to 6.2D.2-5 are increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameter of LCRB which is used to specify valid RB allocation ranges for Outer and Inner RB allocations is replaced by (NRB_alloc + NRB_gap) for almost contiguous allocation cases

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2.4 apply.

<< End of change 1 >>

<< Start of change 2 >>

#### 6.4.2.3	In-band emissions

The in-band emission is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain; however, the minimum requirement applies when the in-band emission measurement is averaged over 10 sub-frames. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one or more symbols, accordingly.

The average of the basic in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4.2.3-1.

Unless otherwise specified, pi/2 BPSK in this clause refers to both variants of pi/2 BPSK referenced in Table 6.2.2-1.

Table 6.4.2.3-1: Requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 8: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. |  |  |  |  |

When the IE [BWExtension -HigherSide-Rel19]  or [BWExtension -LowerSide-Rel19]  is set to 1 for a UE supporting the capability of [MPR improvement with BW extension-r19], in-band emission in the extended CBW is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the NR channel bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB outside the NR channel bandwidth but inside the extended channel bandwidth to the UE output power in an allocated RB inside the NR channel bandwidth. The extended channel bandwidth is half the NR channel bandwidth and configured next to the higher edge of the NR channel bandwidth when [BWExtension -HigherSide-Rel19] is set to 1, or configured next to the lower edge of the NR channel bandwidth when [BWExtension - LowerSide-Rel19] is set to 1.

Table 6.4.2.3-2: Requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) | Applicable Frequencies |
| --- | --- | --- | --- |
| General | dB | max⁡{-25-10lg⁡(NRB/LCRB), 20*log10EVM -3-5*∆RB_o-1LCRB, -57dBm + 10log10(SCS/15kHz)- PRB } | Any non-allocated RB outside the NR CBW but inside the extended CBW (NOTE 2) |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB outside the NR CBW but inside the extended CBW. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General) that apply. PRB is defined in NOTE 7. The extended UL CBW is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB outside the NR CBW but inside the extended CBW to the measured average power per allocated RB inside the NR CBW, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB outside the NR CBW but inside the extended CBW to the measured power in the allocated RB inside the NR CBW with highest PSD. The extended CBW is defined in NOTE 9. NOTE 3: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 4: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 5: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 6: ∆RB_o is the frequency offset between the allocated RB and the measured farthest non-allocated RB inside the NR CBW. For NR CBW with full RB allocation, ∆RB_o =0. NOTE 7: PRB is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 8: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. NOTE 9: The extended channel bandwidth is half the NR channel bandwidth and configured next to the higher edge of the NR channel bandwidth when [BWExtension -HigherSide-Rel19] is set to 1, or configured next to the lower edge of the NR channel bandwidth when [BWExtension -HigherSide-Rel19] is set to 1. |  |  |  |

<< End of change 2>>

<< Start of change 3>>

### 6.5.2	Out of band emission

#### 6.5.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.2	Spectrum emission mask

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

When the IE [BWExtension -HigherSide-Rel19]  or [BWExtension -LowerSide-Rel19]  is set to 1 for a UE supporting the capability of [MPR improvement with BW extension-r19], in-band emission specified in Table 6.4.2.3-2 is applied in the frequency range inside the extended channel bandwidth. When [BWExtension -HigherSide-Rel19] is set to 1, the extended channel bandwidth is half the assigned NR channel bandwidth and configured next to the higher edge of the NR channel bandwidth, and the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the + edge of the extended channel bandwidth. When [BWExtension -LowerSide-Rel19] is set to 1, the extended channel bandwidth is half the assigned NR channel bandwidth and configured next to the lower edge of the NR channel bandwidth, and the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the - edge of the extended channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth.

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW1 |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel1 |  |  | -13 |  |  |
| ± BWChannel1 -(BWChannel1+5) |  |  | -25 |  |  |
| NOTE1: When the IE [BWExtension -HigherSide-Rel19] or [BWExtension -LowerSide-Rel19] is set to 1 for a UE supporting the capability of [MPR improvement with BW extension-r19], channel BW is the assigned NR channel bandwidth before extension. |  |  |  |  |  |

<< End of change 3>>

<< Start of change 4>>

#### 6.5.2.4	Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5.2.4.1	NR ACLR

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

When the IE[BWExtension -HigherSide-Rel19]  or [BWExtension -LowerSide-Rel19]  is set to 1 for a UE supporting the capability of [MPR improvement with BW extension-r19], in-band emission specified in Table 6.4.2.3-2 is applied in the frequency range inside the extended channel bandwidth. The extended channel bandwidth is half the assigned NR channel bandwidth and configured next to the higher edge of the NR channel bandwidth when [BWExtension -HigherSide-Rel19] is set to 1, or configured next to the lower edge of the NR channel bandwidth when [BWExtension - LowerSide-Rel19] is set to 1. NRACLR is the ratio of the filtered mean power centered on the assigned NR channel frequency to the filtered mean power centered on one NR channel frequency at nominal channel spacing, this NR channel is adjacent to the extended channel bandwidth with the same bandwidth as the assigned NR channel.

When the IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is set to 1 for a UE supporting the capability of powerBoosting-pi2BPSK-QPSK-r18 or capability of powerBoosting-pi2BPSK-QPSK-Modified-r18, for power class 2 UE, the ACLR requirement of PC2 applies. For power class 3 UE, the ACLR requirement of PC3 applies.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 3,5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

<< End of change 4>>

<< Start of change 5>>

### 6.5.3	Spurious emissions

#### 6.5.3.0	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [9] and NR operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

#### 6.5.3.1	General spurious emissions

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.1-2 apply for all transmitter band configurations (NRB) and channel bandwidths.

When the IE [BWExtension -HigherSide-Rel19]  or [BWExtension -LowerSide-Rel19]  is set to 1 for a UE supporting the capability of [MPR improvement with BW extension-r19], the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the extended channel bandwidth. The extended channel bandwidth is half the assigned NR channel bandwidth and configured next to the higher edge of the NR channel bandwidth when [BWExtension -HigherSide-Rel19] is set to 1, or configured next to the lower edge of the NR channel bandwidth when [BWExtension - LowerSide-Rel19] is set to 1.

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth1 | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel1+ 5 |
| NOTE1: When the IE [BWExtension -HigherSide-Rel19] or [BWExtension -LowerSide-Rel19] is set to 1 for a UE supporting the capability of [MPR improvement with BW extension-r19], BWchannel is the assigned NR channel bandwidth before extension. |  |

Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

<< End of change 5>>