3GPP TSG-RAN4 Meeting #114	R4-2500731

# Athens, GR, February 17 – 21, 2025

Agenda item:			7.1.1.1.1

Source:	vivo

Title:	Discussion on HPUE MSD simplification

Document for:	Discussion

- Introduction

In RAN4#113 meeting, the WF[1] captures the guidelines and assumptions to provide 1UL and 2UL HPUE MSD simplification rules.

Way forward: WF on 1UL HPUE MSD simplification Agreement: - For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, the 1UL HPUE MSD requirements are specified using a delta MSD relative to the default power class 3 MSD as: PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5, - Companies are invited to evaluate two options, - option 1: RAN4 keeps on specifying two sets PC2 requirements [4,5]: - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB, - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB, - option 2: RAN4 specifies a single set PC2 requirement [3]: - one PC2 LUT/equation PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB. - To ensure that 1UL PC1.5 MSD is greater than 1UL PC2 2Tx MSD, companies are invited to derive the HPUE delta MSD LUT/Equations values assuming: - a [9dB] 1Tx Main/Div interference imbalance. Imbalance assumptions less than 9dB are not precluded, - [0.5]dB RevIM (Rtx) and 3dB interferer increase for harmonic/harmonic mixing and for crossband isolation. - Handling of band combination without MSD for PC3 but with small or moderate PC2/PC1.5 MSD: - option 1: FFS LUT rule and FFS on test point configuration - option 2: Legacy HPUE MSD tables may still be used to capture an exception MSD test point - option 3: no HPUE MSD requirement for those cases. - For proponents of LUT simplification approach, FFS the LUT granularity (e.g. 1dB steps) WF on 2UL HPUE MSD simplification Agreement: - Companies are invited to - Provide 2UL HPUE MSD simplification rules to cover the Rel-19 supported 2UL configurations using either equations/LUT or single value de-ratings relative to the default power class 3 MSD, - Study means to minimize the number of LUTs / set of equations that cover all cases. - Check if all Rel-19 cases have been correctly captured and to look at the initial proposals in [4] and [7]. - FFS if only IMD orders are sufficient to design LUT/ set of equations or if both the IMD order and the IMD indexes should be accounted for. - FFS to account for other IMD contributors than just the power amplifier.

- Discussion

  - 1UL HPUE MSD simplification

The following text provides a comparison of the ∆MSD values under different interference conditions based on option 1 of 1UL. Option 1 in the WF[1] is taken from paper[2], and the complete equations are as follows:

| Agreed PC3 MSD | PC21Tx |
| --- | --- |
| PC3MSD ≥ 0.5dB | min(3, ceil(0.38*PC3MSD + 0.2,0.5)) |

| Agreed PC3 MSD | PC22Tx |
| --- | --- |
| PC3MSD > 5dB | min(7.9, ceil(0.3*PC3MSD + 3.6,0.5)) |
| 0.5dB ≤ PC3MSD ≤ 5.0dB | ceil[0.9*PC3MSD] |

| Agreed PC3 MSD | PC1.5 |
| --- | --- |
| PC3MSD > 5dB | min(10.9, ceil(0.55*PC3MSD + 4.3,0.5)) |
| 0.5dB ≤ PC3MSD ≤ 5.0dB | ceil(1.3*PC3MSD + 1.3) |

Tables 1 to 3 detail these comparisons. In these tables, red numbers signify that the calculated ∆MSD values are lower than the specification values, whereas green numbers indicate that the calculated ∆MSD values are equal to or higher than the specification values.

### UL harmonic

Table 1 presents the UL harmonic ∆MSD values of PC2 compared to PC3 for 1T and 2T architectures. Columns 5 and 8 display the current ∆MSD values of 1TX and 2TX derived from the existing specifications, while columns 6 and 9 show the calculated ∆MSD values of 1TX and 2TX based on option 1. Additionally, column 10 contains the ∆MSD values calculated using a new equation proposed for 2TX.

Table 1. PC2 harmonic MSD with 1TX and 2TX

| UL band | DL band | MSD of PC3 | MSD of PC2 with 1TX | Spec ∆PC2, 1TX | calculated ∆PC2, 1TX based on option1 | MSD of PC2 with 2TX | Spec ∆PC2, 2TX | calculated ∆PC2, 2TX based on option1 | calculated ∆PC2, 2TX based on new equation | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (dB) | (dB) |  |  | (dB) |  |  |  |  |
| n3 | n78 | 23.9 | 27.1 | 3.2 | 3 | 32.3 | 8.4 | 7.9 | 8 | UL2/DL1 direct-hit |
| n8 | n41 | 13 | 15.9 | 2.9 | 3 | 20 | 7 | 7.9 | 8 | UL3/DL1 direct-hit |
| n8 | n41 | 4.5 | 6.5 | 2 | 2 | 9.5 | 5 | 5 | 5 | UL3/DL1 direct-hit |
| n8 | n79 | 12 | 14.9 | 2.9 | 3 | 19 | 7 | 7.9 | 8 | UL5/DL1 direct-hit |
| n8 | n79 | 3.8 | 6.2 | 2.4 | 2 | 8.4 | 4.6 | 4 | 5 | UL5/DL1 direct-hit |
| n25 | n77 | 23.9 | 26.9 | 3 | 3 | 31.9 | 8 | 7.9 | 8 | UL2/DL1 direct-hit |
| n66 | n77 | 23.9 | 26.9 | 3 | 3 | 31.9 | 8 | 7.9 | 8 | UL2/DL1 direct-hit |
| n71 | n2510,11 | 6.9 | 9.4 | 2.5 | 3 | 12.9 | 6 | 6 | 6 | UL3/DL1 direct-hit |
| n71 | n41 | 10.8 | 13.6 | 2.8 | 3 | 17.8 | 7 | 7 | 7 | UL4/DL1 direct-hit |
| n71 | n41 | 3.1 | 5.1 | 2 | 2 | 6.1 | 3 | 3 | 5 | UL4/DL1 direct-hit |
| n71 | n77 | 10.4 | 13.2 | 2.8 | 3 | 17.4 | 7 | 7 | 7 | UL5/DL1 direct-hit |

As can be seen from the table, for harmonic products, the PC2 1TX formula of option 1 has a good fit for most cases. For those cases where PC3 MSD is lower, the calculated values ​​are smaller than the spec values, max (spec ∆PC2- calculated ∆PC2) = 0.4dB, max (calculated ∆PC2- spec ∆PC2) = 0.5dB.

For the harmonic MSD product under the PC2 2TX architecture, the fitting value of the option 1’s equation is slightly smaller in the case where the MSD value of PC3 is very high, and the fitting value of the formula is slightly larger in most cases where the MSD value of PC3 is rather low, max (spec ∆PC2- calculated ∆PC2) = 0.6dB, max (calculated ∆PC2- spec ∆PC2) = 0.9dB.

Observation 1: For harmonic products with option 1’s equation, the PC2 1TX calculated ∆MSD has a good fit for most cases, the PC2 2TX calculated ∆MSD is a little bit lower than the spec value at high PC3 MSD.

Based on the above comparison and analysis, the equation of 2TX can be adjusted a little to more accurately support the simplification of PC2 MSD under the high MSD of PC3. The fitting equation is adjusted as follows:

| Agreed PC3 MSD | PC22Tx |
| --- | --- |
| only 1 equation | min(8, ceil(0.3*PC3MSD + 3.6,0.5)) |

Based on the new equation, it could be seen from column 10 that most of the calculated ∆MSDs ​​of 2TX PC2 can achieve better fitting, which are slightly higher than or basically equal to the corresponding spec values. This also leaves a certain margin for the UE implementation.

Proposal 1: For harmonic and harmonic mixing, the equation for the simplified MSD for PC2 is as follows:

PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5,

- PC21TX = min(3, ceil(0.38*PC3MSD + 0.2,0.5))

- PC22TX = min(8, ceil(0.3*PC3MSD + 3.6,0.5))

### Harmonic mixing

Similar to harmonic product, for harmonic products with option 1’s equation, the PC2 1TX calculated ∆MSD has a good fit for most cases, the PC2 2TX calculated ∆MSD is a little bit lower than the spec value at high PC3 MSD. In the few cases where PC3 MSD is between 2 and 3, the calculated ∆MSD value is smaller. Max (spec ∆PC2- calculated ∆PC2) = 1dB, max (calculated ∆PC2- spec ∆PC2) = 1.2dB. For the 2TX architecture, both the equation of option1 and the new equation have a good fit.

Observation 2: For harmonic mixing products with option 1’s equation and new equation of Proposal 1, the PC2 calculated ∆MSD has a good fit for most cases.

Table 2. PC2 harmonic mixing MSD with 1TX and 2TX

| UL band | DL band | MSD of PC3 | MSD of PC2 with 1TX | Spec ∆PC2, 1TX | calculated ∆PC2, 1TX based on option1 | MSD of PC2 with 2TX | Spec ∆PC2, 2TX | calculated ∆PC2, 2TX based on option1 | calculated ∆PC2, 2TX based on new equation | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (dB) |  |  |  |  |  |  |  |  |
| n25 | n41 | 1.3 | 2.2 | 0.9 | 1 |  |  |  |  | UL4/DL3 |
| n25 | n713 | 26.8 | 29.5 | 2.7 | 3 | 34.5 | 7.7 | 7.9 | 8 | UL1/DL3 |
| n39 | n41 | 2.2 | 4.9 | 2.7 | 2 |  |  |  |  | UL4/DL3 |
| n41 | n186 | 26.3 | 29.3 | 3 | 3 |  |  |  |  | UL1/DL3 |
| n41 | n186 | 21.3 | 24.3 | 3 | 3 |  |  |  |  | UL1/DL3 |
| n41 | n39 | 4.3 | 6.1 | 1.8 | 2 |  |  |  |  | UL3/DL4 |
| n41 | n39 | 0.8 | 1.5 | 0.7 | 1 |  |  |  |  | UL3/DL4 |
| n41 | n77 | 8.9 | 11.6 | 2.7 | 3 |  |  |  |  | UL4/DL3 |
| n41 | n77 | 2.1 | 3.6 | 1.5 | 1 |  |  |  |  | UL4/DL3 |
| n41 | n78 | 9.3 | 12.1 | 2.8 | 3 |  |  |  |  | UL4/DL3 |
| n41 | n78 | 2.3 | 3.8 | 1.5 | 2 |  |  |  |  | UL4/DL3 |
| n77 | n2 | 6.7 | 9.2 | 2.5 | 3 |  |  |  |  | UL1/DL2 |
| n77 | n2 | 2.8 | 4.4 | 1.6 | 2 |  |  |  |  | UL1/DL2 |
| n77 | n3 | 5.7 | 8.1 | 2.4 | 3 |  |  |  |  | UL1/DL2 |
| n77 | n5 | 5.7 | 8.1 | 2.4 | 3 |  |  |  |  | UL1/DL4 |
| n77 | n5 | 0.8 | 1.5 | 0.7 | 1 |  |  |  |  | UL1/DL4 |
| n77 | n7 | 14.7 | 17.7 | 3 | 3 |  |  |  |  | UL2/DL3 |
| n77 | n7 | 2.2 | 3.6 | 1.4 | 2 |  |  |  |  | UL2/DL3 |
| n77 | n8 | 5.7 | 8.1 | 2.4 | 3 |  |  |  |  | UL1/DL4 |
| n77 | n8 | 0.8 | 1.5 | 0.7 | 1 |  |  |  |  | UL1/DL4 |
| n77 | n12 | 31 | 34 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n12 | 18 | 21 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n13 | 31 | 34 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n13 | 27.8 | 30.8 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n14 | 31 | 34 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n14 | 27.8 | 30.8 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n25 | 5.6 | 7.9 | 2.3 | 3 |  |  |  |  | UL1/DL2 |
| n77 | n25 | 0.3 | 0.5 | 0.2 | 1 |  |  |  |  | UL1/DL2 |
| n77 | n28 | 31 | 34 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n772 | n29 | 31 | 34 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n772 | n29 | 27.8 | 30.8 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n30 | 10.4 | 13.2 | 2.8 | 3 |  |  |  |  | UL2/DL3 |
| n77 | n30 | 7.6 | 10.2 | 2.6 | 3 |  |  |  |  | UL2/DL3 |
| n77 | n40 | 11.7 | 14.5 | 2.8 | 3 |  |  |  |  | UL2/DL3 |
| n77 | n40 | 3.6 | 5.5 | 1.9 | 2 |  |  |  |  | UL2/DL3 |
| n77 | n41 | 11.7 | 14.5 | 2.8 | 3 |  |  |  |  | UL2/DL3 |
| n77 | n41 | 3.6 | 5.5 | 1.9 | 2 |  |  |  |  | UL2/DL3 |
| n77 | n85 | 31 | 34 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n77 | n85 | 18 | 21 | 3 | 3 |  |  |  |  | UL1/DL5 |
| n78 | n3 | 5.7 | 8.1 | 2.4 | 3 |  |  |  |  | UL1/DL2 |
| n78 | n5 | 5.7 | [8.1] | 2.4 | 3 |  |  |  |  | UL1/DL4 |
| n78 | n8 | 5.7 | 8.1 | 2.4 | 3 |  |  |  |  | UL1/DL4 |
| n78 | n28 | 3 | 34 | 3 | 2 |  |  |  |  | UL1/DL5 |
| n78 | n40 | 3.6 | 4.4 | 0.8 | 2 |  |  |  |  | UL2/DL3 |
| n79 | n8 | 25 | 28.0 | 3 | 3 |  |  |  |  | UL1/DL5 |

### Crossband isolation

For the crossband isolation product, the PC2 1TX equation of option 1 has a good fit for most of the higher PC3 MSD cases. However, the dispersion is higher for some of the cases with PC3 MSD < 3 dB, and some configurations do not fit well. Max (spec ∆PC2- calculated ∆PC2) = 2 dB, max (calculated ∆PC2- spec ∆PC2) = 5.3 dB, and the value is because the MSD for this configuration PC2 is less than PC3 in spec.

The PC2 2TX equation of option 1 has a lower fitting value for most cases of crossband isolation, which is unwilling for the UE to accept because it will bring certain challenges to the corresponding tests, and from table 3 it could be concluded that max (spec ∆PC2- calculated ∆PC2) = 1.4 dB, max (calculated ∆PC2- spec ∆PC2) = 1.1 dB.

Table 3. PC2 crossband isolation MSD with 1TX and 2TX

| UL band | DL band | MSD of PC3 | MSD of PC2 with 1TX | Spec ∆PC2, 1TX | calculated ∆PC2, 1TX based on option1 | MSD of PC2 with 2TX | Spec ∆PC2, 2TX | calculated ∆PC2, 2TX based on option1 | calculated ∆PC2, 2TX based on new equation | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (dB) |  |  |  |  |  |  |  |
| n7 | n3 | 0.6 | 0.4 | -0.2 | 1 | 0.5 | -0.1 | 1 | 4 | >ALCR2 |
| n39 | n41 | 3.3 | 4.7 | 1.4 | 2 |  |  |  |  | >ACLR2 |
| n40 | n41 | 28.1 | 31.1 | 3 | 3 |  |  |  |  | ACLR2 |
| n41 | n1 | 18.1 | 20.8 | 2.7 | 3 |  |  |  |  | >ACLR2 |
| n41 | n3 | 0.6 | 2.3 | 1.7 | 1 |  |  |  |  | >ACLR2 |
| n41 | n25 | 0.6 | 1.6 | 1 | 1 |  |  |  |  | >ACLR2 |
| n41 | n39 | 1.6 | 2.7 | 1.1 | 1 |  |  |  |  | >ACLR2 |
| n41 | n40 | 31.4 | 34.4 | 3 | 3 |  |  |  |  | ACLR2 |
| n41 | n77 | 8.3 | 10.5 | 2.2 | 3 |  |  |  |  | >ACLR2 |
| n66 | n2 | 1.2 | 1.9 | 0.7 | 1 | 3.3 | 2.1 | 2 | 4 | >ACLR2 |
| n66 | n25 | 1.4 | 2.2 | 0.8 | 1 | 3.8 | 2.4 | 2 | 5 | >ACLR2 |
| n71 | n85 | 8.2 | 10.94 | 2.7 | 3 | 15.9 | 7.7 | 7 | 7 | ACLR2 |
| n71 | n85 | 23 | 265 | 3 | 3 | 32.3 | 9.3 | 7.9 | 8 | ACLR1 |
| n77 | n7 | 4.5 | 6.5 | 2 | 2 |  |  |  |  | >ACLR2 |
| n77 | n401 | 4.5 | 6.5 | 2 | 2 |  |  |  |  | >ACLR2 |
| n77 | n401 | 4.5 | 6.5 | 2 | 2 |  |  |  |  | >ACLR2 |
| n77 | n411 | 4.5 | 6.5 | 2 | 2 |  |  |  |  | >ACLR2 |
| n77 | n411 | 4.5 | 6.5 | 2 | 2 |  |  |  |  | >ACLR2 |
| n78 | n7 | 4.5 | 6.5 | 2 | 2 |  |  |  |  | >ACLR2 |
| n78 | n401 | 4.5 | 1.2 | -3.3 | 2 |  |  |  |  | >ACLR2 |
| n783 | n79 | 2 | 5 | 3 | 1 |  |  |  |  | >ACLR2 |
| n783 | n79 | 2 | 5 | 3 | 1 |  |  |  |  | >ACLR2 |
| n79 | n783 | 2.6 | 5.6 | 3 | 2 |  |  |  |  | >ACLR2 |
| n79 | n783 | 2.6 | 5.6 | 3 | 2 |  |  |  |  | >ACLR2 |

Proposal 2: For the HPUE MSD of crossband isolation, it should be simplified using equations independent of harmonic and harmonic mixing.

  - 2UL HPUE MSD simplification

The IMD products’ power is mainly influenced by factors such as changes in transmit power, variations in the power class of each PA, the number of TX paths, and the IMD index of the frequency bands involved in intermodulation distortion. So in total, from PC3 to PC2, the increase in transmitting power naturally increases the IMD product’s power. The improvement in linearity brought about by the increase in PA power class will in turn lead to a reduction in product power. It can be roughly assumed that an increase in PA power level by one level corresponds to a 3 dB increase in IIPn. For IMD products of the same order, the coefficient distribution ratio of the two frequency bands will also affect the product power, especially when the transmitting power or PA of the two UL are different.

From our paper [3] presented in #111, some analysis of the simplification of 2UL MSD is carried out and the corresponding simplification formula is quoted as follows:

MSD'=10*log10(1+10x10(10MSD10-1))                     (1)

MSD’ belongs to the MSD of HPUE, while MSD belongs to current PC3 MSD in the spec. x is the increment of interference power. Assuming all the component linearity assumptions keep unchanged, the MSD of PC2/PC1.5, i.e., MSD’, could be obtained from the deformation of PC3 MSD.

The formula (1) could be transformed into the following form:

MSD'=10lg⁡[10x10(10MSD10+(10-X10-1))]

=x+10lg10MSD10+10-X10-1

=MSD+x+Δ                                       (2)

where  Δ=10lg10MSD10+10-X10-1-MSD

When MSD is higher than 10dB, 10MSD10 is also higher than 10, which is 10 times that of (10-X10-1), so the (10-X10-1) item could be omitted. And furthermore, Δis approximately equal to 0 and could be omitted either.

And the MSD’ in (2) could be simplified to

MSD'≈MSD+x

So when PC3 MSD>10dB, PC2/PC1.5 MSD = PC3 MSD + x, x is the increment of interference power for PC2/PC1.5.

The original intermodulation product power of PC3 is: a and b is index of band1 and 2 respectively.

PIMDn=aP1+bP2-n-1IIPn,  where a+b=n

After power enhancement, for 2TX PC2, the transmitting power will change from 20dBm+20dBm to 23dBm+23dBm, and the intermodulation product power of PC2 is:

P'IMDn=aP1+3+b(P2+3)-(n-1)IIPn

- When IIPn is constant, it is simplified to:

P'IMDn=PIMDn+a+b3

=PIMDn+3n

- When IIPn is also increased by 3dB, it is simplified to:

P'IMDn=PIMDn+a+b3-3(n-1)

=PIMDn+3

Similarly, for 2TX PC1.5 with 26 dBm +26 dBm, the intermodulation product power of PC1.5 is:

P'IMDn=aP1+6+b(P2+6)-(n-1)IIPn

- When IIPn is constant (This is unrealistic because two PC3 PA cannot support the cross-modulation production of PC1.5), it is simplified to:

P'IMDn=PIMDn+a+b6

=PIMDn+6n

- When IIPn is also increased by 3dB, it is simplified to:

P'IMDn=PIMDn+a+b6-3(n-1)

=PIMDn+3+3n

For PC1.5 with 23dBm+27.7dBm, the intermodulation product power of PC1.5 is:

P'IMDn=aP1+3+b(P2+7.7)-(n-1)IIPn

- When IIPn is constant, it is simplified to:

P'IMDn=PIMDn+b*4.7+3n

- When IIPn is also increased by 3dB, it is simplified to:

P'IMDn=PIMDn+b*4.7+3n-3(n-1)

=PIMDn+b*4.7+3

From (2) it could be seen that x is the increment of interference power. Therefore, from PC3 to PC2 with 23+23, ∆PC2= 3n with constant PC3 PA, ∆PC2= 3 for 2 PC2 PA. From PC3 to PC1.5 with 26+26, ∆PC1.5= 6n with constant PC3 PA(non-existent), ∆PC1.5= 3+3n for 2 PC2 PA.

Observation 3: When PC3 MSD>10dB,

- For PC2 of 23dBm+23dBm, PC2=PC3 PA+PC3 PA, 2TX MSD = PC3 MSD +3n, n is the order of IMD product.

- For PC2 of 23dBm+23dBm, PC2=PC2 PA+PC2PA, 2TX MSD = PC3 MSD +3.

- For PC1.5 of 26dBm+26dBm, PC1.5= PC2 PA+PC2PA, 2TX MSD = PC3 MSD+3n+3, n is the order of IMD product.

Above discussion is based on PC3 MSD>10dB, however for low PC3 MSD cases, the fitting of the data is more complicated. So in the spirit of simplifying the discussion and not adding burden to the UE, it is suggested that the above equation can be generalized to all PC2 & PC1.5 MSD simplification scenarios.

For other power combinations, the corresponding simplified formulas are unified in the following table, which is based on the various cases obtained from the WF of the last meeting, and is a relatively complete analysis of the various power and PA combinations of HPUE 2UL MSD.

Proposal 3: For 2UL HPUE MSD, the simplified equation table is recommended as follows:

Table 4. HPUE MSD simplification equation table

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.7 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.7¦b¦+3n | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3order |  |  | 3dB |  | +3dB +3xorder |  | <4.7¦b¦+3n | +3dB +3xorder |

From the table above, case1 is the baseline of PC3 MSD. Cases 2~6 are PC2 ∆MSD, cases 7~10 are PC1.5 ∆MSD.

Case 2 compared to case1, only the transmit power of each band is increased by 3dB, and the power category of PA is unchanged, ∆PC2=3n.

Case 3 compared to case1, the transmit power of each band is increased by 3dB, and the PA’s power class of band2 is increased from PC3 to PC2, so the overall IMD product increase is less than 3n, i.e., ∆PC2<3n. If only one band’s PA power class is increased, the overall delta MSD will be smaller than all the PA power class remain unchanged.

Observation 4: When the transmit power of each link remains unchanged, ∆MSD of the case where all PA power classes are raised is the largest, ∆MSD of the case where all PA power classes remain unchanged is the smallest, and ∆MSD of the case where only some PA power classes are raised is an intermediate value.

Case 4 compared to case3, the main difference is that the architecture of band2 is divided into 2TX:

- When the index of band2 is 1, ignoring the effect of MRC, the increment of IMD products in the two parts can be considered equal.

- When the index of band2 is not 1, the IMD products of the two parts are different. To simplify the detail of the classification, ∆PC2 of 2TX could be reused.

Proposal 4: For IMD cases of R17/18 PC2 with 2TX or 3TX (case2~4), PC2 MSD = PC3 MSD + 3* IMD order.

Case 5 compared to case 1, the transmitting power of each band is increased by 3dB, and the PA power class of each band is increased from PC3 to PC2, the overall IMD product is increased by = 3n-3 (n-1) =3, i.e., ∆PC2=3dB.

Case 6 compared to case 5, the main difference is that the architecture of band2 is divided into 2TX, ignoring the influence of MRC, it can be considered that the increment of IMD products in the two parts is equal, i.e., ∆PC2=3dB.

Proposal 5: For IMD cases of R19 PC2 with 2TX or 3TX (case5~6), PC2 MSD = PC3 MSD + 3.

Case 7 compared to case 1, the power of both bands is increased by 6dB, and the PA is changed from PC3 to PC2, so the increment is 6n-3(n-1)=3n+3, i.e., ∆PC1.5=3n+3, n is the order of IMD product.

Case 8 compared to case 7, the main difference is that the architecture of band2 is divided into 2TX, ignoring the influence of MRC, it can be considered that the increment of IMD products in the two parts is equal i.e., ∆PC1.5=3n+3, n is the order of IMD product.

Case 9: For PC1.5's 23+24.7+24.7 R18 architecture, it is necessary to consider both the index and order of the IMD product to calculate ∆MSD. Compared to case1, the power of band1 and 2 increases by 3dB and 7.7dB respectively, and the PA in band2 is also upgraded by one power class level, which means its linearity is also optimized. So, if ignoring the MRC difference between 2TX and 3TX, for band 1, ∆MSD = 3n+b*|4.7|; for band2, ∆MSD = 3n+b*|4.7| - 3*(n-1)=|b|*4.7+3. Totally, ∆PC1.5<3n+|b|*4.7, b is index of band2.

Case10: Compared to case4, the power of band1 and 2 both increases by 3dB, and the PA in band1 and band2 is also upgraded by one power class level, so MSD increment from case 4 is 3 ignoring MRC issue, i.e.,  ∆PC1.5<3n+3, n is the order of IMD product. It could also be merged with case7&8 to leave some margin for calculation.

Proposal 6: For IMD cases of R19 PC1.5 with 2TX or 3TX (case7~8,10), PC1.5 MSD = PC3 MSD + 3 + 3* IMD order.

Proposal 7: For IMD cases of R18 PC1.5 with 3TX (case9), PC1.5 MSD < PC3 MSD + 3* IMD order+4.7*|IMD index of PC1.5 band|.

The following table is the comparison of the calculated ∆MSD and spec ∆MSD of PC2 and PC1.5. The calculated MSD uses the equation of PC2 MSD = PC3 MSD + 3* IMD order, and PC1.5 MSD < PC3 MSD + 3* IMD order+4.7*|IMD index of PC1.5 band|. It could be seen from Table 4 that except for very few configurations (n8-n79), the equation for PC2 is still a good fit for most band combinations while leaving some margin for realization, max (spec ∆PC2- calculated ∆PC2) < 1dB.

For PC1.5, except for very few configurations (n2 - n77), the equation for PC1.5 is also a good fit for most band configurations, max (spec ∆PC1.5- calculated ∆PC1.5) <1dB. However, for the few configurations with larger ∆MSD in the spec, considering the specific device parameters when the indicator was formulated, it should be discussed whether to retain its higher HPUE MSD value as a special case outside of the unified simplified solution of the fitting formula. At the same time, the fitting value of each simplified formula of HPUE should be as close to or greater than the spec value as possible so as not to increase the burden of UE, while leaving a reasonable implementation margin.

Proposal 8: The fitting value of each simplified calculation equation of HPUE should be greater than or equal to the spec value.

Proposal 9: For the rare configurations of HPUE MSD in the spec that do not conform well to the unified equation, it should be discussed whether to maintain their existing MSD values as special cases, separate from the simplified solution of the fitting equation.

Table 4: PC2 and PC1.5 HPUE MSD for 2DL/2UL config

| NR CA config |  | index ¦a¦+¦b¦ | PC3 MSD | PC2 MSD | PC1.5 MSD | Source of IMD | PC2 Delta MSD |  | PC1.5 Delta MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DLCA | ULCA |  | 20+20 | 23+ 23/2x20 | 23+ (2x24.7) |  | spec | vivo calc | spec | vivo calc, <the values below |
| n1-n77 | n1-n77 |  | 29.8 | 35.8 |  | 2B_IMD2 | 6 | 6 | N/A | N/A |
| n2-n77 | n2-n77 | 1+1 | 26 | 32.1 | 35.2 | 2B_IMD2 | 6.1 | 6 | 9.2 | 10.7 |
| n3-n77 | n3-n77 |  | 26 | 31.9 |  | 2B_IMD2 | 5.9 | 6 | N/A | N/A |
| n3-n78 | n3-n78 |  | 26 | 31.9 |  | 2B_IMD2 | 5.9 | 6 | N/A | N/A |
| n25-n77 | n25-n77 | 1+1 | 26 | 32.1 | 37.5 | 2B_IMD2 | 6.1 | 6 | 11.5 | 10.7 |
| n25-n78 | n25-n78 |  | 26 | 32.1 |  | 2B_IMD2 | 6.1 | 6 | N/A | N/A |
| n66-n77 | n66-n77 | 1+1 | 31 | 34.33 | 40 | 2B_IMD2 | 3.33 | 6 | 9 | 10.7 |
| n70-n77 | n70-n77 | 1+1 | 31 | 37 | 41.8 | 2B_IMD2 | 6 | 6 | 10.8 | 10.7 |
| n18-n41 | n18-n41 | 2+1 | 15.6 | 24.6 |  | 2B_IMD3 | 9 | 9 | N/A | N/A |
| n3-n41C | n3-n41C |  | 15.3 | 23.3 |  | 3CC_IMD3 | 8 | 9 | N/A | N/A |
| n1-n77 | n1-n77 |  | 8 | 17.8 |  | 2B_IMD4 | 9.8 | 12 | N/A | N/A |
| n1-n78 | n1-n78 |  | 8 | 17.8 |  | 2B_IMD4 | 9.8 | 12 | N/A | N/A |
| n2-n77 | n2-n77 | 3+1 | 8 | 19.1 | 26.4 | 2B_IMD4 | 11.1 | 12 | 18.4 | 16.7 |
| n3-n41 | n3-n41 |  | 8.2 | 18.4 |  | 2B_IMD4 | 10.2 | 12 | N/A | N/A |
| n3-n77 | n3-n77 |  | 8 | 18.5 |  | 2B_IMD4 | 10.5 | 12 | N/A | N/A |
| n3-n78 | n3-n78 |  | 8 | 18.5 |  | 2B_IMD4 | 10.5 | 12 | N/A | N/A |
| n5-n77 | n5-n77 | 3+1 | 8.3 | 18.6 | 25.5 | 2B_IMD4 | 10.3 | 12 | 17.2 | 16.7 |
| n5-n78 | n5-n78 |  | 8.3 | 18.6 |  | 2B_IMD4 | 10.3 | 12 | N/A | N/A |
| n7-n77 | n7-n77 | 2+2 | 7.1 | 15.8 | 29.7 | 2B_IMD4 | 8.7 | 12 | 22.6 | 21.4 |
| n8-n78 | n8-n78 | 3+1 | 8.3 | 15.5 |  | 2B_IMD4 | 7.2 | 12 | N/A | N/A |
| n18-n77 | n18-n77 |  | 8.4 | 18.4 |  | 2B_IMD4 | 10 | 12 | N/A | N/A |
| n26-n78 | n26-n78 | 3+1 | 11.1 | 23.8 |  | 2B_IMD4 | 12.7 | 12 | N/A | N/A |
| n30-n77 | n30-n77 |  | 8 | 17.6 |  | 2B_IMD4 | 9.6 | 12 | N/A | N/A |
| n41-n71 | n41-n71 | 3+1 | 11 | 16.3 | 25.4 | 2B_IMD4 | 5.3 | 12 | 14.4 | 16.7 |
| n2-n77 | n2-n77 | 3+2 | 5 | 20 | 28 | 2B_IMD5 | 15 | 15 | 23 | 24.4 |
| n8-n79 | n8-n79 |  | 4.8 | 21.5 |  | 2B_IMD5 | 16.7 | 15 | N/A | N/A |
| n12-n77 | n12-n77 |  | 5.5 | 11.7 |  | 2B_IMD5 | 6.2 | 15 | N/A | N/A |
| n13-n77 | n13-n77 |  | 5.5 | 20.5 |  | 2B_IMD5 | 15 | 15 | N/A | N/A |
| n14-n77 | n14-n77 |  | 5.5 | 11.7 |  | 2B_IMD5 | 6.2 | 15 | N/A | N/A |
| n18-n77 | n18-n77 |  | 4.5 | 11.7 |  | 2B_IMD5 | 7.2 | 15 | N/A | N/A |
| n28-n77 | n28-n77 |  | 5.5 | 19.2 |  | 2B_IMD5 | 13.7 | 15 | N/A | N/A |
| n66-n77 | n66-n77 | 3+2 | 5 | 11.27 | 26 | 2B_IMD5 | 6.27 | 15 | 21 | 24.4 |
| n70-n77 | n70-n77 | 3+2 | 5 | 18.4 | 27.9 | 2B_IMD5 | 13.4 | 15 | 22.9 | 24.4 |
| n71-n78 | n71-n78 |  | 5.5 | 11.4 |  | 2B_IMD5 | 5.9 | 15 | N/A | N/A |

- Conclusion

Observation 1: For harmonic products with option 1’s equation, the PC2 1TX calculated ∆MSD has a good fit for most cases, the PC2 2TX calculated ∆MSD is a little bit lower than the spec value at high PC3 MSD.

Proposal 1: For harmonic and harmonic mixing, the equation for the simplified MSD for PC2 is as follows:

PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5,

- PC21TX = min(3, ceil(0.38*PC3MSD + 0.2,0.5))

- PC22TX = min(8, ceil(0.3*PC3MSD + 3.6,0.5))

Observation 2: For harmonic mixing products with option 1’s equation and new equation of Proposal 1, the PC2 calculated ∆MSD has a good fit for most cases.

Proposal 2: For the HPUE MSD of crossband isolation, it should be simplified using equations independent of harmonic and harmonic mixing.

Observation 3: When PC3 MSD>10dB,

- For PC2 of 23dBm+23dBm, PC2=PC3 PA+PC3 PA, 2TX MSD = PC3 MSD +3n, n is the order of IMD product.

- For PC2 of 23dBm+23dBm, PC2=PC2 PA+PC2PA, 2TX MSD = PC3 MSD +3.

- For PC1.5 of 26dBm+26dBm, PC1.5= PC2 PA+PC2PA, 2TX MSD = PC3 MSD+3n+3, n is the order of IMD product.

Proposal 3: For 2UL HPUE MSD, the simplified equation table is recommended as follows:

Table 4. HPUE MSD simplification equation table

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.7 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.7¦b¦+3n | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3order |  |  | 3dB |  | +3dB +3xorder |  | <4.7¦b¦+3n | +3dB +3xorder |

Observation 4: When the transmit power of each link remains unchanged, ∆MSD of the case where all PA power classes are raised is the largest, ∆MSD of the case where all PA power classes remain unchanged is the smallest, and ∆MSD of the case where only some PA power classes are raised is an intermediate value.

Proposal 4: For IMD cases of R17/18 PC2 with 2TX or 3TX (case2~4), PC2 MSD = PC3 MSD + 3* IMD order.

Proposal 5: For IMD cases of R19 PC2 with 2TX or 3TX (case5~6), PC2 MSD = PC3 MSD + 3.

Proposal 6: For IMD cases of R19 PC1.5 with 2TX or 3TX (case7~8,10), PC1.5 MSD = PC3 MSD + 3 + 3* IMD order.

Proposal 7: For IMD cases of R18 PC1.5 with 3TX (case9), PC1.5 MSD < PC3 MSD + 3* IMD order+4.7*|IMD index of PC1.5 band|.

Proposal 8: The fitting value of each simplified calculation formula of HPUE should be greater than or equal to the spec value.

Proposal 9: For the rare configurations of HPUE MSD in the spec that do not conform well to the unified equation, it should be discussed whether to maintain their existing MSD values as special cases, separate from the simplified solution of the fitting equation.

- References

- R4-2420380, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#113.

- R4-2419699, 1UL HPUE NR-CA MSD Simplifications for Rel-19, Skyworks Solutions, Inc. RAN4#113.

- R4-2408131, Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx, vivo, #111.