3GPP TSG-RAN WG4 Meeting # 114 	R4-2500743

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on the system parameter of AIoT

Agenda Item:	7.24.1

Document for:	Approval

- Introduction

RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

  - Use band n8 as an example band

In [1], the WID for R19 AIoT was approved:

The system parameter is not explicitly included in the WID scope, but in our understanding, it should be a part of WI and in this contribution, we provide our preliminary consideration.

- Discussion

  - System parameter for R2D

- R2D Channel bandwidth

-	Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. For an OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D ≤ twelve PRBs, and would be down-selected among:

-	Alt 1: Including 180 kHz, 360 kHz

-	Alt 2: Integer multiple(s) of 180 kHz

-	Alt 3: Integer multiple(s) of the subcarrier spacing

-	Occupied bandwidth, Bocc,R2D from a reader perspective: The frequency resources used for transmitting R2D, and potential guard band.

-	Bocc,R2D ≥ Btx,R2D.

In [2], RAN1 give R2D channel bandwidth definition and potential range, as shown in below:

It is noted that the “occupied bandwidth” in RAN1 is equal to RAN4 channel bandwidth which is including transmission bandwidth and guard band. The following table also shows the required RB number for different date rate

Table 1: Starting point for M values and the associated minimum Btx,R2D value [2]

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

The transmission bandwidth for R2D is 1~12 RB based on TR. Since the AIoT has different use case in the field which may require various date rate, it is beneficial to define several bandwidths in the spec to allow operator to chose when deploy the real network. Based on the table above, 1RB, 4RB, 12RB can be considered.

Observation 1: The range of transmission bandwidth in TR is 1~12 dB, which has influence on the peak data rate.

For NB-IoT, the spectrum utilization is 90% (180kHz/200kHz) and the SU of wider bandwidth usually can be higher from filter design perspective. In NR 3MHz, the SU is also 90% (15RB/3MHz), so 90% SU for AIoT different bandwidth seems to be feasible.

Proposal 1: It is suggested RAN4 to discuss the candidate channel bandwidth and corresponding transmission bandwidth for R2D, and 200kHz/1RB, 800kHz/4RB, 2.4MHz/12RB with 15kHz SCS can be considered.

- R2D Channel raster and channel spacing

No matter for in-band operation or standalone operation, the channel raster and channel spacing are still needed. For in-band operation, the AIoT RB allocation should be orthogonal with NR carrier to avoid potential interference. For standalone, it is also an easier way to reused same raster and spacing for deployment.

Proposal 2: Same channel raster and channel spacing as NR can be reused for AIoT R2D.

2.2 System parameter for D2R

- D2R Channel bandwidth

The D2R is not OFDM-based waveform, and the definition will be quite different from NR/LTE. In [2], there are two possible transmissions for D2R:

Figure 1 1SB transmission and 2SB transmission in [2]

The difference between 1SB and 2SB transmission is similar to SSB-ASK and DSB-ASK, and the 1SB transmission is achieved by using additional filter or phase shift, which should be hard for such low-cost device 1, so only the 2SB transmission need to be considered for device 1.

Another discussion point related to the bandwidth definition is the small frequency shift. For a given chip length, Btx,D2R can be derived by the chip duration and small frequency shift factor and different Btx,D2R values represent variable data rates. Similar to R2D, it is better to support several data rates to leave more flexibility for the real network. For the Bocc,D2R , sufficient guard band should be reserved to cover SFO.

Proposal 3: The D2R bandwidth for device 1 is defined based on 2SB transmission and at least the following aspects need to be considered:

- D2R maximum data rate

- Guard band due to the impact of SFO

- D2R Channel raster and channel spacing

The device actually cannot choose the center frequency of D2R which depends on CW, so it is quite meaningless to define channel raster for D2R. As mentioned in previous part, the device1 can support small frequency shift, and the “channel spacing” for D2R can also be implicitly indicated

Observation 2: The CW frequency and small frequency shift of device can implicitly indicate how the D2R spectrum will be allocated

Proposal 4: It is unnecessary to define channel raster and channel spacing for D2R.

- Conclusion

In this contribution, we provide our preliminary views on A-IoT system parameter.

Observation 1: The range of transmission bandwidth in TR is 1~12 dB, which has influence on the peak data rate.

Proposal 1: It is suggested RAN4 to discuss the candidate channel bandwidth and corresponding transmission bandwidth for R2D, and 200kHz/1RB, 800kHz/4RB, 2.4MHz/12RB with 15kHz SCS can be considered.

Proposal 2: Same channel raster and channel spacing as NR can be reused for AIoT R2D.

Proposal 3: The D2R bandwidth for device 1 is defined based on 2SB transmission and at least the following aspects need to be considered:

- D2R maximum data rate

- Guard band due to the impact of SFO

Observation 2: The CW frequency and small frequency shift of device can implicitly indicate how the D2R spectrum will be allocated

Proposal 4: It is unnecessary to define channel raster and channel spacing for D2R.

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

- 3GPP TR 38.769