3GPP TSG-RAN WG4 Meeting # 114 	R4-2500744

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source: 	vivo

Title: 	Discussion on the RF requirement for AIoT BS

Agenda Item:	7.24.2.1

Document for:	Approval

- Introduction

RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

  - Use band n8 as an example band

In [1], the WID for R19 AIoT was approved:

In this contribution, we provide our views on the RF requirements for AIoT BS.

- Discussion

  - Tx Requirement

- Transmit ON/OFF power

The transmit ON/OFF power include the transmit OFF power and transient period and usually only apply to BS with TDD operation. In R19 WI, the CW is located in UL spectrum, so the R2D is transmitted in DL spectrum and the D2R is transmitted in UL spectrum, which is same as current FDD operation.

Observation 1: Since the CW is transmitted in UL spectrum, the R19 AIoT BS operate same as the normal NR FDD BS, i.e., transmit signal (R2D) in DL spectrum and receive signal (D2R) in UL spectrum.

In [2], several timing relationships are identified in RAN1:

A-IoT processing time aspects are studied in terms of the following timing relationships:

TR2D_min:	Minimum time between a R2D transmission and the corresponding D2R transmission following it.

TD2R_min:	Minimum time between a D2R transmission and the corresponding R2D transmission following it.

TD2R_max:	Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions.

TR2D_R2D_min:	Minimum time between two different consecutive R2D transmissions to the same A-IoT device.

TD2R_D2R_min:	Minimum time between two different consecutive D2R transmissions from the same A-IoT device.

In our understanding, these timing relationships mainly for scheduling control purpose which is not need to be defined from RF hardware perspective, so we think it is unnecessary to define the transmit ON/OFF power requirement in this release.

Proposal 1: It is unnecessary to define the transmit ON/OFF power requirement for AIoT BS in R19.

- Transmit signal quality

As described in WID, only OOK-4 need to be supported for R2D, so the modulation quality needs to be guaranteed.

For OOK, such waveform is not used in traditional LTE and NR system, and it is not clear how to define the modulation quality for OOK since the EVM is usually used for QAM signal.

Observation 2: For OOK, there is no precedent of modulation quality requirement and a new requirement need to be introduced.

From OOK demodulation perspective, the OOK need the power difference between ON chip and OFF chip is clear enough, so one possible way to define modulation quality is the power ratio between ON chip and OFF chip. We can also try to use similar way as EVM in time domain. There is reference point for QAM signal for EVM calculation and for OOK, the amplitude 1 & 0 can be used as ideal signal for normalized transmitted signal. In addition, considering similar discussion exist in LP-WUS, we can also wait the conclusion.

Proposal 2: Further discuss the following requirement options for OOK modulation quality:

Option 1: Power ratio between ON chip and OFF chip.

Option 2: The EVM of OOK is defined as the average amplitude difference between normalized transmitted signal and the ideal 1&0 reference signal.

Option 3: Wait for the conclusion of LP-WUS.

  - Rx Requirement

The performance metric for Rx requirement should be decided first. For LTE/NR, 5% throughput loss of reference channel is used which is not suitable for AIoT device. In LP-WUS discussion, the miss detection rate was agreed as metric for Rx requirement, which can be considered used for device. Since in SI, 10% BLER was agreed for LLS evaluation, we think this can also be used for Rx requirement design.

Observation 3: For LP-WUR, the Rx requirement is defined based on 10% miss detection rate.

Proposal 3: It is suggested that the Rx requirement of AIoT BS is defined based on miss detection rate.

- REFSENS

During SI stage, the REFSENS formula was agreed which is same as legacy NR:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

-	BW is the transmission bandwidth for D2R.

-	NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase.

-	Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology.

The main undecided parameter is SNR, and it is necessary to perform LLS to derive it. Considering the short time for R19 AIoT WI, the simulation assumption should be aligned as soon as possible.

Proposal 4: The following simulation assumption can be the starting point for D2R SNR evaluation

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Device velocity | 3 km/h |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 2 |
| Reference data rate | 1 kbps (M) 5 - 7 kbps (M) |
| Message size | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size |
| BLER target | 10% |
| Sampling frequency | Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1 |
| Transmission bandwidth | DSB X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz X = {[15 (M)], [180 (O)]} |
| [OOK/BPSK chip rate] | Companies to report |
| Receiver bandwidth | FFS |
| Waveform (CW) | unmodulated single tone |
| Modulation | OOK |
| Line code | Manchester encoding |
| FEC | No FEC |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |

- Conclusion

In this contribution, we share our initial views on AIoT BS RF requirement.

Observation 1: Since the CW is transmitted in UL spectrum, the R19 AIoT BS operate same as the normal NR FDD BS, i.e., transmit signal (R2D) in DL spectrum and receive signal (D2R) in UL spectrum.

Proposal 1: It is unnecessary to define the transmit ON/OFF power requirement for AIoT BS in R19.

Observation 2: For OOK, there is no precedent of modulation quality requirement and a new requirement need to be introduced.

Proposal 2: Further discuss the following requirement options for OOK modulation quality:

Option 1: Power ratio between ON chip and OFF chip.

Option 2: The EVM of OOK is defined as the average amplitude difference between normalized transmitted signal and the ideal 1&0 reference signal.

Option 3: Wait for the conclusion of LP-WUS.

Observation 3: For LP-WUR, the Rx requirement is defined based on 10% miss detection rate.

Proposal 3: It is suggested that the Rx requirement of AIoT BS is defined based on miss detection rate.

Proposal 4: The following simulation assumption can be the starting point for D2R SNR evaluation.

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Device velocity | 3 km/h |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 2 |
| Reference data rate | 1 kbps (M) 5 - 7 kbps (M) |
| Message size | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size |
| BLER target | 10% |
| Sampling frequency | Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1 |
| Transmission bandwidth | DSB X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz X = {[15 (M)], [180 (O)]} |
| [OOK/BPSK chip rate] | Companies to report |
| Receiver bandwidth | FFS |
| Waveform (CW) | unmodulated single tone |
| Modulation | OOK |
| Line code | Manchester encoding |
| FEC | No FEC |
| ADC bit width | Ideal |
| D2R receiver | Coherent receiver |

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

- 3GPP TR 38.769

- ETSI EN 302 208