3GPP TSG-RAN4 Meeting #114 R4-2500749 # Athens, GR, 17th – 21st February, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.870; CR; 0018; rev; -; Current version:; 18.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; CR to 38.870 on XR OTA test method. Source to WG:; vivo. Source to TSG:; R4. Work item code:; TRP_TRS_MIMO_OTA_Ph3-Core; Date:; 2025-02-05. Category:; B; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; This CR provides content to accommodate XR test method and agreed alternative test methods for XR devices. The draft CR on XR alternative methodologies was endorsed in R4-2419806.. Summary of change:; Extend the general test method to cover XR testing. Add agreed low UL power method as alternative methodology.. Consequences if not approved:; The spec is not appicable for XR UE testing.. Clauses affected:; 6.6, 7.4.2.1, 7.4.2.2, 7.5.2.1, 7.5.2.2, 10, D.4. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

## <<< START OF CHANGES >>>

6.6	Head phantom for Head-mounted devices

6.6.1	General

TBD

6.6.2	Head mounted devices positioning guidance

TBD

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.4.2	TRP test procedure for NR 1Tx configuration

7.4.2.1	UE configuration

For devices containing multiple Tx antennas, the Tx Antenna Switching (TAS) function should be OFF, and the TRP should be measured for each Tx antenna individually. The antenna with better TRP is identified as the primary antenna, and the corresponding TRP result will be used to determine the pass/fail compliance. Otherwise, the primary antenna should be selected based on manufacturer declaration. To ensure the TAS OFF testing, the manufacture should provide either software/guidance to lab to control which Tx antenna is used, or the pre-configured DUT locked at primary antenna.

For Standalone, the NR System Simulator (SS) and DUT shall be configured per TS 38.521-1 [5], section 6.2.1 (UE maximum output power) using the default settings specified in TS 38.521-1 [5] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band for different UE types, as defined in Clause 4.3.

For EN-DC, the SS and DUT shall be configured per TS 38.521-3 [6], Section 6.2B.1 (UE Maximum Output Power for EN-DC) using the default settings specified in TS 38.521-3 [6] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in Clause 4.3. The UL output power of LTE carrier should be set as a constant power of 10dBm, while measuring NR at maximum output power, i.e., with fixed p-MaxEUTRA-r15=10 dBm, and p-NR-FR1 not configured.

7.4.2.2	Test procedure

The 1Tx radiated power test procedure is general for different UE types, e.g., smartphone, RedCap, and head-mounted devices.

For UE configured with 1Tx for NR carrier, SA or EN-DC mode, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-1 [5] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The TRP value is calculated using the TRP integration approaches outlined in Clause 5.1.1.2.

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.5.2	TRS test procedure for NR 1Tx configuration

7.5.2.1	UE configuration

For Standalone, the NR System Simulator (SS) and DUT shall be configured per section 7.3.2 (Reference sensitivity power level) of TS 38.521-1 [5] using the defaults specified in TS 38.521-1 [5] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band for different UE types, as defined in Clause 4.

For EN-DC, the EN-DC SS and DUT shall be configured per section 7.3B.2 (Reference Sensitivity for EN-DC) of TS 38.521-3 [6], using the defaults specified in TS 38.521-3 [6] and TS 38.508-1 [7], as applicable. The measurement should be carried out based on the detailed test parameters for each band, as defined in Clause 4. The UL power configuration for LTE and NR is 50%-50% power splitting, i.e.,

-	For PC3, p-MaxEUTRA-r15=20 dBm, and p-NR-FR1= 20dBm;

-	or PC2, p-MaxEUTRA-r15=23 dBm, and p-NR-FR1= 23dBm.

Different from handheld UE, a Redcap UE is required to be equipped with a minimum of single Rx antenna and maximum of two Rx antenna.

7.5.2.2	Test Procedure

The radiated sensitivity test procedure with 1Tx configuration is general for different UE types, e.g., smartphone, RedCap, and head-mounted devices.

For UE configured with 1Tx for NR carrier, SA or EN-DC mode, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 4 in section 7.3.2.4.2 of TS 38.521-1 [5], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

The TRS value is calculated using the equation outlined in Clause 5.2.2.

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

10	Alternative methodologies to resolve battery issue

10.1	General

This Clause define alternative test methodologies to resolve the battery issue of devices during TRP/TRS testing.

10.2	Low UL power solution

The low UL power solution for XR device is a Single Point Offset Test (SPOT) approach for TRS measurements. To reduce the power battery issue of XR devices under maximum transmission condition, SPOT with maximum transmit power can be performed and the end of the measurement to scale the TRS result measured under low UL power. The low transmit power is set as 10 dBm. The final TRS is achieved by offsetting the TRS from the delta at Peak EIS, using both maximum output power (single point EIS) and low transmit power (full EIS).

The detailed test procedure is as following:

1)	Step 1: perform the TRS measurement based on the procedure defined in clause 7.5, with one exception that the UL power of DUT is configured as 10dBm. TRS value is calculated using the equation and integration approaches outlined in TS 38.161 Clause A.3.5.2. This TRS is called TRSlow_power.

2)	Step 2: identify the peak EIS direction from step 1. The EIS obtained at the peak position is called EISlow_power. Configure the UE to transmit at full power to measure the EISmax_power at the same peak position identified from Step 2.

3)	Step 3: calculate the delta value at peak EIS direction with delta= EISlow_power - EISmax_power, then get the final TRS at maximum output power based on the equation TRSMax_power = TRSlow_power - delta.

FFS whether above test procedure can also be applied to Wearable Redcap UE.

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

D.4	Head Phantom for head-mounted devices

TBD

## <<< END OF CHANGES >>>