3GPP TSG-RAN4 Meeting #114 R4-2500750 # Athens, GR, 17th – 21st February, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.870; CR; Draft; rev; -; Current version:; 18.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; Draft CR to 38.870 on NTN OTA test method. Source to WG:; vivo. Source to TSG:; R4. Work item code:; TRP_TRS_MIMO_OTA_Ph3-Core; Date:; 2025-02-05. Category:; B; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; This CR provides content to accommodate NTN test method and performance metric.. Summary of change:; Add sub-clause for NR-NTN and IoT-NTN TRP testing.. Consequences if not approved:; The spec is not appicable for NTN UE testing.. Clauses affected:; 7.4.5, 7.4.6. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.4.5	TRP test procedure for NR NTN

7.4.5.1	UE configuration

NR-NTN TRP and TRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

The NR NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test.

For NR NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 38.521-5 [31], section 6.2.1 (UE maximum output power) using the default settings specified in TS 38.521-5 [31] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-5 Clause 4.3.

7.4.5.2	Test procedure

For NR-NTN UE support usage scenario 1 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [31] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The TRP value is calculated using the TRP integration approaches outlined in Clause 5.1.1.2.

For NR-NTN UE only support usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [31] and ensure the DUT transmits with its maximum power.

3)	Measure the power at TBD measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The performance metric is TBD.

7.4.6	TRP test procedure for IoT-NTN

7.4.6.1	UE configuration

IoT-NTN TRP and TRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

The IoT-NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test. For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

For cat M1 NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 6.2.A.1 (UE maximum output power for category M1) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-7 Clause 4.3.

For NB-IoT NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 6.2.B.1 (UE maximum output power for category NB1 and NB2) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-9 Clause 4.3.

7.4.6.2	Test procedure for cat M1 NTN

For cat M1 NTN UE support usage scenario 1 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The TRP value is calculated using the TRP integration approaches outlined in Clause 5.1.1.2.

For cat M1 NTN UE only support usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [32]and ensure the DUT transmits with its maximum power.

3)	Measure the power at TBD measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The performance metric is TBD.

7.4.6.3	Test procedure for cat NB-IoT NTN

For NB-IoT NTN UE support usage scenario 1 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The TRP value is calculated using the TRP integration approaches outlined in Clause 5.1.1.2.

For NB-IoT NTN UE only support usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [32]and ensure the DUT transmits with its maximum power.

3)	Measure the power at TBD measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The performance metric is TBD.

## <<< END OF CHANGES >>>