3GPP TSG-RAN WG4 Meeting # 114	R4- 2500773

# Athens, GR, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of SSB adaptation for Rel-19 NES

Agenda item:	7.25.2.2

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of adaptation of SSB for Rel-19 NES.

Discussion

In the previous meeting, following agreement was captured in WF[2].

RAN4#112bis Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement(RAN4#112bis): - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement(RAN4#112bis): - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion. RAN4#113 Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. Issue 1-4-1: SCell activation requirement for SSB adaptation - No requirements when SSB adaptation of SSB periodicity happens during SCell activation. - FFS the requirements impact when SSB adaptation happen before and/or at the same time as SCell activation. RAN1#119 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Obj 3 Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Scenario clarification based on RAN1 progress

In RAN4#112bis meeting, scenarios and impacted requirements has been agreed, and sent LS to RAN2 regarding SMTC configuration . In addition, requirements of transition period between SSB adapatation indication has been discussed, but not captured in WF.

Until now, basic mechanisms of SSB adapatation are still open. However, there was not much progress in the previous meeting. Only precluding IDLE mode was concluded. Given the situation, we think RAN4 needs to focus on L1 measurement and transition period first. Once RAN2 decides whether to introduce new SMTC configuration, L3 measurement can be revisited.

According to agreement in previous RAN1 meeting, PCell is not required to be supported for SSB adaptation. So wet think PCell can be precluded

Proposal 1: Defer L3 measurement requirement pending on RAN2 progress

Proposal 2: RAN4 to preclude PCell from the scenario

L1 measurement requirements

In general, L1 measurement can be happened before SSB adaptation and after SSB adaptation. As shown in Table 1, in the legacy requirement of L1 measurement, the period of SSB is used for the requirements. For L1-RSRP, L1-SINR, L1 measurement during SCell activation, TCI state swiching delay, ssb-periodicityServingCell which is the periodicity of the SSB-Index configured for L1-RSRP measurement are used. On the other hand, BFD and CBD, the periodicity of SSB in the set q0 and q1 are used.

Table 1 L1 measurement requirements for SSB adaptation

| Requirements | As is | To be |
| --- | --- | --- |
| L1-RSRP, L1-SINR, L1 measurement during SCell activation, TCI state swiching delay | ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement | the periodicity of the SSB-Index indicated for SSB adaptation configured for L1-RSRP measurement |
| BFD ,CBD | the periodicity of SSB in the set q0, q1 | the periodicity of SSB indicated for SSB adaptation in the set q0,q1 |

Proposal 3: Apply indicated period of SSB adaptation for following L1 measurement requirement

- L1-RSRP

- L1-SINR

- L1 measurement during SCell activation

- TCI state switching delay

- Beam failure detection, Candidate beam detection

Conclusion

In this contribution, we provide views on RRM requireiments for SSB adaptation, and we propose

Proposal 1: Defer L3 measurement requirement pending on RAN2 progress

Proposal 2: RAN4 to preclude PCell from the scenario

Proposal 3: Apply indicated period of SSB adaptation for following L1 measurement requirement

- L1-RSRP

- L1-SINR

- L1 measurement during SCell activation

- TCI state switching delay

- Beam failure detection, Candidate beam detection

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2420242, “WF on RRM requirements for Netw_Energy_NR_enh_Part2,” Huawei