3GPP TSG-RAN WG4 Meeting #114	R4-2500783

Athens, Greece, 17-21 Feburary, 2025

Source: 	Huawei, HiSilicon

Title: 					BS requirements for ATG enhancements

Agenda Item:	7.12.2

Document for:	Discussion

Introduction

The new WID on Enhancements for Air-to-ground network for NR was approved at the RAN#103 meeting [1] and the revised WID was approved at RAN#106 [2]. The RAN4 BS RF relative objectives are copied as below:

Copied from [2]:

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify the TAE requirements for support of DL MIMO for BS. [RAN4].

- Specify signaling and UE capabilities if necessary [RAN2].

In this paper we provide our view on R19 ATG BS RF requirements.

Discussion

For ATG BS supporting CA and DL MIMO, TAE requirements should be applied.

Current TAE requirements can be reused for ATG BS.

Proposal 1: Reuse current TAE requirements of NR BS for ATG BS supporting CA and DL MIMO.

In the current TS38.104, the TAE requirements for ATG BS, both conducted and radiated, in sections 6.5.3.1 and 9.6.3 respectively, are described as following：

There is no TAE requirement for ATG BS.

Giving that ATG BS DL MIMO is included in the updated WID, and ATG BS supports intra-band contiguous CA and inter-band CA, the sentence “There is no TAE requirement for ATG BS.” should be removed.

Proposal 2: Delete the sentences “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 in TS38.104.

Conclusion

In the contribution, we provide our opinions on R19 ATG BS RF requirements and have the following proposals:

Proposal 1: Reuse current TAE requirements of NR BS for ATG BS supporting CA and DL MIMO.

Proposal 2: Delete the sentences “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 in TS 38.104.

Reference

[1] RP-240839, New WID on Enhancements for Air-to-ground network for NR, Moderator (Apple), RAN#103, Mar., 2024

[2] RP-243285, Revised WID on Enhancements for Air-to-ground network for NR, CMCC, CATT, RAN#106, Dec., 2024