3GPP TSG RAN WG4 Meeting #114	R4-2500829

Athens, Greece, 17th – 21st February, 2025

Agenda item:			5.18.2

Source:	vivo

Title:	Further discussion on UE capability and requirements for multi-carrier enhancements

Document for:	Discussion

# Introduction

In the RAN4#112bis meeting, there were discussions triggered by contribution [1] on UE capability and corresponding requirements for simultaneous DCI based BWP switch delay on multiple CCs. The conclusion on this issue was captured in the WF [2].

Topic #1: UE capability value for multi-carrier enhancements Issue 1-1: Report value for FG6-3 and FG38-9 <Way forward> - Option 1: The same value D is assumed in the requirements between RAN4 features 6-3 and 38-9. - Option 2: The different value D is assumed in the requirements between RAN4 features 6-3 and 38-9. Topic #2: Incremental BWP switch delay definition Issue 2-1: Requirements for DCI based BWP switch delay on multiple CCs <Way forward> - Option 1: Keep current requirements. - Option 2: Update is needed, e.g., different increment value for different UE capabilities is used in the requirements, and will be discussed under Rel-18 maintenance. - Option 3: Further discuss whether update on D is needed for simultaneous switching between non-dormant BWPs and switching between non-dormant and dormant BWPs by using DCI 0-1/1-1/2-6.

In the RAN4#113 meeting, we provided views on requirements for simultaneous DCI based BWP switch delay on multiple CCs for the new UE capability in [3]. There were discussions online and offline. But no agreements can be reached on whether to revise the requirements.

In this contribution, we further provide our views on the UE capability in terms of reporting value for BWP switch and corresponding requirements.

# Discussion

The overall BWP switch delay for simultaneous DCI based BWP switch delay on multiple CCs is based BWP switching delay on single CC plus incremental delay D for each additional CC. The relevant requirements are copied below for convenience.

UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay + Y, which is defined as: TMultipleBWPswitchDelay = TBWPswitchDelay + D*(N-1) Where: - TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. TBWPswitchDelay shall be based on the smallest SCS among SCS of all involved CCs before and after BWP switch. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay shall be based on the smallest SCS among all SCS values of all involved CCs. - D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs. - For UE which is capable of per-FR gap, and no BWP switch involves SCS change, N is the number of CCs in same FR; For UE which is not capable of per-FR gap, or the BWP switches on any CC involves SCS changing, N is the number of CCs undergoing simultaneous BWP switch. - Y=0, if the serving cell where UE receives DCI for BWP switch is same as the serving cell on which BWP switch occurs for each involved serving cell. Y equals to the length of one slot at smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change, - if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. If both scheduling cell and scheduled cell are in FR2-2, Y shall follow the SCS of 120 KHz. .

The related three UE capabilities for UE to report incremental delay value for BWP switch on multiple CCs are as follows.

bwp-SwitchingMultiCCs-r16 (FG 9-1): Incremental delay for BWP switch processing on additional CCs in timer/DCI based simultaneous BWP switching on multiple CCs.

bwp-SwitchingMultiDormancyCCs-r16 (FG 6-3): Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells with DCI 0-1/1-1/2-6.

bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 (FG 38-9): Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells with DCI 0-3/1-3.

There are two type of BWP switch delay, i.e., switching between non-dormant BWPs and switching between dormant and non-dormant BWP. For BWP switch from one non-dormant BWP to another non-dormant BWP, the incremental value D depends on UE capability bwp-SwitchingMultiCCs-r16 (FG 9-1), and for BWP switch from dormant BWP to non-dormant BWP, the incremental value D depends on UE capability bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 (FG 38-9) or bwp-SwitchingMultiDormancyCCs-r16 (FG 6-3).

FG 38-9 and FG 6-3 are both for dormant BWP switching. UE may report different values, but we don’t see any reason to report different values since both are switching from dormant BWP to normal BWP. The UE processing on the switching would be the same.

Proposal 1: The same value D is assumed in the requirements between RAN4 features FG 6-3 and FG 38-9

FG 38-9 and FG 9-1 are different on the other hand. FG 9-1 is for UE to report incremental delay for switching between normal BWPs. The incremental delay could highly be different than switching from dormant BWP to normal BWP. Smaller value is expected for FG 9-1.

If there are multiple types of BWP (non-dormant BWP and dormant BWP) to switch, which is allowed by using DCI 0-3/1-3, it is ambiguous which value shall be applied to calculate BWP switching delay, if different values are reported by the UE, based on existing requirements. Therefore, it is better to use different incremental value D for the different feature groups in the requirements.

One of the reasons that the value D should not be changed would be it was agreed to reuse incremental value D for BWP switching on multiple CCs as captured in the WF [4].

Agreement: - Delay requirements for switching of multiple SCells between dormancy and non-dormancy shall be based on corresponding delay requirements for switching of multiple SCells between non-dormant BWPs, i.e., TMultipleBWPswitchDelay or TMultipleBWPswitchDelay+Z for simultaneous BWP switching case. - Incremental value D is reused from BWP switching on multiple CCs - Companies can as part of maintenance further look in to whether to introduce a separate capability with respect to incremental value D for SCell dormancy switching.

However, there could be different interpretations of the agreements.

Option 1: UE can report different values, but from requirement perspective the same value is used because simultaneous BWP switching of the two types are not considered.

Option 2: UE can report different values, but requirements only apply if same value is reported.

Option 3: UE reports one value, just based on the larger one so that UE can pass the test.

Option 4: The set of value D is reused, but UE can report different values.

In the contribution [5] from Qualcomm, following proposal was made.

- BWP switch delay: - TdormantBWPSwitchDelay + Ddormancy * max(G, Gmin), where - T_dormantBWPSwitchDelay is the longest dormant and/or active BWP switching delay that would have taken if each BWP switching had been triggered by spCell DCI individually and non-simultaneously - A set of candidate values for Ddormancy (incremental delay factor) is the same as those for DCI/Timer based simultaneous (non-dormant) BWP switching requirement, and UE can support different Ddormancy from that for DCI/Timer based BWP switch - G is #SCell groups that are configured with dormant BWP by higher-layer signaling, which can be up to 5 if configured - Gmin is determined by UE capability, e.g. Gmin={0,1,…,5}

The proposal is aligned with interpretation option 4. In our understanding, reporting different value D can be beneficial to both NW and UE sides. Shorter BWP switch delay can be achieved. The BWP switch includes DCI decoding and processing time and RF switch time. The RF switching time can largely be different for the two types of BWP switching.

For multi-carrier DCI, simultaneous BWP switching of the two types should be supported. At least, we need to define requirements to reflect the case that UE reports different values.

Proposal 2: Different incremental values are assumed in the requirements between RAN4 features FG 9-1 and FG 38-9.

Regarding FG 6-3, it is also FFS whether different values should be used if there are simultaneous multiple types of BWP switch. In general, same principle applies, i.e., RAN4 features FG 9-1 and FG 6-3 could report different values and requirements should be updated accordingly. The problem is that FG 6-3 was introduced in Rel-16. If update is made, it needs to be made from Rel-16.

Proposal 3: Different incremental values are assumed in the requirements between RAN4 features FG 9-1 and FG 6-3. The requirements update is made from Rel-16.

# Summary

In this contribution, we further provided views on the UE capability in terms of reporting value for BWP switch and corresponding requirements. Following proposals are present.

Proposal 1: The same value D is assumed in the requirements between RAN4 features FG 6-3 and FG 38-9

Proposal 2: Different incremental values are assumed in the requirements between RAN4 features FG 9-1 and FG 38-9.

Proposal 3: Different incremental values are assumed in the requirements between RAN4 features FG 9-1 and FG 6-3. The requirements update is made from Rel-16.

- References

- R4-2415200	Discussion on UE capability for multi-carrier enhancements, vivo

- R4-2416878	WF on UE report of incremental BWP switch delay for multi-carrier enhancements, NTT DOCOMO

- R4-2418761	Further discussion on UE capability and requirements for multi-carrier enhancements, vivo

- R4-2012278	Way Forward on SCell Dormancy, Ericsson

- R4-2010118	Scell BWP dormancy, Qualcomm Incorporated