3GPP TSG-RAN WG4 Meeting # 114		R4-2500842

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	5.21.3

Source: 	CMCC

Title: 			 Discussion on core maintenance and LS from RAN2 for NTN enhancement

Document for:	Discussion

- Introduction

In past several meetings, RAN4 focused on the Rel-18 NTN maintenance issues, the latest progress has been agreed in the WF [1]. In this contribution, we further discuss on the leftovers.

Besides, in RAN2#128 meeting, a LS was sent to RAN4 and RAN1 about soft satellite switch [2], we give our analysis on this LS.

- Discussion

Issue 5-2-S: Soft Satellite switch

FFS:

- A UE that supports soft satellite switching shall mandatorily support parallelMeasurementWithoutRestriction-r17.

We don’t support the introduction of UE capability bundling with following reasons:

- The network is still workable without UE capability bundling. The duration of the scheduling restriction from network side depends on the maximum propagation delay difference between source cell and target cell, which further depends on the practical network scenario. The worst case is that the source cell could not schedule any data transmission during the whole period between t-SeviceStart and t-Service. However, the time duration between t-ServiceStart and t-Service is configured by network, and typically, this time duration is short. The throughput loss is tolerable.

- There is no relation between soft satellite switching capability and parallelMeasurementWithoutRestriction capability. Therefore, it is feasible that UE supports soft satellite switching capability but not support parallelMeasurementWithoutRestriction-r17. For this kind of UE, if network fail to use soft satellite switching due to no idea of scheduling, then network can configure hard satellite switching with resynchronization, same effect as limiting UE support for soft satellite switching capability by the mandatory condition of supporting parallelMeasurementWithoutRestriction-r17, but depends on network decision. From UE side, the feature will be less attractive if UE capability bundling is considered.

Proposal 1: Don’t consider UE capability bundling for satellite switching with re-synchronization.

LS from RAN2

For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. # 2. Actions: To RAN4, RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

In order to guarantee “non-colliding SSB with same PCI at UE side” during t-ServiceStart and t-Service, and no SMTC reconfiguration after t-Service, RAN2 agreed one possible solution which introducing a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. RAN4 should further investigated whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

Following the methodology designed by RAN2, an example of the transmitted SSBs at source cell and target cell is illustrated in the figure below:

In our understanding, as long as UE is knowledge of this ‘shifted SSB pattern’ mechanism, during [t-ServiceStart, t-Service], UE could finish the target cell synchronization by using:

- the periodicity and location of SSB of the source satellite;

- the ssb-TimeOffset;

- the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE;

After t-Service, since the system timing of target cell is unchanged, UE could maintain the synchronization, and found SSB/SMTC by using the RRC configuration of SSB and SMTC.

Proposal 2: The pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Don’t consider UE capability bundling for satellite switching with re-synchronization.

Proposal 2: The pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

- Reference

- R4-2420235, WF on RRM requirements for NR_NTN_enh, Moderator (Qualcomm Incorporated).

- R2-2410978, LS on soft satellite switch with re-sync, RAN2