3GPP TSG-RAN WG4 Meeting #114     	R4-2500845

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.18.2

Source: 	CMCC

Title: 			 Simulation results for 8Rx UE interference suppressing

Document for:	Discussion

- Introduction

Based on the simulation assumption approved in last meeting[1], we give our simulation results and observations.

- Simulation results

PDSCH requirements with inter cell interference (baseline receiver)

|  |  |  | SNR for HomoNet(dB) | SNR for HetNet(dB) |
| --- | --- | --- | --- | --- |
| FDD | 2T8R Rank 1 | MMSE-IRC | 5.56 | 3.60 |
|  |  | MMSE-MRC | 10.06 | 8.09 |
|  |  | Gain | 4.50 | 4.49 |
|  | 2T8R Rank 2 | MMSE-IRC | 8.41 | 7.47 |
|  |  | MMSE-MRC | 13.68 | 12.31 |
|  |  | Gain | 5.27 | 4.84 |
|  | 4T8R Rank 4 | MMSE-IRC | 10.32 | 9.14 |
|  |  | MMSE-MRC | 15.85 | 14.49 |
|  |  | Gain | 5.53 | 5.35 |
| TDD | 2T8R Rank 1 | MMSE-IRC | 6.93 | 4.02 |
|  |  | MMSE-MRC | 11.02 | 8.14 |
|  |  | Gain | 4.09 | 4.12 |
|  | 2T8R Rank 2 | MMSE-IRC | 8.99 | 7.75 |
|  |  | MMSE-MRC | 13.74 | 12.29 |
|  |  | Gain | 4.75 | 4.54 |
|  | 4T8R Rank 4 | MMSE-IRC | 10.44 | 9.16 |
|  |  | MMSE-MRC | 15.81 | 14.45 |
|  |  | Gain | 5.37 | 5.29 |

- HomoNet：

- HetNet：

PDSCH requirements with inter cell interference (simplified receiver)

|  |  |  | SNR for HomoNet(dB) | SNR for HetNet(dB) |
| --- | --- | --- | --- | --- |
| FDD | 2T8R Rank 1 | MMSE-IRC | 6.37 | 3.87 |
|  |  | MMSE-MRC | 10.10 | 8.13 |
|  |  | Gain | 3.73 | 4.26 |
|  | 2T8R Rank 2 | MMSE-IRC | 10.82 | 8.66 |
|  |  | MMSE-MRC | 14.54 | 13.06 |
|  |  | Gain | 3.72 | 4.40 |
|  | 4T8R Rank 4 | MMSE-IRC | 15.79 | 13.42 |
|  |  | MMSE-MRC | 18.28 | 16.87 |
|  |  | Gain | 2.49 | 3.45 |
| TDD | 2T8R Rank 1 | MMSE-IRC | 7.72 | 4.11 |
|  |  | MMSE-MRC | 11.06 | 8.38 |
|  |  | Gain | 3.34 | 4.27 |
|  | 2T8R Rank 2 | MMSE-IRC | 11.07 | 8.99 |
|  |  | MMSE-MRC | 14.62 | 13.09 |
|  |  | Gain | 3.55 | 4.10 |
|  | 4T8R Rank 4 | MMSE-IRC | 15.96 | 13.48 |
|  |  | MMSE-MRC | 18.28 | 16.89 |
|  |  | Gain | 2.32 | 3.41 |

- HomoNet：

- HetNet：

PDSCH requirements with intra cell inter user interference (baseline receiver)

|  |  |  | SNR(dB) |
| --- | --- | --- | --- |
| FDD | 4T8R Rank 2+2（MCS17） | MMSE-IRC | 10.57 |
|  |  | MMSE-MRC | N/A |
|  |  | Gain | Inf |
|  | 4T8R Rank 2+2（MCS19） | MMSE-IRC | 12.02 |
|  |  | MMSE-MRC | N/A |
|  |  | Gain | Inf |
|  | 8T8R Rank 4+4 | MMSE-IRC | 17.71 |
|  |  | MMSE-MRC | N/A |
|  |  | Gain | Inf |

PDSCH requirements with intra cell inter user interference (simplified receiver)

|  |  |  | SNR(dB) |
| --- | --- | --- | --- |
| FDD | 4T8R Rank 2+2 (MCS17) | MMSE-IRC | 13.75 |
|  |  | MMSE-MRC | N/A |
|  |  | Gain | Inf |
|  | 4T8R Rank 2+2（MCS19） | MMSE-IRC | 15.43 |
|  |  | MMSE-MRC | N/A |
|  |  | Gain | Inf |
|  | 8T8R Rank 4+4 | MMSE-IRC | N/A |
|  |  | MMSE-MRC | N/A |
|  |  | Gain | N/A |

- Simulation assumption

PDSCH requirements with inter cell interference

Simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 1: 2x8 ULA Low Rank 2: 2x8 ULA Low Rank 4: 4x8 ULA Low |
| Propagation condition and antenna correlation |  |  | TDLC300-100 for 2 interfering cells with rank 1 (HomNet) TDLA30-10 for all other configurations. |
| MCS |  |  | Rank 1: MCS 17 (Table 1) Rank 2: MCS 17 (Table 1) Rank 4: MCS 13 (Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Single-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Test metric |  |  | SNR@70% max TP |
| Receiver |  |  | Baseline SU-MIMO 8Rx receiver Simplified SU-MIMO 8Rx receiver |

| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| --- | --- |
| PDSCH allocation in frequency domain | In frequency domain: Full PRB In time domain: Full PDSCH allocation on all slots where serving cell has PDSCH grant |
| Precoding of interfering PDSCH | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |
| SSB configuration | All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources |
| INR values for Homogeneous deployment | INRs 7.77 and 2.29 dB in case of 2 interference cells |
| INR values for Heterogeneous deployment | INRs 7.58 dB in case of 1 interference cells |
| Transmission rank of interfering PDSCH | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| Modulation order of interfering PDSCH | 16QAM randomly modulated symbols |
| Timing and frequency offset | The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 |
| Interference cell modeling | Interfering cell has 2Tx and random precoding with PRB granularity of 2. Interfering cell has same channel model as for serving cell and it is independently generated for each cell. |

PDSCH requirements with intra cell inter user interference

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | 10MHz/15kHz for FDD and 40MHz/30kHz for TDD |  |
| Duplex mode |  | FDD and TDD |  |
| TDD pattern |  | 7D1S2U S=6D+4G+4U |  |
| MCS |  | Rank 2+2: MCS17/19 Rank 4+4: MCS13 | 16 QAM random symbols |
| Allocation for interference UE and target UE | Rank allocation | 2 | 2 |
|  |  | 4 | 4 |
|  | DMRS port allocation | For rank 2+2: Port 1000 and 1001 For rank 4+4: use different CDM group Port 1000, 1001,1004, 1005 | For rank 2+2: Port 1002 and 1003 For rank 4+4:use different CDM group Port 1002, 1003, 1006, 1007 |
| MIMO configuration |  | -Rank 2+2 for 4T8R ULA Low -Rank 4+4 for 8T8R ULA Low Note: Will require DMRS len=2 |  |
| Number of CDM groups without data |  | 2 for case with rank 2+2 and rank 4+4 |  |
| HARQ process number |  | 4 for FDD and 8 for TDD |  |
| Precoding model | Target UE | Random precoding with Single panel Type 1 4Tx: (N1, N2) = (2,1) 8Tx: (N1, N2) = (4,1) | For case with rank 2+2 and 4+4: Select the precoding matrix to ensure orthogonality with Target UE |
| PDSCH configuration | Mapping type | Type A |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PRB bundling size | 2 |  |
|  | PRB bundling type | Static |  |
| PDSCH DMRS configuration | DMRS Type | DMRS Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |  |
| Propagation conditions |  | TDLA30-10 |  |
| Receiver type |  | Baseline SU-MIMO 8Rx receiver Simplified SU-MIMO 8Rx receiver | N/A |
| Test metric |  | SNR @ %70 of maximum Throughput SNR = (STargetUE+ SInterfUE)/N | N/A |

- Reference

[1] R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, Moderator (Nokia)