3GPP TSG-RAN WG4 Meeting # 114		R4-2500852

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.29.4.1

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed (e)RedCap UEs over NR FR1-NTN. The WF has been approved in [1].

In this contribution, we further discuss the RRM impact and specific requirements.

- General consideration on RRM requirements

Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN?

<Way Forward>:

- Proposal 1 (CATT, Xiaomi, LG, Samsung CMCC, Apple, Ericsson, HW, ZTE):

  - RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

- Proposal 2 (vivo):

  - RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Considering some areas may not been covered by 5G network yet, e.g. desert, coastal, and forest area edge, which are typical NTN service cases, we support to also define the cell re-selection requirement for inter-RAT (NTN to E-UTRA TN) measurement, to guarantee the service consistency between NTN and TN network.

Proposal 1: Define the cell re-selection requirement for inter-RAT (NTN to E-UTRA TN) measurement.

Issue 1-1-2: The Requirements to be further determined

<Way Forward>:

- For SDT requirements in RRC_INACTIVE state,

  - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements.

    - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN.

    - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission.

    - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN.

    - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable)

  - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

- For MDT requirements in RRC_IDLE and RRC_INACTIVE state

  - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK):

    - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system.

    - Proposal 1a (CATT): Deprioritize to define.

  - Proposal 2 (CMCC):

    - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec.

  - Proposal 3 (ZTE):

    - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

Regarding CG-SDT, in TN network, the procedure may rely on TA validation based on the RSRP change criterion. However, in NTN network, due to in-apparent near-far-effect, the cg-SDT-RSRP-ChangeThreshold for TA validation methodology may not valid anymore.

Similar cases happens in IOT-NTN, at that time,  RAN4 agreed to use pur-TimeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE to determine the TA validation, the specification is as follows:

The UE is allowed to transmit using PUR provided that TA is valid provided that pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as defined in [2] The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

We think similar approach can be referred for CG-SDT, that using cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE to determine the TA validation.

Proposal 2: For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN.

Regarding MDT, we think it is beneficial to support MDT capability for Redcap UE over NTN.

Firstly, For NTN network with large/moving cell coverage, it is barely impossible to use the traditional drive test, which requires support from both operation and testing equipment, the difficulty and cost of network optimization will increase a lot. While MDT could help network to optimize the scheduling with low cost.

Secondly, the link performance seen between handheld UE and Redcap UE may be different, especially for 1Rx Redcap and HD-FDD Redcap. Therefore, although NTN network has already have MDT report from handheld UE, the MDT report from Redcap UE can still provide more information to network.

Lastly, as mentioned in the summary of last meeting, TS 38.306 states that all other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise. We think the MDT can be supported for RedCap UE, but if companies have different view on this, we are open to sent LS to RAN2 to check the capability.

- Proposal 3: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

- Proposal 4: If companies have different views on whether MDT can be supported for RedCap UE, LS can be sent to RAN2 to check the capability.

Issue 1-2-1: Relaxed measurement

< Agreement >:

- For supported types of relaxations for (e)RedCap UE with FR1-NTN,

  - Only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection.

- RAN4 to further discuss the thresholds and specific relaxation factor.

In last meeting, RAN4 agreed that only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection, which means only requirements of UE fulfilling low mobility criterion and/or not-at-cell edge criterion will be defined.

For the thresholds of relaxation criterion, it is up to network configuration, legacy signaling can be reused, no need to have the discussion.

For the relaxation factor, due to it is hard for network to distinguish the UE mobility state by RSRP variation, to guarantee the mobility performance, we prefer to reuse the legacy relaxation factor of TN R16 relaxation, which is 3 for UE only fulfill one of the relaxation criterion, and no requirement when both relaxation criterion are fulfilled.

- Proposal 5: For the thresholds of relaxation criterion, legacy signaling can be reused, threshold value is up to network configuration.

- Proposal 6: Reuse the legacy relaxation factor of TN R16 relaxation, which is 3 for UE only fulfill one of the relaxation criterion, and no requirement when both relaxation criterion are fulfilled.

Issue 1-2-2: eDRX configuration

< Agreement >:

- For earth-moving LEO scenario:

  - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario.

- For quasi-earth fixed LEO scenario:

  - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN.

    - Requirements are applicable for 2.56s and 5.12s eDRX cycle

    - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

In earth-fixed LEO deployment, considering about 300s service time per satellite, requirement are applicable for up to 5.12s eDRX cycle, Tdetect is 117.76s per carrier frequency, roughly, UE is able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria for intra-frequency or one inter-frequency carrier frequency.

- Proposal 7: For earth-fixed LEO deployment, requirements are applicable for up to 5.12s eDRX cycle.

Issue 1-2-4: Multiple SMTCs

<Agreements>:

- RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs.

- For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs?

  - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support

  - Option 2 (HW, Nokia): FFS

- Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs.

- Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period.

- Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

In R17 NTN, according to the UE capability parallelSMTC-r17, UE is capable of performing RRM measurements in parallel on at least 2 SMTCs with different offsets and same periodicity, optionally on 4 SMTCs, which configured by SSB-MTC4List-r17, which means UE is able to sample in ‘2 or optional 4 SMTCs’ in parallel during one SMTC periodicity.

For Redcap over NTN, since the network scenario is unchanged, RAN4 still need to define RRM requirements for multiple SMTC-s for a single frequency carrier to guarantee at least 2 target satellites including serving LEO satellite will be monitored. However, when define the requirement, we think there are two ways corresponding to different UE capability:

- Alt1: Same as R17 NR-NTN, UE is capable of performing RRM measurements in parallel on at least 2 SMTCs with different offsets and same periodicity, optionally on 4 SMTCs. The requirements in 9.2C apply provided that the total number of measurement window offsets does not exceed the UE capability parallelSMTC-r17.

- Alt2: UE is not capable of performing RRM measurements in parallel on multiple SMTCs. The requirements in 9.2C should be scaled by the number of SMTCs configured by the network.

- Currently, we think Alt1 can be used for 2Rx Redcap UE, and Alt 2 can be used for 1Rx Redcap UE which capability is lower.

Proposal 8: Define the measurement requirement by using Alt1 for 2Rx Redcap UE, Alt2 for 1Rx Redcap UE.

- Alt1: Same as R17 NR-NTN, UE is capable of performing RRM measurements in parallel on at least 2 SMTCs with different offsets and same periodicity, optionally on 4 SMTCs. The requirements in 9.2C apply provided that the total number of measurement window offsets does not exceed the UE capability parallelSMTC-r17.

- Alt2: UE is not capable of performing RRM measurements in parallel on multiple SMTCs. The requirements in 9.2C should be scaled by the number of X, X= min(UE supported number of SMTCs, number of SMTCs configured by the network).

- Regarding Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell (Feature 25-2), Parallel measurements with multiple measurement gaps (Feature 25-3), and Parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5), we think these capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

- Proposal 9: For feature 25-2, 25-3 and 25-5, these capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

- As for the number of target LEO satellites the UE can monitor per carrier including serving satellite, currently, for legacy UE, in NGSO deployments, cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-LEO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-LEO-SatellitesPerCarrier-r17. We think same UE capability can be reused for (e)RedCap UE.

- Proposal 10: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7) can be reused.

Issue 1-2-5: The number of searchers and CSSF

<Way Forward>:

- Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

The CSSF is a fundamental parameter for further defining the measurement requirements. For Redcap UE, it has been defined in Clause 9.1A.5 TS 38.133, which only considering NR SA single carrier mode. We think it can be the starting point for CSSF of Redcap over NTN.

Proposal 11: Define the CSSF for Redcap over NTN, the legacy CSSF for Redcap can be the baseline.

Issue 1-2-7: NCD-SSB

<Way Forward>:

- For the terminology of SSB used in the spec,

  - Proposal 1 (CATT, Ericsson, HW, CMCC): Use the terminology of SSB without differentiating CD-SSB/NCD-SSB.

- For whether to add additional requirement applicability about CD-SSB/NCD-SSB,

  - Option 1 (CATT, Ericsson, HW): Add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

    - The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

  - Option 2 (CMCC): No need to add additional requirement applicability about CD-SSB/NCD-SSB.

we propose to use the terminology of SSB without differentiating CD-SSB/NCD-SSB as R17 NTN, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

Proposal 12: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

Issue 1-2-8: Other features

<Way Forward>:

- Proposal 1 (CMCC): For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed.

  - Set 1: propose to be optionally supported by Redcap over NTN

    - SMTC adjustment in idle/inactive

    - Location-based measurement report trigger

    - Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

    - The number of target LEO satellites the UE can monitor per carrier including serving satellite

    - Further discuss the max number

  - Set 2:

    - Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell

    - Parallel measurements on multiple NGSO satellites within a SMTC

For R17 and R18 NTN, RAN1 to RAN4 introduced a lot of new UE features and related requirements for network access, measurement, mobility, and so on. For the optional NTN features below which have impact on RAN4 requirements, we think they can be optionally supported by Redcap UE.

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

Proposal 13: For the optional NTN features which have impact on RAN4 requirements, they can also be optionally supported by Redcap UE..

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

- Reduction in the number of UE Rx branches

Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

< Agreement >:

- For 2Rx (e)RedCap UEs with FR1-NTN bands,

  - RAN4 to reuse the legacy mobility requirements in RRC_IDLE / RRC_INACTIVE state defined for normal UEs with FR1-NTN in high level.

    - Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised.

- For 1Rx (e)RedCap UEs with FR1-NTN bands,

  - RAN4 to reuse the same offset of 1Rx RedCap UEs in TN compared to 2Rx (e)RedCap UEs.

    - For Qrxlevmin and Qqualmin: -1 dB offset can be reused.

    - For the measurements of intra- and inter-frequency NR cells,

      - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline.

      - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked.

    - In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria.

      - For 1 Rx RedCap by a margin of at least:

        - 6 dB in FR1 for reselections based on ranking or

        - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

        - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

  - RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring.

    - 6 NR inter-frequency carriers can be reused.

  - The parameters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)RedCap UE, and the requirements for both will be the same.

  - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

- Define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands.

  - FFS: How to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands.

- FFS: RAN4 to further discuss how to define the cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx and 2 Rx (e)RedCap UE after issue1-1-1.

Firstly, we think the enhanced cell reselection measurement will be defined for RedCap UE not configured with eDRX_IDLE cycle. Then, for the delay requirements, we think the enhanced cell reselection measurement requirement for normal UEs in NR NTN can be reused.

For inter-RAT E-UTRAN cells with TN carrier cell re-selection measurement requirements, the legacy requirements defined for normal UEs can be reused for (e)Redcap UE not configured with eDRX_IDLE cycle. For (e)Redcap UE configured with eDRX_IDLE cycle, the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 can be reused.

Proposal 14: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused.

Proposal 15: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements,  reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle.

Issue 2-1-5: Satellite switching with re-sync

< Agreement >:

- For Satellite switching with re-sync requirements,

  - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy Satellite switching with re-sync requirements for normal UEs (including both hard satellite switch with re-sync and soft satellite switch with re-sync).

  - For hard Satellite switching with re-sync requirements, for 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

    - Define the relaxation on Tsearch by extending one more [SSB or SMTC] periodicity on the top of Tfirst_SSB:

      - Tsearch = Tfirst_SSB + [TSSB or TRS] ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

  - FFS: whether to define soft Satellite switching with re-sync requirements for 1Rx (e)RedCap UEs with FR1-NTN bands.

For hard satellite switching with re-sync, 1Rx (e)RedCap UEs should have one more sample to search the target cell. We prefer to extend the Tsearch by one more SSB periodicity to minimize the satellite switching delay.

For soft satellite switching with re-sync, from UE perspective, whether UE support soft satellite switching depends on its capability, same with normal UE and 2Rx Redcap UE. From network perspective, although the Tsearch will be extended comparing with 2Rx Redcap UE, the scheduling strategy keeps same with other cases during t-serviceStart to t-sevice. Therefore, the 1Rx case is same with legacy case from both UE and network perspective, the requirement should also be defined. Same analysis as hard satellite switching, prefer to extend the Tsearch by one more SSB periodicity to minimize the satellite switching delay.

Proposal 16: For hard satellite switching with re-sync requirement of 1Rx (e)RedCap UEs, define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB

Proposal 17: Define soft satellite switching with re-sync requirement of 1Rx (e)RedCap UEs, define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB

Issue 2-4-1: L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands,

  - Proposal 1 (CATT, Samsung, CMCC, HW): RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level.

    - Depend on the supported UE capabilities (e.g., multiple SMTC),, the existing NR NTN requirements may need be revised.

- For 1Rx (e)RedCap UEs with FR1-NTN bands,

  - Proposal 1 (CATT, Samsung, CMCC, ZTE, HW): The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

        - Not to extend the lower boundary.

    - Proposal 1a (QC): Extended delays for PSS/SSS detection.

    - Proposal 1b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples.

  - Proposal 2 (CATT, [Samsung?], CMCC, HW): The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

        - Extend the lower boundary from ‘120 ms’ to ‘160ms’.

    - Proposal 2a (QC): Extended delays for SSB index identification (PBCH decoding).

    - Proposal 2b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples.

  - Proposal 3 (CATT, CMCC, HW): The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Extend the lower boundary from ‘200 ms’ to ‘400ms’

      - Keep the same number of samples for T SSB_measurement_period_inter.

  - Proposal 4 (CATT, CMCC, ZTE, HW): The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

  - Proposal 5 (CATT, CMCC, HW): The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

For measurement requirements RRC_CONNECTED mode, except the impact from some optional features which haven’t discussed or agreed so far, for instance, parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell, and Parallel measurements on multiple NGSO satellites within a SMTC, the way forward from RAN4#112bis meeting exactly follow the same principle for (e)RedCap UE in TN network, we support all of them.

Proposal 18: For measurement requirements RRC_CONNECTED mode, except the impact from some optional features which haven’t discussed or agreed so far, for instance, parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell, and Parallel measurements on multiple NGSO satellites within a SMTC, support the way forward from RAN4#112bis meeting.

Issue 2-5-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands,

  - Proposal 1 (CATT, Samsung, CMCC, ZTE): RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands,

  - Proposal 1 (CATT, Xiaomi, Samsung, [QC?], CMCC, ZTE, [OPPO?], HW): RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

    - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

      - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

      - L1-RSRP: relax by 3dB.

For measurement accuracy requirement, the way forward from RAN4#112bis meeting exactly follow the same principle for (e)RedCap UE in TN network, we support all of them.

Proposal 19: For measurement accuracy requirement, support the way forward from RAN4#112bis meeting.

- The impact of HD-FDD

Issue 3-1-1: General HD-FDD related impact

<Way Forward>:

- Proposal 1 (CATT, Xiaomi, Samsung, QC, CMCC, ZTE, Ericsson, HW):

  - The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

  - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions.

  - Proposal 1a (CATT, HW, MTK):

    - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

      - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

      - Handover interruption time

      - Random access

      - SA: RRC Connection Release with Redirection

      - Minimum requirement for L1 indication for RLM and LR

      - MAC-CE/DCI/RRC based uplink spatial relation switch delay

      - Scheduling availability of UE performing intra/inter measurements

      - Scheduling availability of UE during L1-RSRP measurement

  - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects:

    - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions.

  - Proposal 1c (CMCC):

    - Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

    - Wait for further RAN1 progress which may impact Scheduling availability requirement.

Firstly, we think the same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

Proposal 20: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

For NTN specific aspects, the DL and UL collision error due to mismatch between actual TA used by UE and assumed TA at the gNB may need to further considered. RAN1 has achieved several solutions listed as below:

Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL

After checking on the current scheduling availability, although some priority rules are defined in RAN1, the existing scheduling availability requirement can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured.

Issue 3-2-1: The impact on NR Handover due to HD-FDD

<Way Forward>:

- Proposal 1 (CMCC):

  - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

    - For RedCap NTN UE with HD-FDD, the requirements are met provided that

      - SSB is available at the UE once every SMTC period during Tsearch

      - One SSB is available during T∆

      - One SSB is available during TIU.

  - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

    - For RedCap NTN UE with HD-FDD, the requirements are met provided that

      - One SSB is available during T∆

      - One SSB is available during TIU

- Proposal 2 (Ericsson):

  - RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover and NR Satellite switching with re-synchronization requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case,

    - For RedCap NTN UE with HD-FDD, the handover requirements and Satellite switching with re-synchronization requirements are met provided that

      - SSB is available at the UE once every SMTC period during Tsearch

      - One SSB is available during T∆

      - One SSB is available during TIU.

- Proposal 3 (HW):

  - For HD-FDD (e)RedCap UE with FR1-NTN bands, the CHO and satellite switching with re-sync requirements are met provided that

    - SSB is available at the UE once every SMTC period during Tsearch

    - One SSB is available during T∆

    - One SSB is available during TIU.

- Proposal 4 (Nokia):

  - Only support hard satellite switching for RedCap UEs

- For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

- For Time/location-based CHO with L3 measurement, due to the target cell must be the known cell, no Tserach needed in the interruption, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 21: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 22: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU

- Others

Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1 (Nokia): RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

  - Proposal 1a (CMCC): Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

  - Proposal 1b (Ericsson): RAN4 to define a new capability declaring (e) RedCap for NTN.

For (e)Redcap UE capability declaring, both way as stated in proposal 1 is workable. Considering the principle of no need to introduce new capability if existing capability can be used, we slightly prefer the implicit way to save the workload of RAN2.

Proposal 23: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: Define the cell re-selection requirement for inter-RAT (NTN to E-UTRA TN) measurement.

Proposal 2: For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN.

- Proposal 3: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

- Proposal 4: If companies have different views on whether MDT can be supported for RedCap UE, LS can be sent to RAN2 to check the capability.

- Proposal 5: For the thresholds of relaxation criterion, legacy signaling can be reused, threshold value is up to network configuration.

- Proposal 6: Reuse the legacy relaxation factor of TN R16 relaxation, which is 3 for UE only fulfill one of the relaxation criterion, and no requirement when both relaxation criterion are fulfilled.

- Proposal 7: For earth-fixed LEO deployment, requirements are applicable for up to 5.12s eDRX cycle.

Proposal 8: Define the measurement requirement by using Alt1 for 2Rx Redcap UE, Alt2 for 1Rx Redcap UE.

- Alt1: Same as R17 NR-NTN, UE is capable of performing RRM measurements in parallel on at least 2 SMTCs with different offsets and same periodicity, optionally on 4 SMTCs. The requirements in 9.2C apply provided that the total number of measurement window offsets does not exceed the UE capability parallelSMTC-r17.

- Alt2: UE is not capable of performing RRM measurements in parallel on multiple SMTCs. The requirements in 9.2C should be scaled by the number of X, X= min(UE supported number of SMTCs, number of SMTCs configured by the network).

- Proposal 9: For feature 25-2, 25-3 and 25-5, these capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

- Proposal 10: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7) can be reused.

Proposal 11: Define the CSSF for Redcap over NTN, the legacy CSSF for Redcap can be the baseline.

Proposal 12: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

Proposal 13: For the optional NTN features which have impact on RAN4 requirements, they can also be optionally supported by Redcap UE..

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

Proposal 14: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused.

Proposal 15: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements,  reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle.

Proposal 16: For hard satellite switching with re-sync requirement of 1Rx (e)RedCap UEs, define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB

Proposal 17: Define soft satellite switching with re-sync requirement of 1Rx (e)RedCap UEs, define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB

Proposal 18: For measurement requirements RRC_CONNECTED mode, except the impact from some optional features which haven’t discussed or agreed so far, for instance, parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell, and Parallel measurements on multiple NGSO satellites within a SMTC, support the way forward from RAN4#112bis meeting.

Proposal 19: For measurement accuracy requirement, support the way forward from RAN4#112bis meeting.

Proposal 20: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

Proposal 21: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 22: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU

Proposal 23: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Reference

- R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)