3GPP TSG-RAN WG4 Meeting #114			R4-2500854

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.30.3

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for IOT NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 clarified the definition of NTA for IOT NTN phase 3, the WF has been approved in [1]. In this contribution, we further discuss the RRM impact and requirements based on the progress of RAN1/2/3.

- EDT enhancement

Timing requirement for EDT enhancement

RAN2 agreed to consider CB-msg3 EDT transmission for the cases confirmed by RAN1/RAN4. The related agreements are as follows:

- Agreements before RAN2#128 - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN. - Agreements from RAN2#128 - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

For the timing related requirements, due to no further available relevant agreement from RAN2, the discussion of this part can be hold on.

Proposal 1: Suspend the discussion of UL timing requirement until further relevant agreement from RAN2 is available.

RAN2 introduced a RSRP threshold that determines whether CB-Msg3 can be used. Then how UE derive the RSRP may need further discussion, for example, UE uses Srxlev to compare with RSRP threshold, or UE use the RSRP value derived by new delay requirement to compare with RSRP threshold. We can wait further clarification from RAN2 to see whether RAN4 need to define some new delay requirements.

- Conclusion

Based on the discussion above, the following proposal is concluded.

Proposal 1: Suspend the discussion of UL timing requirement until further relevant agreement from RAN2 is available.

- Reference

- R4-2420106, WF on RRM requirements for IoT_NTN_Ph3, MediaTek