3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2500862

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.26.2.2

Source: 	CMCC

Title: 	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In this contribution, we focus on discuss of UE LP-WUS RF requirements including REFSENSE, ACS and ASCS.

- Discussion

2.1 REFSENSE requirements

During the last meeting, RAN4 reached the following agreement below.

Issue 2-2-1: General views on REFSENS for LP-WUS Agreements: - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values. Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR - Proposals - Proposal 1: check the following input from companies and converge the range for OOK-based LR. (Moderator) FR1 OOK-based receiver; Huawei; Sony; Spread trum; Apple; vivo; QC; Samsung; LGE; CMCC; ZTE; Nokia; E//. IM(dB); 2.5; 2.5; 2.5; >=2.5; 2.5; 2.5; 3; 2.5; 2.5; 2.5; 1; 2.5. NF(dB) (assume MR=9dB); 17; 10; 17; 16; 15; 11.5; 15; 16; 12; 14; 12; 12. SNR(dB); -2; -7~-2; -3; -2; -1 for sequence length of 18; -2; -2.5; -2; -1.5~ -5. Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF. Agreements: - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported. - The total value of NF+IM can be considered. - On how to handle the NF values: - Option 1: For NF whether average value can be adopted. - Option 2: two sets NFs for different LP-WUR types with different power consumption target - Option 3: also consider coverage - Capture the table in WF as background information Issue 2-2-4: companies input on SNR, NF and IM value for FR1 OFDM-based LR - Proposals - Proposal 1: check the following input from companies and converge the range for OFDM-based LR. (Moderator) FR1 OFDM-based receiver; Huawei; Sony; Spread trum; Apple; vivo; QC; Samsung; LGE; CMCC; ZTE; Nokia; E//. IM(dB); 2.5; 2.5; 2.5; >=2.5; 2.5; 2.5; 2.5; 2.5; 1; 2.5. NF(dB)(assume MR=9dB); 11; 10; 11; 16; 11; 12; 9; 11; 12; 12~13. SNR(dB); -2; -7~-2; -3; -2 or -5; -4.7; -5; -3.2. Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF. Agreements: - Further discuss next meeting

Considering that SNR is relevant to RF and demodulation, and that RAN1 has two options of OFDM signal and OOK signal, the required SNR for demodulation of OOK and OFDM signals could be different, so two sets of SNR values could be specified. Based on the simulation results, the OFDM receiver performs better than the OOK receiver. Since the case OOK-4 (M=4) has the worst performance, the target SNR should be chosen based on this case.

Table 1 SNR@1%BLER with 4bit ADC under AWGN

| Modulation | Channel structure | Receiver type | SNR@1% BLER |
| --- | --- | --- | --- |
| OOK-1 | 8bit | Envelope | -6.5dB |
|  |  | OFDM | -10.6dB |
|  | 16bit | Envelope | -6.1dB |
|  |  | OFDM | -10.3dB |
| OOK-4 M=2 | 8bit | Envelope | -4.7dB |
|  |  | OFDM | -7.6dB |
|  | 16bit | Envelope | -4.2dB |
|  |  | OFDM | -7.2dB |
| OOK-4 M=4 | 8bit | Envelope | -2.5dB |
|  |  | OFDM | -4.7dB |
|  | 16bit | Envelope | -2.2dB |
|  |  | OFDM | -4.3dB |

Proposal 1: -2.5dB SNR for OOK-based signal, and 4.7dB for OFDM-based signal.

Considering the differences between OFDM and OOK signals, two sets of noise figure requirements should be specified. For the OFDM signal, the NF values of the OFDM receiver and legacy UE receiver are similar, so we could assume the legacy UE receiver noise figure value 9dB as the baseline for the OFDM signal.

Proposal 2: Two sets of NFs for different LP-WUR types need to be specified.

Proposal 3: Set 9 dB as the baseline OFDM noise figure for the OFDM signal

For the OOK signal receiver, the NF value is much larger than the NF of legacy UE. During the study item phase, we listed some example NF value ranges:

- RF envelope detection: The noise figure is in the range of 12~22 dB.

Considering zero-IF receiver as a baseline architecture for envelop-based LP_WUR. We could choose the same NF range for the LP-WUS signal, which is 12-15dB. Since RAN1 chooses 7dB as the NF of MR, and the gap between MR and LR is 2/5/8dB, it is reasonable to choose 12 dB as the noise figure for the OOK signal.

Proposal 4: Choose 12 dB noise figure as the baseline for the OOK signal.

And the Table is shown as below:

Table 2. REFSENS with values for each element

| Type | NF | SNR |
| --- | --- | --- |
| OOK-based | 12dB | -2.5dB |
| OFDM-based | 9dB | -4.7dB |

2.1 ASCS and ACS

When defining ACS and ASCS for LP-WUR, two factors should be considered: the ASCS value on a dB scale and the required guard RB. If we only define the guard RB requirement without specifying an ASCS value, there would be no restriction on the in-band selectivity of LP-WUR. In extreme cases, even LP-WUR with the worst performance might pass the test, as there is only a guard RB limitation. Conversely, if we only define the ASCS value without any guard RB limitation, LP-WUR with the worst performance could also pass the test in extreme cases, as long as the guard RB is large enough.

Proposal 5: For ASCS and ACS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

Since ASCS requirement cannot be specified directly, we should define the test case for the ASCS requirement in the specification. The test parameters should include the power level, bandwidth, MDR, and other relevant parameters. The MDR will be verified in the test with the guard RBs as defined in the specification. And we could use the following table as the starting point to define the ASCS test parameters.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [0 or 1] at each side |
| MDR |  | 1% |

Proposal 6: Use the table 3 as the starting point of the ASCS test parameters.

For ACS requirement, we can use the legacy requirement as the starting point, and further consider to relax the value with guard RB. Based on the current simulation assumptions, we conducted simulations of 3rd-order and 5th-order filters under the conditions of OOK-4 and M=4, and the results are shown in the figure below. The simulation results show that the number of guard RBs has a significant impact on the ACS value. Under the condition of 1 guard RB, the ACS value ranges from 22 to 24 dB. Considering the impact of 1 RB, and combining the above analysis, we recommend selecting 1 RB as the guard RB and setting the ACS value to [22-24 dB].

Proposal 7: Select 1 RB as the guard RB and setting the ACS value to [22-24 dB].

# 3. Conclusions

In this contribution, ASCS, ACS and REFSENSE are discussed with following observations and proposals.

Proposal 1: -2.5dB SNR for OOK-based signal, and 4.7dB for OFDM-based signal.

Proposal 2: Two sets of NFs for different LP-WUR types need to be specified.

Proposal 3: Set 9 dB as the baseline OFDM noise figure for the OFDM signal

Proposal 4: Choose 12dB noise figure as the baseline for the OOK signal.

Table 2. REFSENS with values for each element

| Type | NF | SNR |
| --- | --- | --- |
| OOK-based | 12dB | -2.5dB |
| OFDM-based | 9dB | -4.7dB |

Proposal 5: For ASCS and ACS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

Proposal 6: Use the table 3 as the starting point of the ASCS test parameters.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [1 or 0] at each side |
| MDR |  | 1% |

Proposal 7: Select 1 RB as the guard RB and setting the ACS value to [22-24 dB].

# 4. Reference

- R4-2420528, WF on UE RF requirements for LP-WUS