3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500865

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.1

Source: 	CMCC

Title: 	Discussion on SBFD general part

Document for:	Discussion

# 1. Introduction

During the previous session, the WF on SBFD BS requirements was approved with multiple consensus points [2]. In this submission, we proceed with the discussion on the general aspects, building upon the WF that was agreed upon in the last meeting.

- Discussion

  - SCS restriction for SBFD operation

During the study item phase, to achieve convergence in simulation results, we set the simulation assumptions to a 100MHz bandwidth with a 30kHz SCS for the FR1 bands and a 200MHz bandwidth with a 120kHz SCS for the FR2-1 bands. However, when it comes to actual deployment and vendors implement s, the 60kHz SCS has not been ruled out. Indeed, considering that industrial scenarios are crucial application areas for SBFD, the 60 kHz SCS is highly significant in such scenarios. Therefore, the use cases for 60 kHz SCS should be retained. Considering there is insufficient justification for its exclusion, it is suggested that we do not impose restrictions on the use of the 60kHz SCS.

Proposal 1: Do not set restriction on use of 60kHz SCS.

  - DL/UL subband configurations support for SBFD

In last meeting, agreements are listed as below.

Issue 1-3-1: Clarification on declaration and flexibility Agreement: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

The conclusions reached at the last meeting were all related to the restrictions on SBFD subband configurations. After simulation results of the study item, for MR BS and LA BS, the vast majority of companies believe that coexistence can be achieved. Therefore, there is no need to impose restrictions on the subband configurations for MR BS and LA BS

Proposal 2: There is no need to set restrictions on subband configurations for MR and LA BS.

Regarding the discussion on restrictions for macro base stations, the conclusion from the last meeting was that a maximum of [2 or 3] specific Uplink (UL) subband sizes are allowed to be declared and should be selected from the channel bandwidth sizes listed in Table 5.3.2-1 of TS38.104. In our view, one of the key benefits of SBFD is its flexible frame structure. To retain this advantage, we advocate for no restrictions on the size of subbands. However, considering the concerns of some companies, as well as preserving the feasibility of future base station implementations, we believe that allowing three types of UL subband configurations could already effectively restrict the WA BS.

Proposal 3: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared for WA BS.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Do not set restriction on use of 60kHz SCS.

Proposal 2: There is no need to set restrictions on subband configurations for MR and LA BS.

Proposal 3: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared for WA BS.

# 4. Reference

[1] TR 38.858,

[2] R4-2419907, Way Forward for [113][306] NR_duplex_evo_General