3GPP TSG-RAN WG4 Meeting # 114	                                        R4-2500885

Athens, GR, Feb. 17 – Feb. 21, 2025

Agenda Item: 7.27.2.1

Source: MediaTek Inc.

Title: Discussion on event triggered L1 report for LTM

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of event triggered L1 report.

# Discussion

## 2.1 Capabilities for Support of Event Triggering L1 Reporting

#### Issue 2-7: whether to introduce/update requirements for reporting criteria per measurement category <Candidate options> - Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> - Continue discussion.

Last meeting, there was proposal to let RAN1 discuss the number of events supported by UE at first. We are open to this.

Proposal 1: Open to let RAN1 discuss the number of events supported by UE at first.

## 2.2 L1 event triggered reporting delay

Last meeting, RAN4 further discussed how to define L1 event triggered reporting delay and reached the following progress. In the next, we would like to discuss the open parts.

#### Issue 2-4: periodic report and single report <Agreement> - RAN4 requirements shall cover both Event triggered periodic MR and Event triggered MR. #### Issue 2-5: Event triggered reporting delay for L1 reporting <Agreement> - Agree on the following high level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. #### Issue 2-6: impact of time-to-trigger (TTT) on RRM <Candidate options> - Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi) <Recommended WF> - Continue discussion. #### Issue 2-15: Impact of UL resource waiting time. <Candidate options> - Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi) - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore. - Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo) - Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///) - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel. <Recommended WF> - Continue discussion.

The first issue is how to deal with the case that measurement period of the cells corresponding to the event are different. We don’t think this is an issue. Even for L3 events involving serving cell and inter-frequency neighbor cells, there is similar issue. In our understanding, in legacy L3 event triggered reporting delay, the measurement period is referring to the maximum one of the involved frequency layers. In the real field, UE behavior will be as following. As shown in the following figure, assume UE has measured serving cell and neighbor cell before t0. Assume UE will measure serving cell first after t0. Then at t1, from UE’s point, RSRP of serving cell becomes RSRP1’ after the most recent measurement and the difference between serving cell and neighbor cell is RSRP2-RSRP1’. If RSRP2-RSRP1’> threshold, the report can be triggered. Otherwise, at t2, after measuring neighbor cell, the difference between serving cell and neighbor cell is RSRP2’-RSRP1’ which would be larger than the threshold and the event is met.

|  | t0 | t1 | t2 |
| --- | --- | --- | --- |
| Serving cell | RSRP1 | RSRP1’ | RSRP1’ |
| Neighbor cell | RSRP2 | RSRP2 | RSRP2’ |

Fig. 1

For L1 event triggered reporting delay, we are open to use similar wording as legacy L3 event triggered reporting delay or make it clear that the measurement period is the maximum one of the cells involved.

Proposal 2: For L1 event triggered reporting delay, open to make it clear that the measurement period referred is the maximum one of the cells involved.

Another issue is whether Treport in L1-RSRP measurement period shall be 0 in L1 event triggered reporting delay. For event triggered aperiodic MR, we think it is reasonable to set Treport =0 in L1-RSRP measurement period. But for event triggered periodic MR, we don’t think so.

Proposal 3: For event triggered aperiodic MR, Treport = 0 in L1-RSRP measurement period but not for event triggered periodic MR.

The next issue is whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements. In the legacy L3 measurement related test cases, TTT is set to 0, so it is not necessary to take TTT (time to trigger) into account. We are open to further check this issue if TTT can not be set to 0 in RAN2.

The last issue is whether to consider the impact of UL resource waiting time. We think it is enough to follow L3 event triggered reporting delay, i.e., add the following when writing the CR “This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.”

Proposal 4: Regarding the impact of UL resource waiting time, capture it in the same way as L3 event triggered reporting delay, i.e., add the following when writing the CR “This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.”

## 2.3 L1 filtering

At RAN2#126 (May 2024), it was agreed that the decision on whether the specified L1 filtering is needed should be left to RAN1 as below:

- RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

In our understanding, RAN2 assumption is that L1 filtering is needed and whether to have specified L1 filtering is up to RAN1. Since RAN1 has no concrete agreement yet, RAN4 needs to wait for more RAN1 input.

Proposal 5: Wait for more RAN1 progress on whether and how to consider L1 filtering.

# Summary

In this paper, we provide some views on event triggered L1 report in R19 mobility. We have the following proposals:

Proposal 1: Open to let RAN1 discuss the number of events supported by UE at first.

Proposal 2: For L1 event triggered reporting delay, open to make it clear that the measurement period referred is the maximum one of the cells involved.

Proposal 3: For event triggered aperiodic MR, Treport = 0 in L1-RSRP measurement period but not for event triggered periodic MR.

Proposal 4: Regarding the impact of UL resource waiting time, capture it in the same way as L3 event triggered reporting delay, i.e., add the following when writing the CR “This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.”

Proposal 5: Wait for more RAN1 progress on whether and how to consider L1 filtering.

# 4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#113.