3GPP TSG-RAN WG4 Meeting # 114	                                                         R4-2500888

Athens, GR, Feb. 17 – Feb. 21, 2025

Agenda item:	7.19.3

Source:	MediaTek Inc.

Title:	Discussion on RRM core requirements of AI/ML BM use case

Document for:	Discussion

# Introduction

In this contribution, we provide our views on core requirements of AI/ML BM use case.

# Discussion

## 2.1 Whether Rx beam information is known for predicted beams

In the past few meetings, RAN4 discussed whether Rx beam information is known for predicted beams. We would like to provide our view here.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

As captured in TR38.843 [1], in R18 SI, both DL Tx beam prediction and beam pair prediction are studied.

TR38.843 For both sub-use cases, the following alternatives are studied for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction (deprioritized) - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) Using BM-case1 as an example:

In R19 WI, as pointed out in WID, only DL Tx beam prediction is in scope and beam pair prediction is not in the scope.

RP-243244 - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Observation 1: Both DL Tx beam prediction and beam pair prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

In the next, we will focus on DL Tx beam prediction at first.

Take BM-case 1 as an example. Two alternatives had been studied:

- Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

- Alt. ii): Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

In Alt. i), if CSI-RS to be predicted is QCL Type-D to SSB in Set B, then Rx beam of the associated SSB can be applied to CSI-RS Tx beam. In this scenario, Rx beam of the predicted Tx beam can be known.

In R18 SI, the following options were evaluated for DL Tx beam prediction. It is possible to use a fixed Rx beam for all the beams at inference and Rx beam of the predicted beams can be assumed to be the fixed Rx beam. Also as argued last meeting, it is UE implementation whether to use a fixed Rx beam for inference or use Rx beam related information as model input.

TR38.843 For the purpose of DL Tx beam prediction evaluations, consider the following options for Rx beam as AI/ML model input for training and/or inference if applicable: - Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample. - Companies expected to report how to select the "best" Rx beam(s). - Option 2: Measurements of specific Rx beam(s). - Companies expected to report how to select specific Rx beam(s). - Option 3: Measurements of random Rx beam(s) per model input sample. - Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. Companies expected to report the measurement/RS overhead together with the beam prediction accuracy, as well as, how to find the quasi-optimal Rx beam with "previous measurement". - Other options are not precluded and can be reported by companies.

Proposal 1: Rx beam of the predicted Tx beam which is not measured can be known if the beam is QCL Type-D to a Tx beam in Set B.

Proposal 2: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

      - 2.2 Scenarios to define requirements for?

We had done some simulations with different Set B patterns when Set B is a subset of Set A. In the simulation, there are 32 Tx beams with 2 elevation angles and 16 azimuth angles in Set A and 8 Tx beams in Set B. We tried different set B patterns (e.g., in pattern 0, Set B = the first 8 beams in Set A) and got the Top 5/1 prediction accuracy requirements as in Fig. 1. It can be observed that the top 5/1 prediction accuracy of different patterns are varied. The gap is more than 10% between the best and worst.

Fig. 1

Observation 2: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Usually, when we define accuracy requirements in RAN4, the requirements defined is scenario agnostic. Take L1-RSRP measurement accuracy as an example. The requirements are applicable to all the channel models, all UE speeds, all BS radiation patterns and etc. But for AI/ML BM models, it seems not possible to define a scenario-agnostic performance requirements based on the above observations. In R18 MIMO, we faced similar issue when discussing how to define requirements for TDCP. At that time, due to the requirements vary with different UE speeds, different serving frequency as well as different channels, RAN4 decided to define the test cases for limited cases (i.e., under certain doppler shift and channel model) instead of defining scenario-agnostic accuracy requirements. So we propose to discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Proposal 3: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

As mentioned above, for each BM sub-case, there are different scenarios. If it is not possible to define define scenario-agnostic performance requirements, we shall discuss which scenarios to define requirements and test cases for. Take BM case 1 as an example. There are several candidate scenarios:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: RSRP with specific Rx beam

- Scenario 2: Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

  - Scenario 2a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2b: RSRP with specific Rx beam

As shown by the evaluation results in TR38.843, there is some performance loss compared to best Rx beams obtained from one shot measurements using a specific or sub-optimal Rx beam for DL Tx beam prediction. In addition, there are several ways to choose the specific Rx beam. We prefer to define requirements only for Scenario 2a which is the most basic one.

Evaluation results in TR38.843: For BM-Case 1 DL Tx beam prediction without UE rotation, for Top-1 beam prediction accuracy, compared to the best Rx beams obtained from one shot measurements, with quasi-optimal Rx beam performance degradation is observed: - evaluation results from 1 source show 2% beam prediction accuracy degradation when Set B = 1/2 Set A and 7% beam prediction accuracy improvement when Set B = 1/4 or 1/8 Set A, when using the best Rx beams obtained from previous exhaustive sweeping (20ms ago) of all beams in Set A, comparing with using the best Rx beam for each Tx beams in Set B obtained from current exhaustive sweeping, without considering UE rotation for 3km/h UE speed. Such beam prediction accuracy improvement may not exist when considering UE rotation and higher UE speed. - evaluation results from 1 source show 2.5% beam prediction accuracy degradation using the best Rx of each Tx beams obtained from previous exhaustive sweeping (20ms ago) than using the best Rx of each Tx beams obtained from current exhaustive sweeping, without considering UE rotation for 3km/h UE speed. - evaluation results from 1 source shows 6.6%/6.9%/32.1%/45% degradation using a stochastic model in which the UE Rx beam is randomly selected with average probability that the best Rx beam is selected equal to 87.1%/75.1%/34.3%/10.9% compared to using the best Rx of each Tx beams obtained from current exhaustive sweeping, without considering UE rotation - evaluation results from 1 source show 13% beam prediction accuracy degradation, with the assumption of the best Rx beam for each Tx beam obtained from previous exhaustive sweeping over all beams in Set A in a SSB-like structure (in the past 160ms for each Rx beam with every 20ms a burst of Set A of beams) without considering UE rotation for 3km/h UE speed. - evaluation results from 1 source show 3%~11% beam prediction accuracy degradation, with the assumption of the best Rx beam obtained from one specific Tx beam which is 1st Tx beam in Set B. - evaluation results from 1 source show 12% beam prediction accuracy degradation with the assumption of the best Rx beams obtained from one specific Rx beam which is the best between the same Rx beam for different panels. - In addition, evaluation results from 3 sources show 1%~4% and 6%~12% beam prediction accuracy degradation, with the assumption of the best Rx beam is used for 90% and 80% of the model input samples and random Rx beam for the remaining samples respectively. - Even though, AI/ML can still provide better performance than non-AI baseline option 2 (exhaustive beam sweeping in Set B of beams), e.g., 50%~60% beam prediction accuracy difference in terms of Top-1 beam prediction accuracy based on the evaluation results from 2 sources, where non-AI baseline option 1 (exhaustive beam sweeping in Set A of beams) provides 100% prediction accuracy.

Also it is up to UE implementation whether to use beam pair prediction models. However, as beam pair prediction is not in the WI scope, we propose not to define separate requirements or test cases for beam pair prediction.

Proposal 4: Further discuss which scenario among the following to define requirements and test cases for:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: RSRP with specific Rx beam

- Scenario 2: Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

  - Scenario 2a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2b: RSRP with specific Rx beam

Proposal 5: Not to define separate requirements or test cases for beam pair prediction in R19 WI.

      - 2.3 Impact of measurement error

Last meeting, the simulation settings were further discussed to evaluate the impact of measurement error in R4-2420480. In our simulations, Option 2 in R4-2420480 is adopted to get the distribution of BB error under different SNRs, i.e., TDL-C channel and single Tx beam. For a single SNR, Tx power is set as the normalized power 1 and noise is generated following Gaussian distribution. After tens of simulations, the average and variance of BB error under a fixed SNR can be obtained. Following above procedures, we get the mean and variance of BB measurement error under SNR= [-24, 10] dB with step 3dB at first (as in Fig. 2). In our previous simulation, due to lack of enough simulation on low SNR, the distribution of BB measurement error in our last meeting’s Tdoc R4-2417893 is not correct.

Fig.2

During inference, the way to add measurement error is shown in Fig. 3:

- In SLS simulation, SINR of each Tx-Rx beam pair is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 10dB which is as guided in TR38.843 Table 6.3.1-1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam pair, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24,3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

Fig. 3 Method of adding measurement error

We simulated the performance without RF error of BM-case 1 with classification model at first. The simulation results are provided as in Table 1. We simulated several cases to study the impact of measurement error. Considering that it is possible to train a model without measurement in the simulation but not possible in the real field, we simulated both case 2/3 (train without measurement error) and case 4/5 (train with measurement error). Also considering it is possible to get the genie aided ground truth without measurement error in simulation but not possible in the real field, we simulated both case 2/4 (genie aided ground truth without measurement error) and case 3/5 (ground truth with measurement error). It can be observed that the performance degraded the worst in case 3, train without error and inference with measurement error in both set B and set A. Although Case 5 (train with measurement error) is more aligned with how to train a model in the real field, it is ok to define requirements based on Case 3 considering how to train a model is totally up to UE implementation and RAN4 defines the minimum requirements.

Table 1 Simulation results for BM-case 1 with classification model – impact of measurement error

|  | Train with Error | Inference with Error | Input with error | Ground truth with error | BB error assumption | top-1/1 beam prediction accuracy (%) | top-3/1 beam prediction accuracy (%) | top-5/1 beam prediction accuracy (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | No | No | No | No | N.A. | 72.28 | 94.76 | 98.15 |
| Case 2 | No | Yes | Yes | No | Gaussian distribution with mean and variance corresponding to the calculated SINR | 52.75 | 81.11 | 90.11 |
| Case 3 | No | Yes | Yes | Yes |  | 45.56 | 71.8 | 81.27 |
| Case 4 | Yes | Yes | Yes | No |  | 55.19 | 83.12 | 92.7 |
| Case 5 | Yes | Yes | Yes | Yes |  | 50.58 | 76.83 | 84.87 |
| Note: In this set of simulations, there are 80% indoor UEs and 20% outdoor UEs. |  |  |  |  |  |  |  |  |

As shown in our another contribution R4-2500889, the upper bound of RSRP during the test is about -83.5dB. In the test, the impact of the upper bound should be considered. We have done some simulation and the results are shown in Table 2. It can be observed that filtering out the data exceeds TE’s maximum Tx power has little impact on the prediction accuracy no matter considering measurement error or not. However, the performance degrades little when all the UEs are outdoor UEs if measurement error is considered while the performance degrades significantly when most are indoor UEs. After checking RSRP distribution as shown in Fig. 4 and Fig. 5, it can be observed that when most of the UEs are indoor UEs, the percentage of RSRP lower than -120dB is much higher than 100% outdoor UEs. Low RSRP kind of leads to larger measurement error.

Table 2 Simulation results for BM-case 1 with classification model – impact of TE dynamic range and measurement error

|  | Train with Error | Inference with Error | Train: upper bounded by -83.5dBm | Inference: upper bounded by -83.5dBm | top-1/1 beam prediction accuracy (%) | top-3/1 beam prediction accuracy (%) | top-5/1 beam prediction accuracy (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 80% indoor, 20% outdoor | No | No | No | No | 72.28 | 94.76 | 98.15 |
|  | No | No | No | Yes | 72.46 | 94.87 | 98.25 |
|  | No | No | Yes | Yes | 72.1 | 94.81 | 97.95 |
|  | Yes | Yes | No | No | 50.58 | 76.83 | 84.87 |
|  | Yes | Yes | Yes | Yes | 47.77 | 74.34 | 82.79 |
| 100% outdoor | No | No | No | No | 76.56 | 95.82 | 98.15 |
|  | No | No | No | Yes | 72.93 | 94.55 | 97.76 |
|  | No | No | Yes | Yes | 71.67 | 93.38 | 97.08 |
|  | Yes | Yes | No | No | 71.85 | 93.92 | 97.2 |
|  | Yes | Yes | Yes | Yes | 65.53 | 90.07 | 96.79 |
| Upper bounded by -83.5dBm: The group of data is filtered out if RSRP of any Tx beam in set B is above -83.5dB. |  |  |  |  |  |  |  |

Fig. 4 RSRP distribution with 80% indoor+20% outdoor UEs

Fig. 5 RSRP distribution with 100% outdoor UEs

Observation 6: The performance degrades a bit when all the UEs are outdoor UEs if measurement error is considered while the performance degrades significantly when most are indoor UEs.

Observation 7: Filtering out the data exceeds TE’s maximum Tx power has little impact on the prediction accuracy.

Proposal 5: Discuss which UE distribution to be used in the test and to be defined requirements for, e.g., 80% indoor+20% outdoor UEs or 100% outdoor UEs.

Proposal 6: When defining the accuracy requirements for AI/ML BM models, the impact of measurement error should be considered.

Proposal 7: When defining the accuracy requirements for AI/ML BM models, the impact of the dynamic range of TE should be considered.

      - 2.4 LCM of AI/ML BM models

RAN1 had some discussion on the signalling procedure on applicable functionality reporting for beam management UE-sided model based on the RAN2 input in LS R2-2407848.

They reached the following agreement. In our understanding, after step 4, the corresponding AI/ML model used for the configured for applicable CSI report configuration(s) should be activated. If we consider legacy procedures, e.g., HO, PSCell addition and etc, CSI report configuration(s) can be configured at HO command or PSCell addition command. Now we are not sure whether CSI report configuration(s) in HO command or PSCell addition command will trigger model activation/switch. This depends on further discussion in RAN1/2. If CSI report configuration(s) in HO command or PSCell addition command will trigger model activation/switch, RAN4 should discuss whether there is impact on HO or PSCell addition delay.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Proposal 8: If some key parameter changes in HO will trigger BM model activation/switch, RAN4 should discuss whether and how to extend HO delay.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Observation 1: Both DL Tx beam prediction and beam pair prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

Proposal 1: Rx beam of the predicted Tx beam which is not measured can be known if the beam is QCL Type-D to a Tx beam in Set B.

Proposal 2: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

Observation 2: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Proposal 3: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Proposal 4: Further discuss which scenario among the following to define requirements and test cases for:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: RSRP with specific Rx beam

- Scenario 2: Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

  - Scenario 2a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2b: RSRP with specific Rx beam

Proposal 5: Not to define separate requirements or test cases for beam pair prediction in R19 WI.

Observation 6: The performance degrades a bit when all the UEs are outdoor UEs if measurement error is considered while the performance degrades significantly when most are indoor UEs.

Observation 7: Filtering out the data exceeds TE’s maximum Tx power has little impact on the prediction accuracy.

Proposal 5: Discuss which UE distribution to be used in the test and to be defined requirements for, e.g., 80% indoor+20% outdoor UEs or 100% outdoor UEs.

Proposal 6: When defining the accuracy requirements for AI/ML BM models, the impact of measurement error should be considered.

Proposal 7: When defining the accuracy requirements for AI/ML BM models, the impact of the dynamic range of TE should be considered.

Proposal 8: If some key parameter changes in HO will trigger BM model activation/switch, RAN4 should discuss whether and how to extend HO delay.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface