3GPP TSG-RAN WG4 Meeting # 114	                                                         R4-2500889

Athens, GR, Feb. 17 – Feb. 21, 2025

Agenda item:	7.20.2

Source:	MediaTek Inc.

Title:	Discussion on testing framework and procedure for one-sided models

Document for:	Discussion

# Introduction

In this contribution, we provide our views on c testing framework and procedure for one-sided models.

# Discussion

      - 2.1 Test setup for AI/ML BM

### 2.1.1 Testing goal

In R18 SI, the following principle was identified: The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field (in TR38.843).

In General part, RAN4 discussed the testing goals of this WI and reached the following agreement in RAN4#112.

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

As observed in the simulation results in Fig.1, in terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference. RAN4 once agreed to use synthetic channels in R19 AI/ML test. Now we wonder whether the simulation results under synthetic channels can reflect the performance of AI/ML models in the real field. If not, we think it may be necessary to design a test system that is possible to use field data in the test. The key issue to use field data in the test is how to get the test dataset. We understand that it is not easy to figure this out.

Considering the potential fact that the performance under synthetic channels may not reflect the performance of AI/ML models in the real field and the difficulty to use field data in the test, we think at least for AI/ML BM, RAN4 needs to discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 1: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 2: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 3: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

### 2.1.2 Test set up

#### 2.1.2.1 Single AoA test system

For DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated, no matter RSRP of Tx beams in Set B is the one obtained from exhaustive sweeping or a specific fixed Rx beam. It is only the final L1-RSRP value that matters. All the spatial characteristics will be reflected on RSRP. Instead of emulating CDL channels and [64-128] Tx beams through multiple AoAs and AoDs, the spatial-selective and time-varying propagation conditions can be finally reflected on received RSRP at the test UE through adjusting Tx power at TE on each beam at different time. This is also the method which is widely used in legacy RAN4 RRM tests.

Proposal 4: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. In the next, we would like to explain that using real field data is also possible with single AoA test system.

As shown in Fig.2, in the data collected from real field, the measured RSRP (at baseband, including Rx beamforming gain) of Tx#1 and Tx#2 are P1 and P2, respectively. Using single AoA test system, with Rx beam lock, the Rx beamforming gain during the test can be fixed, assuming it is G. If the transmit RSRP at TE for Tx#1 and Tx#2 is P1-G and P2-G, then the received RSRP at UE baseband will be P1 for Tx#1 and P2 for Tx#2 which are exactly the same as the measured RSRP in real field. The Rx beamforming gain G can be derived by letting TE transmits a fixed high power P0, G is equal to P0’-P0 where P0’ is the RSRP measured at UE. This can be done multiple times to derive an average G to reduce the impact of noise.

Fig. 2

If to use field data, TE transmission power for each Tx beam in set B can be determined as shown in Fig.4.

Fig. 3

If to use synthetic channels defined in 38.901, BS/UE antenna configuration & radiation pattern defined in 38.901 can be used to determine TE transmission power for each Tx beam in set B (as shown in Fig.4).

Fig. 4

In computer simulations, it is possible to get RSRP of each Tx beam under a specific gNB settings (Tx spatial radiation pattern, BS height, downtilt angle and etc.), a specific channel (e.g., UMi) and a specific UE radiation pattern (defined in 38.901) at a random UE location with random UE rotation. Though computer simulation, it is possible to get a set of RSRP values, as shown in Table 1. Take Data 1 as an example, if the transmission power at the probe is RSRP#B1-1 for Tx beam B1, the channel between the probe and UE is purely AWGN channel, and UE beamforming gain is 1, then the measured RSRP of Tx beam B1 at UE is RSRP#B1-1. As the channel between the probe and UE may be not purely AWGN channel and UE beamforming gain is not 1, the real transmission power at the TE shall be calibrated.

Table 1

| Data set | Set B | Set A |
| --- | --- | --- |
| Data 1 | RSRP#B1-1, …,RSRP#BM-1 | RSRP#A1-1, …,RSRP#AN-1 |
| Data 2 | RSRP#B1-2, …,RSRP#BM-2 | RSRP#A1-2, …,RSRP#AN-2 |
| …… |  |  |
| Data L | RSRP#B1-L, …,RSRP#BM-L | RSRP#A1-2, …,RSRP#AN-L |

As the data set can be easily obtained through simulation for varied scenarios and settings, we think single AoA test system can be easily extended to different scenarios and settings. The issue of single AoA test system is that UE radiation pattern is used to generate the data set and the radiation pattern of UEs varies.

We evaluated the performance of models trained with a UE antenna radiation pattern (case 2) and inference on UEs with varied antenna radiation patterns (case 1,3, 4) as shown in Fig. 5. It can be observed that AI/ML BM Tx beam prediction model generalize well for different antenna radiation patterns. Therefore, we think it is enough to use a specified UE radiation pattern during the test.

Fig. 5

Observation 1: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

For set A, TE can get the generated RSRP value for each beam as shown in Table 1. Then after some calibration, we think TE can also know the expected RSRP of Tx beams at UE baseband, i.e., roughly transmitted RSRP at TE + fixed Rx beamforming gain.

Observation 2: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

With single AoA, existing IFF and enhance IFF test systems can be re-used for AI/ML BM test.

Proposal 5: Use existing IFF or enhance IFF test systems with single AoA to test R19 DL Tx beam prediction models.

Proposal 6: Standardized UE radiation pattern in 38.901 can be assumed in R19 AI/ML BM test with single AoA test system.

#### 2.1.2.2 Multiple AoAs test system

In the past few meetings, there were some proposals to further enhance multiple AoAs test system for AI/ML BM test. In our understanding, the intention is to use enhanced FR2 MIMO OTA test systems for AI/ML BM test. After studying the Tdocs, we still have some concerns.

We also think UE rotation is necessary if to use enhanced FR2 MIMO OTA test systems. Without UE rotation during the test, Rx beamforming gain for each AoA is basically fixed. For BM-Case2, to evaluate temporal DL Tx beam prediction, there should be some variation in beam direction over time, including UE rotation. But according to the information provided by TE vendors [2-5], it may be difficult and costly to support frequent and flexible UE rotation, i.e., rotation 2o, 5o. Now, the time needed to rotate UE is about 1s, which is not acceptable.

Proposal 7: If to use multiple AoAs test system for AI/ML BM test, UE rotation should be considered.

Observation 3: The time needed to rotate UE in FR2 MIMO OTA test systems, i.e., about 1s, is not acceptable for AI/ML.

To reduce the complexity, how to generate a simplified CDL channel was studied. It seems the analysis focused on the similarity of the simplified CDL channel and CDL channel. But we think it is the impact on model performance that really matters. Proponents should analyse the impact on AI/ML BM models by using simplified CDL channels.

Proposal 8: The impact on AI/ML BM models by using simplified CDL channels should be studied at first before concluding to use multiple AoAs test system.

To use multiple AoAs test system, the test will include two steps. In the first step, TE generates the channel and transmits Tx beams in Set B only. UE reports prediction results. In the next step, TE tries to generate the same channel as in the first step and transmits all the Set A Tx beams. The order of the above steps can also be reversed. In the test, as the measurement results may be 10dB lower or 20dB higher the transmitted RSRP at TE, it is difficult to check whether UE reports the real measurement results of Set A or reports fake values according to the previous predicted results to pass the test. Another issue is how to ensure the channels in these two steps are the same. We observed there were proposals to make the mapping relation of RS index and beam index different in the two steps. We admit it is workable to avoid UE cheating in the test. But this may also make model monitoring fail and is not preferred.

Proposal 9: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

#### 2.1.2.3 Dynamic range of Single AoA and Multiple AoA test systems

In early releases, dynamic range of IFF and DFF test systems had been analysed. Since several years have passed, there may be some new progress. But we are not experts, so here we just copy the previous results in TR38.810 here for information.

Table 2

| Test methods | Wanted signal (dBm/SCS) at reference point: upper bound | SNR at baseband (dB): upper bound | SNR at baseband (dB): upper bound – 30dB |
| --- | --- | --- | --- |
| IFF, peak fine beam | -83.5 | 25.1 | -4.9 |
| IFF, peak rough beam | -83.5 | 18.1 | -11.9 |
| IFF, non-peak fine beam | -83 | 13 | -17 |
| IFF, non-peak rough beam | -83 | 6 | -24 |
| DFF, peak fine beam | -83.8 | 24.8 | -5.2 |
| DFF, peak rough beam | -83.8 | 17.8 | -12.2 |
| DFF, non-peak fine beam | -83.3 | 12.7 | -17.3 |
| DFF, non-peak rough beam | -83.3 | 5.7 | -24.3 |

Considering the upper bound of SNR with rough beam are much lower than that with fine beam as shown in Table 1, fine beam should be used in AI/ML BM test to get a larger dynamic power range. Next we will only focus testing using fine beam.

Observation 4: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 10: Fine beam should be used in AI/ML test.

It can be observed that the power level at reference point (before UE Rx beam forming) for different cases are basically the same as it is determined by maximum Tx power at TE.

The upper bound of SNR at UE baseband is determined by several factors. One of the factors is Tx power at TE. If TE can do better now, the upper bound of SNR can be higher.

Regarding the lower bound, in our understanding, it is not quite a limitation. As observed through the simulation results given in our contribution R4-2417893, the week beams should be excluded otherwise the predict accuracy of AI/ML BM model will be very poor after considering the impact of measurement error.  In our simulation, we excluded the beams which is 30dB weaker than the best beam, and the Top 5/1 prediction accuracy is above 90%. Therefore 30dB dynamic range may be enough for the test based on our initial simulations. We can get the corresponding SNR at baseband in the last column. It can be observed that SNR in the last column is above -6dB only when using peak fine beam. It is important to have a large SNR dynamic range, otherwise the measurement error will be large for many beams. If to use multiple AoA test system, it is not possible to use peak fine beam only. With single AoA, it is possible to use peak fine beam. And we propose to use peak fine beam in AI/ML BM test. It can be observed that the upper bound of SNR with non-peak fine beam is 12.1dB lower than that of peak fine beam, which means SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

Observation 5: Among all the test methods, only using peak fine beam can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 6: Multiple AoA test systems can not use peak fine beam only.

Proposal 11: Using peak fine beam in AI/ML BM test with single AoA test system.

Observation 7: SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Proposal 1: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 2: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 3: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

Proposal 4: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Observation 1: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

Observation 2: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

Proposal 5: Use existing IFF or enhance IFF test systems with single AoA to test R19 DL Tx beam prediction models.

Proposal 6: Standardized UE radiation pattern in 38.901 can be assumed in R19 AI/ML BM test with single AoA test system.

Proposal 7: If to use multiple AoAs test system for AI/ML BM test, UE rotation should be considered.

Observation 3: The time needed to rotate UE in FR2 MIMO OTA test systems, i.e., about 1s, is not acceptable for AI/ML.

Proposal 8: The impact on AI/ML BM models by using simplified CDL channels should be studied at first before concluding to use multiple AoAs test system.

Proposal 9: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

Observation 4: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 10: Fine beam should be used in AI/ML test.

Observation 5: Among all the test methods, only using peak fine beam can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 6: Multiple AoA test systems can not use peak fine beam only.

Proposal 11: Using peak fine beam in AI/ML BM test with single AoA test system.

Observation 7: SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2411177, Testability and interoperability issues for beam management, Korea Testing Laboratory, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 # 112, Aug. 2024.