3GPP TSG-RAN WG4 Meeting # 114	                                                           R4-2500890

Athens, GR, Feb. 17 – Feb. 21, 2025

Agenda item:	7.21.2

Source:	MediaTek Inc.

Title:	Discussion on impact on RRM requirements of AI mobility

Document for:	Discussion

# Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction use-case. In this contribution, we would like to discuss the related RRM requirements.

# Discussion

In RAN4#112bis, RAN4 agreed to start the discussion on scenario 2, 3, 4. In the next, we will focus on the three scenarios.

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

## 2.1 Metrics used to define the model performance

For AI/ML BM, there were a lot of discussions on the metrics used to define the model performance. Similar as AI/ML BM, the metrics should be discussed for AI/ML mobility too. RAN2 agreed to use average RSRP difference as prediction accuracy metric for RRM measurement prediction.

RAN2#126 Agreements Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases. average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged.

In the past two meetings, RAN4 also discussed the metric(s) to be used and reached the following progress:

RAN4#112bis Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress. RAN4#113 Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2 RAN4#113 Issue 2-1-1: Absolut and/or Relative RSRP Prediction Accuracy Way Forward: Session Chair: Encourage companies to bring proposals on the definition of relative RSRP and absolute RSRP. Subsequently, companies may also consider following candidate options to introduce RSRP accuracy requirements: - Option 1: RAN4 to consider relative RSRP accuracy requirements only. - Option 2: RAN4 to consider absolute RSRP accuracy requirements only. - Option 3: RAN4 to consider both absolute and relative RSRP accuracy requirements.

One issue to discuss is the metric to use. In our understanding, the divergence is whether to use beam-level metric for sub-use case 1. We think using cell-level RSRP is enough. In sub-use case 1, even the output of the model is beam-level result, cell level result will be generated. In our understanding, the three sub-use cases correspond to different models. Which kind of model to use is totally up to implementation. And no matter which model used, it is the accuracy of the final cell level result that matters. That’s why RAN2 agreed to the prediction accuracy metric “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases.

For information Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

Proposal 1: For RRM measurement prediction, only use cell level RSRP as the performance metric.

Another issue is the definition of absolute RSRP and relative RSRP. Last meeting, there was some question on why legacy definition is not applicable.

In legacy definition, absolute RSRP accuracy = measured result – ground truth, and relative RSRP = (measured RSRP of cell 1 – measured RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2).

In legacy test case set up, there is single SSB beam, no L3 filtering and the transmitted power at TE is fixed during the test duration. Although how to do L1 filtering is up to UE implementation and L3 filtering can be adjusted based on NW configuration, under the legacy setup, the RSRP at A=A1=B=C in Fig.1. So in FR1 the transmitted power at TE is also the ground truth, and in FR2 the range of ground truth is derived through transmitted power at TE and UE Rx beamform gain range.

Fig. 1

But in AI mobility test, the test setups are in discussion, whether to set single SSB beam or L3 filtering is not settled down. There may be two ways to handle this. One is to define absolute RSRP and relative RSRP accuracy as legacy and define the accuracy requirements with legacy definition. But extra margin should be added on top of the defined accuracy requirements considering that the ground truth may be derived based on transmitted power at TE and other factors, which is similar as the legacy way in FR2 (the range of ground truth is derived through transmitted power at TE and UE Rx beamform gain range). Another way is to define absolute RSRP and relative RSRP accuracy according to the test setup. In this direction, the definition of ground truth should be further discussed. And the accuracy requirements are to be defined under a certain test setup.

Although there are no simulation results in R4 yet and R2 is investigating the generalization of AI mobility model, we think the accuracy requirements finally defined in RAN4 will be scenario specific from the experience in AI/ML BM. With this understanding, there is no difference to go with either of the above two directions.

Proposal 2: For RRM measurement prediction, make down-selection from the following two options:

- Option 1

  - Absolute RSRP and relative RSRP accuracy is defined as legacy, i.e.:

    - absolute RSRP accuracy = reported result – ground truth

    - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2)

  - extra margin should be added on top of the defined accuracy requirements in the test considering that transmitted power at TE may not be used as the ground truth directly.

- Option 2

  - Absolute RSRP and relative RSRP accuracy is defined based on the test setup, i.e.:

    - absolute RSRP accuracy = reported result – “ground truth”

    - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (“ground truth” of RSRP of cell 1 – “ground truth” of RSRP of cell 2)

  - “ground truth” is the referenced RSRP to compare with in the test.

Regarding the referenced RSRP to compare with in the test, it will be highly related to the test setup. In our view, for scenarios 2 and 3, it is enough to set single SSB beam and not to use L3 filtering. If so, the RSRP at A1=B=C. But we don’t think it is reasonable to keep the transmitted power at TE fixed during the test duration, which means A≠A1. With an assumed L1 filtering algorithm, it is possible to derive a reference ground truth based on the transmitted power at TE with conductive test. But UE may not use the assumed L1 filtering algorithm. Extra margin should be added.

Proposal 3: For scenario 2 and 3, set single SSB beam and not to use L3 filtering in the test.

Proposal 4: For scenario 2 and 3, the “ground truth”, i.e., referenced RSRP to compare with in the test, can be derived based on the transmitted power at TE and the assumed L1 filtering.

In the next, we would like to discuss the accuracy requirements to define for each scenario.

For scenarios 2 and 4, i.e., intra-frequency intra-cell temporal prediction, the final output will be cell-level measurement result of a single cell. So the only requirement to define is the absolute accuracy of predicted/generated L3 cell level RSRP.

Proposal 5: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated cell level RSRP.

For scenario 3, i.e., FR1 to FR1 inter-frequency inter-cell prediction (frequency domain), now RAN2 focuses on the prediction on single cell on a single inter-frequency layer. In our understanding, the benefit of scenario 3 comes from allowing to use a much longer MGRP or skipping some gap occasions for inter-frequency measurements. As well known in RAN4, if UE only predicts the measurement results on a single cell, it is not possible to reduce MGRP. UE is supposed to use MG to search and measure all the inter-frequency cells on the same and different frequency layers. We believe the scenario is more valuable to predict the measurement results for multiple cells and cells on multiple frequency layers. According to RAN2 agreement, they will focus on cluster based approach and the outputs are RSRP of several cells.

RAN2#128 Agreement: - For inter-frequency, cluster as input (i.e. measurements from different cells as inputs) can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. - In cluster approach the model takes measurements from more than one cell as inputs.

There will be absolute and relative accuracy requirements for discussion. In our understanding, the requirements to define shall meet the needs. If the predict measurement results are used directly for HO decision, both absolute and relative accuracy of the predict results are important.

Proposal 6: For scenario 3 of RRM measurement prediction, define the absolute and/or relative accuracy requirements of predicted/generated cell level RSRP.

## 2.2 Cell identification

It is observed that there were some proposals to study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML or whether the predicted cell is known or unknown. We agree that this is an important issue for further study. Due to cell identification related discussion is usually conducted in RAN4, RAN2 may not realize the issue yet and it is more appropriate to discuss the issue in RAN4.

For scenario 2 and 4, since they are intra-cell prediction, UE is supposed to detect and measure the cell before prediction. So cell identification is not an issue.

For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), since measurement on target cell is not always conducted, RAN4 should discuss the impact on cell identification. As mentioned above, RAN2 will focus on frequency domain for cluster based approach. Here we also focus on cluster-based approach. As shown in the example in Fig. 2, the inputs are RSRP of 21 cells (assume Cell#0,1, …,20) on frequency#1, the outputs are RSRP of 21 cells (assume Cell#0’,1’, …,20’) on frequency#2. Cell#X and Cell#X’ are co-located.

Fig. 2

Though cell detection, UE can get Cell ID and rough timing. In collocated scenario as shown in Fig. 2, it is possible to inform UE cell ID of Cell#X’ which is co-located with Cell#X. But if extending Scenario#3 to non-collocated cases, we are not sure whether it is workable to inform UE cell ID of the neighbor cell(s) by serving cell.

Observation 1: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases.

Regarding timing information, it is not possible to get from serving cell unless collocated cells are well synchronized. So in scenario 3, the cells predicted but not measured recently are unknown.

Proposal 7: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown.

In AI/ML BM, there is similar discussion on whether the predicted TCI state is known or unknown. And the final conclusion will have impact on TCI state switching procedures but would have little impact on model monitoring. But for AI mobility, whether the predicted cell is known or not would also have some impact on model monitoring, as UE is supposed to measure the predicted cells during model monitoring. For the cells which have been detected recently, e.g., in 5s, UE can measure the cell directly without cell search. But if the cell has not been detected yet, UE should perform PSS/SSS detection at first and then L3 measurement. So, if model monitoring is performed frequently, the gain will become less. But if UE performs model monitoring every several seconds. The time needed to finish one round of monitoring will be long.

We think it is important to study when UE performs cell detection on the predicted cells as this would have impact on model monitoring.

Proposal 8: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring.

## 2.4 Other RRM impacts

UE measurement and/or report behavior will be different from legacy for all the scenarios 2,3,4. RAN4 shall study whether there is any impact on measurement delay and report delay requirements. Regarding the impact on report requirements, there will be similar issue for AI/ML BM. We suggest to wait for more progress in AI/ML BM.

Proposal 9: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Next, we will focus on the impact on measurement delay requirements.

Scenario 2, FR1 to FR1 intra-frequency intra-cell temporal domain case B prediction, is to predict a subset of measurement instances in temporal domain of the same cell. This scenario is similar as BM case 2. The impact on measurement delay would take BM case 2 as a basis.

Proposal 10: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Scenario 3, FR1 to FR1 inter-frequency inter-cell (frequency domain), in our understanding, this scenario includes “prediction on an inter-frequency layer based on measurement on an intra-frequency layer” and also “prediction on an inter-frequency layer based on measurement on a different inter-frequency layer”. As discussed in previous part, the scenario is more valuable to predict the measurement on multiple frequency layers. In our understanding, MG still needs to be configured at least for model monitoring. If there are more than one inter-frequency layers involved, RAN4 needs to discuss the gap sharing mechanism among different inter-frequency layers both for measurement and monitoring. There would be some impact on measurement delay requirements.

Proposal 11: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Scenario 4, FR2 to FR2 intra-frequency intra-cell temporal domain case A, is to predict measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell without reducing any measurement in observation window. There is no impact on UE measurement behavior, so there is no impact on the measurement delay.

Proposal 12: Scenario 4 has no impact on measurement delay requirements.

# Conclusion

In this contribution, we provide our views for AI mobility. Proposals are summarized as follows.

Proposal 1: For RRM measurement prediction, only use cell level RSRP as the performance metric.

Proposal 2: For RRM measurement prediction, make down-selection from the following two options:

- Option 1

  - Absolute RSRP and relative RSRP accuracy is defined as legacy, i.e.:

    - absolute RSRP accuracy = reported result – ground truth

    - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2)

  - extra margin should be added on top of the defined accuracy requirements in the test considering that transmitted power at TE may not be used as the ground truth directly.

- Option 2

  - Absolute RSRP and relative RSRP accuracy is defined based on the test setup, i.e.:

    - absolute RSRP accuracy = reported result – “ground truth”

    - relative RSRP = (reported RSRP of cell 1 – reported RSRP of cell 2) – (“ground truth” of RSRP of cell 1 – “ground truth” of RSRP of cell 2)

  - “ground truth” is the referenced RSRP to compare with in the test.

Proposal 3: For scenario 2 and 3, set single SSB beam and not to use L3 filtering in the test.

Proposal 4: For scenario 2 and 3, the “ground truth”, i.e., referenced RSRP to compare with in the test, can be derived based on the transmitted power at TE and the assumed L1 filtering.

Proposal 5: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated cell level RSRP.

Proposal 6: For scenario 3 of RRM measurement prediction, define the absolute and/or relative accuracy requirements of predicted/generated cell level RSRP.

Observation 1: For scenario 3 (inter-frequency measurement prediction), it is possible to inform UE cell ID of the inter-frequency neighbor cell(s) to be predicted for collocated cases while may be not possible for non-collocated cases.

Proposal 7: For scenario 3 (inter-frequency measurement prediction), the cells predicted but not measured recently are unknown.

Proposal 8: Study when UE performs cell detection on the predicted cells, e.g., before model monitoring or during model monitoring.

Proposal 9: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility.

Proposal 10: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2.

Proposal 11: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

Proposal 12: Scenario 4 has no impact on measurement delay requirements.

# Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.