3GPP TSG-RAN WG4 Meeting #114	R4-2500895

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for (e)RedCap in NTN

Agenda item:	7.29.4.1

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on open issues for RedCap NTN RRM requirements based on the approved WF [1].

Discussion

General consideration on RRM requirements

Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN

<Agreement in RAN4#112bis> - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement. <Agreement in RAN4#112> - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - NR measurements for positioning in RRC_INACTIVE state - NR Handover to other RATs - Note: For NTN (e)RedCap, it refers to, for example, NTN to E-UTRA handover - RRC connection release with redirection to E-UTRAN - deriveSSB-IndexFromCell tolerance - Uplink spatial relation switch delay - Inter-RAT measurements in idle/inactive/connected mode - PRS-RSRPP measurements - NR measurements with autonomous gaps

In RAN4#112 meeting, there had agreement with a misclassification of the requirements for inter-RAT measurement in IDLE/INACTIVE mode for (e)RedCap and FR1-NTN. In IDLE and INACTIVE mode, inter-RAT E-UTRAN cells measurement has been defined for both NTN and (e)RedCap UE. Therefore, inter-RAT E-UTRA TN cells measurement requirements should be defined for NTN (e)RedCap UE in Rel-19.

- Proposal 1: Intorduce inter-RAT (NTN to E-UTRA TN) measurement in IDLE/INACITVE mode for NTN (e)RedCap UE in Rel-19.

The Requirements to be further determined

<Way Forward>: - For SDT requirements in RRC_INACTIVE state, - Proposal 1: RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements. - Proposal 1a: TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - Proposal 1b: TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission. - Proposal 1c: For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Proposal 1d: For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable) - Proposal 2: CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be: Measurement; FR1. RSRP1; (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)). RSRP2; (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2. Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment..

For SDT requirements in INACTIVE mode, there was FFS how to consider TA validation for NTN (e)RedCap UE. Basically, UE to support NTN can pre-compensate the feeder and service link delay, i.e., NTA,adjcommon and NTA,adjUE, when UE transmit uplink signal. Therefore, before transmission, if UE updates the NTA,adjcommon and NTA,adjUE, TA validation procedure could be removed. Or, if NTN (e)RedCap UE support RACH-less HO, TA validation procedure could be removed in SDT requirements.

- Proposal 2: RSRP-based TA validation procedure in CG-SDT requirements can be removed if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission.

eDRX configuration

< Agreement >: - For earth-moving LEO scenario: - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario. - For quasi-earth fixed LEO scenario: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - Requirements are applicable for 2.56s and 5.12s eDRX cycle - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

For quasi-earth fixed LEO deployment, there is FFS for 10.24s eDRX cycle and larger than 10.24s cycle. Considering LEO speed and cell size, measurement delay for 10.24s eDRX cycle would be longer, so it might affect cell reselection operation. Up to 5.12s eDRX cycle would be reasonable for quasi-earth fixed LEO deployment.

- Proposal 3: Prefer not to consider 10.24s eDRX cycle for quasi-earth fixed cell LEO deployment

Multiple SMTCs

RAN4 agreed to support RRM measurement on target cells belonging to multiple SMTC-s on single frequency carrier for RedCap NTN UE which is UE capability ‘parallelSMTC’.

<Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

For maxNumber-NGSO-SatellitesWithinOneSMTC and parallelMeasurementWithoutRestriction UE capability, considering (e)RedCap UE as reduced capability, this capability would introduce high impelementation complexity.

| maxNumber-NGSO-SatellitesWithinOneSMTC-r17 Indicates the number of different NGSO satellites for target cells that the UE supports of simultaneous measurements within a SMTC with value n1 corresponds to 1, value n2 corresponds to 2 and so on. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

- Proposal 4: Do not considr maxNumber-NGSO-SatellitesWithinOneSMTC and parallelMeasurementWithoutRestriction for RedCap NTN UE.

Requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Basically, for 2Rx (e)RedCap NTN, exsiting NR NTN requirmeents can be reused for L1/L3 measurement. However, depending on supporting UE capability (e.g., multiple SMTC) of NR NTN for (e)RedCap NTN, existing NR NTN requirements could be revised.

- Proposal 5: For 2Rx (e)RedCap NTN, exsiting NR NTN L1/L3 measurements requirements can be reused, and depending on supporting UE capability (e.g., multiple SMTC) of NR NTN for (e)RedCap NTN, the requirements sould be revised.

For 1Rx RedCap NTN, in RAN4 specification, extended measurement delay for RRM requirements for 1Rx RedCap UE have already considered, so RRM requirements for 1Rx RedCap NTN can be defined based on existing 1Rx RedCap RRM requirements. RAN4 needs to consider the following requirements that are not defined on 1Rx RedCap

- Proposal 6: For 1Rx RedCap NTN, existing 1Rx RedCap L1/L3 measurement requirements can be reused for 1Rx RedCap NTN

L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

The L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UE, the existing measurement accuracy for NR NTN and 1Rx RedCap can be reused for 2Rx RedCap NTN and 1Rx RedCap NTN UEs, respectively.

- Proposal 7: For L1/L3 measurement accuracy, the existing measurement accuracy for NR NTN and 1Rx RedCap can be reused for 2Rx RedCap NTN and 1Rx RedCap NTN UEs, respectively.

The impact of HD-FDD

In the last RAN1 meeting, RAN1 agreed the handling of collision between UL and DL in HD-FDD (case 3) due to TA mismatch as below.

<RAN1#119 Agreements> Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above. - This is signaled by RRC configuration. Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL.

For collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode, it is up to UE implementation, and for others, DL is prioritized as default priority if UL priority is not indicated by network. However, from the RRM measurement perspective, further clarification for measurement behavior is needed for following case;

- Collision between SMTC and UL when network indicates UL overriding DL

Even if UL has priority than DL by network indication, the UE needs to perform RRM measurement within SMTC duration. So, RAN4 needs to further discussion of measurement behavior for collision between SMTC and UL when network indicates UL overriding DL.

- Proposal 8: RAN4 should further discuss measurement behavior for collision between SMTC and UL when network indicates UL overriding DL in HD-FDD.

Conclusion

In this contribution, we provide views on open issues for (e)RedCap NTN, and we propose

- Proposal 1: Intorduce inter-RAT (NTN to E-UTRA TN) measurement in IDLE/INACITVE mode for NTN (e)RedCap UE in Rel-19.

- Proposal 2: RSRP-based TA validation procedure in CG-SDT requirements can be removed if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission.

- Proposal 3: Prefer not to consider 10.24s eDRX cycle for quasi-earth fixed cell LEO deployment

- Proposal 4: Do not considr maxNumber-NGSO-SatellitesWithinOneSMTC and parallelMeasurementWithoutRestriction for RedCap NTN UE.

- Proposal 5: For 2Rx (e)RedCap NTN, exsiting NR NTN L1/L3 measurements requirements can be reused, and depending on supporting UE capability (e.g., multiple SMTC) of NR NTN for (e)RedCap NTN, the requirements sould be revised.

- Proposal 6: For 1Rx RedCap NTN, existing 1Rx RedCap L1/L3 measurement requirements can be reused for 1Rx RedCap NTN

- Proposal 7: For L1/L3 measurement accuracy, the existing measurement accuracy for NR NTN and 1Rx RedCap can be reused for 2Rx RedCap NTN and 1Rx RedCap NTN UEs, respectively.

- Proposal 8: RAN4 should further discuss measurement behavior for collision between SMTC and UL when network indicates UL overriding DL in HD-FDD.

Reference

[1] R4-2420104, “WF on RRM requirements for NR_NTN_Ph3_Part1,” Moderator (CATT)