3GPP TSG-RAN WG4 Meeting #114	R4-2500898

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for downlink coverage enhancement in NTN

Agenda item:	7.29.4.2

Document for:	Discussion

Introduction

In the last RAN meeting, revised WID [1] has been approved including extended periodicity of SSB. In this contritubtion, we provide our views on open issues for downlink coverage enhancement for NR NTN based on the approved WF [1].

Discussion

In the last RAN#106, extended SSB periodicity was aggred and captured in revised WID as below.

- Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

In last RAN4 meeting, RAN4 disucussed which requirements would be impact due to up to 160ms SSB periodicity as below.

Agreement (online): For SSB periodicity up to 160ms: - No further discussion on the impact on initial access from the perspective of RRM requirement - Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW - RRC Re-establishment: TSMTC,i assumption - RRC connection release with redirection to NR: Trs assumption - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption - Further discuss the details on how to update the requirement. - Further discuss the following for SSB periodicity > 20ms: - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync - Further discuss whether and how to update the requirement

Based on revised WID, the maximum of the additional default value for SSB periodicity is 160ms. It means that the default value for SSB periodicity could be one of {20, 40, 80, 160}ms. Since there are multiple the default values, RAN1 will discuss whether and how to indicate the default value for SSB periodicity to UE, e.g., explicit/implicit indication or no indication, etc. Therefore, default value for SSB and SMTC periodicity in RAN4 RRM requirements listed in the above could be discussed based on RAN1’s furhter discussion.

- Proposal: Modifying default value for SSB and SMTC periodicity in RAN4 RRM requirements could be discussed based on RAN1’s furhter discussion (e.g., whether to indicate default value explicitly/implicitly or not).

Conclusion

In this contribution, we provide views on open issues for downlink coverage enhancement for NR NTN, and we propose

- Proposal: Modifying default value for SSB and SMTC periodicity in RAN4 RRM requirements could be discussed based on RAN1’s furhter discussion (e.g., whether to indicate default value explicitly/implicitly or not).

Reference

[1] RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” Thales, CATT

[1] R4-2420105, “WF on RRM requirements for NR_NTN_Ph3_Part2,” Moderator (Qualcomm)