# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500942

# Athens, Greece, 17th-21th Feb, 2025

# Title: 	Discussion on the RF requirement for fragmented carrier

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.6.3

# Document for:	Approval

# 1 Introduction

In the last meeting, companies continue discussing the methods of reducing the number of UE Rx chain[1], and there are some agreements captured in the approved WF [2]. Meanwhile, there still some open issues which was not discussed due to lack of time.

In this contribution, we give some views on some left open issues on the RF requirement for reducing the number of Rx chain for fragment carriers.

# 2	Discussion

One of the open issue is the maximum power level of in-gap interference, the WF is:

Issue 3-2: Discussion on maximum power level of in-gap interference For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS

There are two test case of parameters for ACS testing with different interferer signal, where up to -25dBm interferer signal is assumed. As we discussed in [3], up to -25dBm should be keep as the maximum power level of in-gap interferer to align with the existing intra-band non-contiguous CA ACS, where the UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active, where the -25dBm is the same as the maximum input level requirements. We didn’t see there are reasons to change it when the Rx chain is changed to fully shared RF chain for fragment carriers.

Proposal 1: -25dBm maximum acceptable absolute power of in-gap interferer for fragmented carriers using a single Rx RF chain.

For the PSD different, as we discussed in [3], we observed that the power difference, PSD difference and the carrier channel bandwidths are dependent each other, since PSD difference = Power different - 10log(BWinterferer/BWwanted). In current ACS/IBB test, the BWinterferer =5MHz is adopted for <2700MHz bands. We think this test configuration should be reused for fragment carriers in a single Rx RF chain. However, the channel bandwidth PCC or SCC are different for different band combination, and for some band combinations, 5MHz is not used for PCC or SCC. Looking at the existing intra-band DL NC CA case, the largest PCC channel bandwidth is 40MHz (for CA_n25(2A) where the aggregated channel bandwidth (PCC+SCC)= 40MHz +5MHz), which is the wanted carrier with the lower PSD (assume the power for 40MHz PCC and 20MHZ SCC are the same).

For the power different, 25dB is defined in the specification, and it is for non-collocated. But by using the value, the PSD difference = 25 - 10log(5/min(BWPCC, BWSCC).

Proposal 2: The [PSD] difference could be [25] - 10log(5/min(BWPCC, BWSCC).

For the ΔRIBNC re-evaluation for PCC and SCC, we give some initial link budge calculation for n3 based on the templates given in the WF.

There are two configurations for CA_n3(2A) in the specification, the roughly calculation can be found in Annex.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0 |  |

Form the calculations, we observe:

Observation: For CA_n3(2A) using fully-shared Rx chain, it may:

For PCC=5MHz, SCC=5MHz, RIBNC_PCC_5M = 12.3dB. RIBNC_SCC_5M = 13.1dB

For PCC=40MHz, SCC=5MHz, RIBNC_PCC_5M = 12.2dB. RIBNC_SCC_5M = 13.1dB

# 3 Conclusion

In this contribution, we give some views on some left open issues on the RF requirement for reducing the number of Rx chain for fragment carriers.

Proposal 1: -25dBm maximum acceptable absolute power of in-gap interferer for fragmented carriers using a single Rx RF chain.

Proposal 2: The [PSD] difference could be [25] - 10log(5/min(BWPCC, BWSCC).

Observation: For CA_n3(2A) using fully-shared Rx chain, it may:

For PCC=5MHz, SCC=5MHz, RIBNC_PCC_5M = 12.3dB. RIBNC_SCC_5M = 13.1dB

For PCC=40MHz, SCC=5MHz, RIBNC_PCC_5M = 12.2dB. RIBNC_SCC_5M = 13.1dB

# 4	Reference

[1] R4-2418146, Topic summary for [113][124] FS_NR_DL_Frag_Carrier, Mediatek

[2] R4-2420519, WF on fragmented DL study, Mediatek

[3] R4-2418695, View on Fragmented carrier RF requirements, ZTE Corporation, Sanechips

# Annex:

For 5M+65Mgap+5MHz configuration:

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise (11 bit): -68.0dBFs (6.02N+1.76) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to 0dBFs (PAPR is not considered) | Tx leakage mapping to -12dBFS (PAPR is considered) |
| PA out | 27 | - | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - | - |
| FE loss | - | 4 | 4 | - | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - | - |
| LPF attenuation | 12.7 | 1 | 1 | - | - | - |
| LPF output (dBm) | -35.7 | -102 | -97.3 | - | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - | - |
| NF increase due to AGC adjustment | - | - | - | 11 | - | - |
| Noise floor | - | - | - | -90 | - | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -96.3 | - | - | - |
| ADC output (dBm) | - | - | - | - | -114.1 | -102.1 |
| SNR after ADC (dB) | - | - | - | - | 12(PCC) | 12.3(PCC) |
|  |  |  |  |  | 12.9(PCC) | 13.1(PCC) |

For 5M+45Mgap+5MHz configuration:

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise (11 bit): -68.0dBFs (6.02N+1.76) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to 0dBFs (PAPR is not considered) | Tx leakage mapping to -12dBFS (PAPR is considered) |
| PA out | 27 | - | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - | - |
| FE loss | - | 4 | 4 | - | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - | - |
| LPF attenuation | 14.1 | 1 | 1 | - | - | - |
| LPF output (dBm) | -37.1 | -102 | -97.3 | - | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - | - |
| NF increase due to AGC adjustment | - | - | - | 11 | - | - |
| Noise floor | - | - | - | -90 | - | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -96.3 | - | - | - |
| ADC output (dBm) | - | - | - | - | -115.5 | -103.5 |
| SNR after ADC (dB) | - | - | - | - | 12(PCC) | 12.2(PCC) |
|  |  |  |  |  | 12.9(PCC) | 13.1(PCC) |