3GPP TSG-RAN4 Meeting # 114	R4-2500943

Athens, Grace, Feb.17th – 21th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	On EESS new NS modified MPR behaviour

# Agenda item:	6.22.2

# Document for:	Approval

# 1 Introduction

In last RAN4 meeting, the A-MPR associated newly NS_XX were discussed, and the agreements were captured in the WF[1], where for the newly NS_205and CA_NS_205, the agreements are:

<Way forward for NS value>: - Introduce NS_205 and CA_NS_205 for the limit of -5dBm/200 MHz in the 23.6-24.0 GHz frequency range for n257 and n258. - Apply modifiedMPRbehavior indication, which applies for both n257 and n258. NS_205 and CA_NS_205 is mandatory and the corresponding modifiedMPRbehavior bit shall be indicated for UE(s) from Rel-15.

And for the legacy NS_202 the agreements are:

Updated NS_202 by replacing +1 dBm/200 MHz with -5dBm/200MHz EESS passive services protection in the 23.6GHz-24GHz for band n257 and n258, and define modifiedMPRbehaviour bit to differentiate between legacy NS_202 and new NS_202 requirements. - It is mandatory to report modifiedMPRbehaviour bit from Rel-15 for new UEs. - A-MPR for updated NS_202 - For PC1: 11.0 dB - For other PCs: 2.0 dB The Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the channel bandwidth. - A-MPR for updated CA_NS_202 - For PC1: For contiguous UL CA with contiguous allocation, 11.0 dB For contiguous UL CA with non-contiguous allocation or NC UL CA, [13.0 dB] if offset frequency < BWintraCA, 11.0dB otherwise. - For other PCs: For contiguous UL CA with contiguous allocation, [5.0 dB] if offset frequency < BWintraCA, 2.0dB otherwise. For contiguous UL CA with non-contiguous allocation or NC UL CA, [8.0 dB] if offset frequency < BWintraCA, 2.0dB otherwise. The Offset frequency is defined as the frequency from the upper edge of the protected frequency range to the lower edge of the lowest CC among the configured UL CA.

In this contribution, we give some discussions on how to implement the above modifiedMPRbehavior for the newly NS_XX and legacy NS.

# 2	Discussion

The purposes of adopting the modifiedMPR-Behaviour bits indication from Rel-15 are not only to give Rel-15 UE chance to improve its performance compliance with the new MPR or A-MPR requirements modified in a given version of later release specification, but also to let NW know which NS values each of UEs can support in the case of two types of UE existing in a real environment: one is UE working with an existing NS and the other is UE working with a new NS.

Using modifiedMPR-Behaviour to indicate the MPR or A-MPR that UE complies is common in 38101 series.

In terms of the above WF, we can observe:

Observation 1: ModifiedMPRbehavior indication applies for newly NS_205 and CA_NS_205 for both n257 and n258.

Observation 2: ModifiedMPRbehavior indication applies for legacy NS_202 for both n257 and n258.

It seems the legacy CA_NS_202 is missing in ModifiedMPRbehavior indication in the WF since the A-MPR values for CA_NS_202 are also updated. Thus for the same purpose of NS_202, ModifiedMPRbehavior indication should also applied for legacy CA_NS_202 for both n257 and n258 to differentiate between legacy CA_NS_202 and new CA_NS_202 requirements.

Proposal: Define modifiedMPRbehaviour bit to differentiate between legacy CA_NS_202 and new CA_NS_202 requirements.

The impacts on the TS38.101-2 specification can be found in the companion draft CR can be found in [3] in this meeting.

# 3 Conclusion

In this contribution, some discussions on the modifiedMPRbehavior for the newly NS_XX and legacy NS.

Proposal: Define modifiedMPRbehaviour bit to differentiate between legacy CA_NS_202 and new CA_NS_202 requirements.

The impacts on the TS38.101-2 specification can be found in the companion draft CR can be found in [3] in this meeting.

# 4	Reference

[1] R4-2420356, WF on UE RF requirements for EESS protection, Huawei, China Unicom

[2] R4-2418706, On EESS mmWave UE spurious emission, ZTE Corporation, Sanechips

[3] R4-2500944, Draft CR to TS 38101-2 - EESS protection(Annex H), ZTE Corporation, Sanechips