3GPP TSG-RAN4 Meeting # 114	R4-2500946

Athens, Grace, Feb.17th – 21th, 2025

# Title: 	On 3T6R SRS antenna switching requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.1.3.3

# Document for:	Approval

# 1 Introduction

In Oct. Meeting, the ∆TRxSRS requirements for 3T6R SRS switching cases were discussed initially, in which t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 are included:

Agreement: - As this is the first RAN4 meeting since the confirmation that RAN1 will consider t3r6, companies are encouraged to provide ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 at RAN4#113 for further discussion with the Apple and Qualcomm proposals.

In Nov. Meeting, companies provides the propoals and it comes to be converged. However, the antenna switching cases and the final ∆TRxSRS values will be needed for further checking.

In this contribution, we continue giving the discussions on the ΔTRxSRS requirements for the 3T6R SRS antenna switching.

# 2	Discussion

The agreemeents were included in the approved WF [1], where there are some open issues related to the 3T6R ∆TRxSRS requirements, inclduing the available patterns and the values.

For the available patterns, there are two options:

Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS Way Forward: RAN4 to further discuss the following options. - Option 1: Consider a subset of available patterns for 3T6R AS-SRS. FFS on the subset details. - Option 2: Consider all available patterns for 3T6R AS-SRS.

By looking back 4Rx and 8Rx, RAN4 never restrict the SRS antenna switching patterns, instead RAN4 define the requirements for all the available SRS antenna switching patterns which are aligned with RAN1, this is also true for 1T6R and 2T6R discussion. For 3T6Rx, we do not see there are reasons to only consider a subset of available patterns. Therefore, RAN4 should consider all the available 3T6R AS-SRS patterns.

Proposal 1. Consider all available patterns for 3T6R AS-SRS.

For the ∆TRxSRS values, lots of companies give the proposals for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6, and the suggestion from moderator are:

Issue 1-2-1: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Way Forward: RAN4 to check whether the ∆TRxSRS values in the table below could be agreed. The average values from company proposals, as well as the existing values for other AS-SRS configurations, were considered as the starting point and were rounded to align with 0.5dB increments to keep consistency with the existing specification approach. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t3r6 (dB); ΔTRxSRS t1r6-t3r6 (dB); ΔTRxSRS t2r6-t3r6 (dB); ΔTRxSRS t1r6-t2r6-t3r6 (dB). Band n41, n77, n78; 3.0; 4.5; 4.0; 5.5. Band n79; 4.5; 6.0; 5.5; 7.0. Band n104; 5.0; 7.0; 6.0; 8.0.

In our understanding, different RFFE architecture assumption would cause different ∆TRxSRS values since the IL depends on which types of and how many switchers (such as SPDT, DPDT, SP4T, and etc), and our RFFE architecture assumption and evaluations can be found in [2] in detail. However, there are no discussion and agreements on the reference architectures and assumption, instead RAN4 discussed the value directly. Therefore, it would be fairness to consider all the inputs and use the averaging method.

For t3r6 SRS antenna switching, we think the situation is the same for t4r8 and t2r4, Therefore, it is propose to use the current ∆TRxSRS requirements of t2r4 or trt8 for t3r6, which are 3dB for n41/n77/n78 and 4.5dB for n79. For the others t3r6 SRS antenna switching, the values in the WF are fine to us except t1r6-t3r6 values for n104. We think the delta value between n79 and n104 for t1r6-t3r6 and t2r6-t3r6 should be the same considering the similar complexity, which is 0.5dB delta.

Proposal 2: Use the following 3T6R numbers as a reference when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 6.5 | 6.0 | 8.0 |

# 3 Conclusion

In this contribution, we continue discussing the ∆TRxSRS requirements for 3T6R SRS antenna switching. The conclusions are:

Proposal 1. Consider all available patterns for 3T6R AS-SRS.

Proposal 2: Use the following 3T6R numbers as a reference when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 6.5 | 6.0 | 8.0 |

# 4	Reference

[1] R4-2420376, WF on 6Rx UE RF requirements, AT&T

[2] R4-2418693, On 3T&6R SRS antenna switching requirement, ZTE, Sanechipes