3GPP TSG-RAN4 Meeting # 114	R4-2500951

Athens, Grace, Feb.17th – 21th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	On the open issues for R19 PC1.5 Intra-band UL CA

# Agenda item:	7.1.1.1.2

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements are included in the WF[1], and there are still some open issues. In this contribution, we give some further discussions on these open issues for PC1.5 intra-band contiguous and non-contiguous UL CA.

# 2	Discussion

The first open issues are MPR values for both handheld UE and FWA for both PC1.5 intra-band UL NC CA and CCA, and also the PSD assumptions. It was agreed to use equal PSD assumption to define MPR/A-MPR requirements for intra-band UL CCA.

For PC1.5 intra-band UL NC CA, some agreements are listed below:

#### 1.2 PSD assumption to derive MPR/A-MPR requirements for intra-band NC ULCA (with dualPA) Agreement: - Equal PSD as starting point - Companies are encouraged to check the MPR difference with 6dB PSD imbalance - Inform RAN5 about the PSD condition for MPR derivation in RAN4 - The MPR difference compared to PC2 is supposed to be less than 3dB for PC1.5 in order to have performance gain over PC2 Companies to clarify the PA configuration assumptions for MPR evaluation for PC3, PC2 and PC1.5 intra-band NC UL CA, for example, (PC3+PC3) for PC3

It can be see that the MPR difference compared to PC2 intra-band UL NC CA should be less than 3dB for PC1.5 intra-band UL NC CA. With our experience in the past, we think this assumption is reasonable to keep the 3dB increased power benefit. To achieve this goal for PC1.5 intra-band UL NC CA, some optimistic parameters such as IQ image shall be assumed in the simulation in the case of the PA model is suppose to be the same. Otherwise, the MPR difference would be higher than 3dB due to the higher IMD3 caused by two higher RBs/CCs in the PC1.5 intra-band NC CA cases, especially 6dB PSD imbalance between two RB allocations in the CCs are assumed. In other words, without some optimistic parameters, it would be difficult to achieve the goal of keeping the MPR difference within 3dB.

Observation 1: To achieve the goal of keeping the MPR difference within 3dB, some optimistic parameters shall be assumed in the simulation.

It is true that when the large non-equal PSD between CCs is assumed, then emission out of each CC would be large different, and lower RB would cause higher leakage emission. The IM3 limits can be calculated as 2*P1+P2-2*OIP3, thus 1dB power increased for P1 will cause 2dB power increased for IM3 limits out of the side of P1 and 1dB power increased for IM3 limits out of the side of P2, which is simply shown in Fig 1.

Fig 1. Increased IMD v.s. increased power

We think the situation is existed for either intra-band contiguous CA or intra-band non-contiguous CA no matter PC3/2/1.5 power class. More power increased, then more IMD3 can be seen which needs more MPR. In RAN4 spec, the MPR requirements should be applied to any UL PSD imbalance across uplink carriers, no matter equal or unequal PSD is assumed.

In last meeting, company[2]] gave some simulation results considering 6dB power imbalance between two RB allocations in the CCs, and the IQ image is assumed as 34dBc, rather than the traditional one (28dBc). Consequently, up to 2dB MPR difference between equal PSD and 6dB PSD imbalance for SEM of -13dBm/MHz and SE of -30dBm/MHz.

However, it seems there are no consequences for the MPR value for the equal PSD assumption for PC1.5 intra-band UL NC CA. If the MPR different between PC2 and PC1.5 intra-band UL NC CA with the assumption of the equal PSD is already higher than 1dB, then up to 2dB MPR difference between equal PSD and 6dB PSD imbalance will cause larger 3dB MPR different between PC2 and PC1.5 intra-band UL NC CA. In this case, it will lose the performance gain over PC2 for PC1.5.

Considering the above, we have the following proposals:

Proposal 1. No matter equal or unequal PSD assumption, the MPR difference compared to PC2 should be less than 3dB for PC1.5 in order to have performance gain over PC2

For Pcmax and Pcmax,c, the agreements are:

- PCMAX,C is upper bounded by 26dBm - How to reflect it in spec is FFS - For PCMAX, FFS in next meeting - Option 1: No change of current spec - Option 2: PPowerClass,CA = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm assuming 6dB PSD difference , or, =26+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2)) assuming equal PSD, can be used as a starting point

In our understanding, PCMAX is the total configured maximum output power and is set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

- For PC1.5 UL inter-band CA, it was already agreed that the upper bound of Pcmax is 29dBm for both intra-band contiguous CA and intra-band non-contiguous CA, which 29dBm is the PPowerClass,CA =PC1.5. Hence it seems no changes of specification for Pcmax for both intra-band contiguous ULCA with 2Tx-TxD and intra-band UL NC CA with dual PA architecture.

- Proposal 2: No changes of specification for Pcmax.

- The only issue would be how to reflect the Pcmax,c for PC1.5 intra-band UL NC CA in the specification.

- For intra-band contiguous CA with TxD, it was already agreed that no changes of specification for Pcmax,c, the current specification can well reflect the the upper bound of Pcmax,c is 29dBm per CC.

- For intra-band UL NC CA with dualPA-architecture, due to the upper bound of Pcmax,c is 26dBm per CC, rather than 29dBm, so some changes should be done to reflect the 3dB lower of the CA power class, i.e. Ppowerclass,CA-3.

- Proposal 3: Some changes should be done to reflect the upper bound of Pcmax,c is 3dB lower than the CA power class (29dBm) for PC1.5 intra-band UL NC CA with dualPA-architecture.

# 3 Conclusion

In this contribution, we give further discussions on some open issues for PC1.5 Intra-band non-contiguous UL CA. The proposals and conclusions are:

Observation 1: To achieve the goal of keeping the MPR difference within 3dB, some optimistic parameters shall be assumed in the simulation.

Proposal 1. No matter equal or unequal PSD assumption, the MPR difference compared to PC2 should be less than 3dB for PC1.5 in order to have performance gain over PC2

- Proposal 2: No changes of specification for Pcmax.

- Proposal 3: Some changes should be done to reflect the upper bound of Pcmax,c is 3dB lower than the CA power class (29dBm) for PC1.5 intra-band UL NC CA with dualPA-architecture

# 4	Reference

[1] R4-2420329, WF on HPUE for CA in TN, Samsung

[2] R4-2419155, HPUE for intra-band UL CA, LG Electronics