3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500990

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Further discussion on coexistence evaluations for Ku-band for NR NTN

Agenda item:	7.11.2

Document for:	Approval

- Introduction

In RAN4#113 meeting, a WF [1] on NTN Ku band coexistence study was agreed, and remaining issues on Ku-band coexistence assumptions in WF require further discussion.

In this contribution, we provide our views on coexistence evaluation for Ku-band for NR NTN.

- Discussion

Conducted power (before ohmic loss) per antenna element (dBm) for VSAT phased array antenna

As per the WF [1], the agreement concerning conducted power per antenna element for VSAT phased array antenna for Ku-band NTN is shown as below:

### Sub-topic 2-1: VSAT antenna pattern Agree for VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel. Agree to use 38.921 unless there is a need to use sub array then 38.803 The following parameters are agreed: Table 2-1 VSAT phased array antenna characteristics Ku band VSAT phased array antenna. 1; Ku band VSAT phased array antenna Characteristics. 1.1; Antenna pattern; Table 2-2 and 2-3 (TR 38.921). 1.2; Element gain (dBi) (Note 2); 0 or 3.5 ?. 1.3; Y axis / Z axis element 3 dB beam width of single element (degree); 90° for Y axis 90° for Z axis. 1.4; Y axis / Z axis element front-to-back ratio (dB); 30 dB. 1.5; Antenna polarization; Circular（RHCP or LHCP）. 1.6; Antenna array configuration (Y axis × Z axis)(Note 4); 45x45 elements. 1.7; Number of supported polarizations, P; 1. 1.8; Y axis / Z axis radiating element spacing; 0.5 lambda. 1.9; Array Ohmic loss (dB) (Note 2); 2. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); [4.44 dBm] TBA. 1.11; maximum coverage angle in the horizontal plane (degrees); 0~360 degrees. 1.12; vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees. 1.13; normal direction; Toward X+ axis. Note 1: Void Note 2: The element gain in row 1.2 includes the loss given in row 1.9. Note 3: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 4: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Agree 3.5 dB element gain

As above, the Conducted power (before Ohmic loss) per antenna element is [4.44 dBm] TBA, since 40dBm (10W) VSAT max Tx power was agreed in WF in RAN4#112bis [2], and a 45 x 45 = 2025 element antenna[1] was agreed for VSAT phased array (flat-panel) antenna. Then, the conducted power (before Ohmic loss) per antenna element (dBm) is 10*log10(10*1000)-10*log10(45*45) = 6.9357dBm. We propose the conducted power (before Ohmic loss) per antenna element (dBm) = 6.94dBm.

Proposal 1: Adopt 6.94dBm for conducted power (before Ohmic loss) per antenna element (dBm) for VSAT phased array antenna.

VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna

As per the WF [1], the agreement concerning VSAT Max Tx antenna gain for 100cm aperture parabolic antenna for Ku-band NTN is shown as below:

### Sub-topic 2-4: VSAT Max Tx antenna gain Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type. The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

As above, the 100 cm aperture (41.3dBi) parabolic antenna was agreed for Ku-band NTN coexistence, the VSAT Max TX antenna gain is 41.3 dBi for 14GHz UL, but the VSAT Max RX antenna gain for 11GHz DL is missing. For parabolic antenna, one 100 cm aperture parabolic antenna can support both 14GHz UL and 11GHz DL at the same time, but the antenna gain for DL and UL is different.

3dB beamwidth is dependent on frequency(λ) and antenna diameter (D) as following equation.

2θ-3dB=2×Arcsin1.616×λπD                                                                                                                       (Eq.1)

So, using Eq.1, for 14GHz,  2θ-3dB14GHz=1.2631 degree, and for 11GHz, 2θ-3dB11GHz=1.6076 degree.

We can use 41.3 dBi for 14GHz and 1.2631degree 3dB beamwidth as reference, the antenna gain for 11GHz can be calculated by following Eq.2.

G11GHz=G14GHz-20log102θ-3dB11GHz2θ-3dB14GHz=41.3-20*log101.60761.2631=39.2                                            (Eq.2)

So, we propose that VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna is 39.2dBi.

Proposal 2: Adopt 39.2dBi for VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna.

- Conclusion

This contribution provides analysis on coexistence assumptions for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Adopt 6.94dBm for conducted power (before Ohmic loss) per antenna element (dBm) for VSAT phased array antenna.

Proposal 2: Adopt 39.2dBi for VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna.

- References

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#113

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis

- R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112bis

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#106

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104