3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500992

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Further discussion on BS demodulation performance for MMSE-IRC

Agenda item:	7.18.3

Document for:	Approval

- Introduction

In RAN4#113 meeting, a WF [1] on NR demodulation performance: Phase 5 was agreed, and the remaining issues on BS demodulation performance for MMSE-IRC in WF require further discussion.

In this contribution, we provide our views on BS demodulation performance for MMSE-IRC.

- Discussion

2.1 Test parameters

Channel bandwidth

As per the WF [1], the remaining open issue concerning channel bandwidth is shown as below:

Channel bandwidth - Proposals on the CHBW for requirement definition for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: - For 15kHz SCS, cover 5MHz, 10MHz - For 30kHz SCS, cover 10MHz, 40MHz - Option 3: minimum CBW - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS - Proposals for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz

It is sufficient to define minimum CBW for MMSE-IRC. We prefer the Option 3(minimum CBW) for CP-OFDM and Option1 for DFT-s-OFDM.

Proposal 1: Adopt Option 3 for CBW for CP-OFDM as below:

  - Option 3: minimum CBW

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Proposal 2: Adopt Option 1 for CBW for DFT-s-OFDM as below:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

MCS

As per the WF [1], the remaining open issue concerning MCS is shown as below:

MCS - Consider the following options for performance evaluation and as candidates for further down-selection: o For 2Rx:  Option 1: MCS 2  Option 2: MCS 16 o For 4Rx:  Option 1: MCS 16 o For 8Rx:  Option 1: MCS 20  Option 2: MCS 16 - Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively. - Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration.

We prefer MCS 2 for 2Rx, MCS 16 for 4Rx, and MCS 20 for 8Rx, meanwhile the same FRCs for them can be reused.

Proposal 3: Define MCS 2 for 2Rx, MCS 16 for 4Rx, and MCS 20 for 8Rx.

Propagation condition

As per the WF [1], the remaining open issue concerning propagation condition is shown as below:

Propagation condition – For serving cell, consider the following options for performance evaluation and as candidates for further down-selection: o Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet o Option 2: Consider different channel for different MCS  Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 – For the interference cell, use TDLC300-100.

We can refer to propagation condition for normal PUSCH demodulation, the TDLB100-400 Low, TDLC300-100 Low, TDLA30-10 Low, and TDLA30-10 Low are defined for QPSK, 16QAM, 64QAM and 256QAM respectively. The propagation condition choosing is dependent on MCS choosing, so for QPSK, 16QAM, 64QAM are chosen, the TDLB100-400 Low, TDLC300-100 Low, and TDLA30-10 Low are needed.  For Option 1, the TDLA30-10 HetNet and TDLB100-400 for HomNet are part of Option 2A, but MCS has not been considered. We prefer Option 2A.

Proposal 4: Adopt Option 2A for propagation condition for serving cell as below:

o    Option 2: Consider different channel for different MCS

  Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10

Waveform

As per the WF [1], the remaining open issue concerning waveform is shown as below:

Waveform – whether to cover DFT-s-OFDM - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

Since the CP-OFDM and DFT-s-OFDM were defined for NR PUSCH normal demodulation, so it is reasonable to define NR PUSCH MMSE-IRC demodulation for CP-OFDM and DFT-s-OFDM, so we prefer to define both CP-OFDM and DFT-s-OFDM waveform. In order to reduce workload, we can accept not to define DFT-s-OFDM demodulation for MMSE-IRC.

Proposal 5: Adopt Option 1 for waveform for MMSE-IRC as below:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

DMRS configuration

As per the WF [1], the remaining open issue concerning DMRS configuration is shown as below:

DMRS configuration - Cover DMRS configuration type 1. FFS whether to include DMRS configuration type 2. - Use DMRS port #0 for both serving and interfering PUSCH - On DMRS sequence generation, use different DMRS sequence for each gNB: - For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively - FFS DMRS sequence generation configuration for DFT-s-DFDM

We think that only DMRS configuration type 1 is sufficient because the density of DMRS configuration type 1 is greater than that of DMRS configuration type 2. So we prefer to not define DMRS configuration type 2.

Proposal 6: Not define DMRS configuration type 2.

According to sub-clause 6.4.1.1.1 of TS 38.211, the group hopping and sequence hopping are only supported by DFT-s-OFDM, the “NID0=0, group hopping and sequence hopping are disabled” are defined for DFT-s-OFM PUSCH normal demodulation, so it is reasonable to define DMRS sequence generation configuration as below:

For DFT-s-OFDM, group hopping and sequence hopping are disabled for cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively.

Proposal 7:  Define DMRS sequence generation configuration for DFT-s-OFM as below:

For DFT-s-OFDM, group hopping and sequence hopping are disabled for cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively.

Test requirement definition

As per the WF [1], the remaining open issue concerning test requirement definition is shown as below:

Test requirement definition - Candidate options: - Option 1: FFS whether SINR based definition is needed - Option 2: Use the SNR based requirement definition

For INR-based modelling in TS 38.101-4, the INR is defined as a test parameter, INR has shown the relationship between interference and noise, so SNR as test requirement definition is sufficient, theoretically SINR-based requirement and SNR-based requirement are equivalent.

SINR based requirement was used for DIP-based modelling for LTE PUSCH and PDSCH MMSE-IRC demodulation requirement in TS 36.104 and 36.101. In last meeting, RAN4 agreed that INR-based modelling for NR PDSCH MMSE-IRC demodulation requirement in TS 38.101-4 will be used for NR PUSCH MMSE-IRC demodulation requirement, in  TS 38.101-4, NR PDSCH MMSE-IRC demodulation requirement was defined as SNR based requirement, so it is reasonable to use SNR based requirement for NR PUSCH MMSE-IRC demodulation requirement. So we prefer Option 2: SNR based requirement definition.

Proposal 8: Adopt Option 2 for test requirement definition as below:

  - Option 2: Use the SNR based requirement definition

- Conclusion

This contribution provides analysis on NR BS MMSE-IRC demodulation. The following proposals and observations are concluded as follows:

Proposal 1: Adopt Option 3 for CBW for CP-OFDM as below:

  - Option 3: minimum CBW

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Proposal 2: Adopt Option 1 for CBW for DFT-s-OFDM as below:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Proposal 3: Define MCS 2 for 2Rx, MCS 16 for 4Rx, and MCS 20 for 8Rx.

Proposal 4: Adopt Option 2A for propagation condition for serving cell as below:

o    Option 2: Consider different channel for different MCS

  Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10

Proposal 5: Adopt Option 1 for waveform for MMSE-IRC as below:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

Proposal 6: Not define DMRS configuration type 2.

Proposal 7:  Define DMRS sequence generation configuration for DFT-s-OFM as below:

For DFT-s-OFDM, group hopping and sequence hopping are disabled for cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively.

Proposal 8: Adopt Option 2 for test requirement definition as below:

  - Option 2: Use the SNR based requirement definition

- References

- R4-2419780, Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS, China Telecom, MediaTek, RAN4#113

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telcom, RAN4#112bis

- R4-2413570, WF on NR demodulation performance: Phase 5, China Telcom, RAN4#112

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements, Moderator (vivo), RAN#103

- R4-2411118, Discussion on BS demodulation performance for MMSE-IRC, CATT, RAN4#112