3GPP TSG-RAN WG4 Meeting #114	                                                                                            R4-2500999

Athens, GR, 17 Feb –21 Feb, 2025

Source: 	CATT

Title: 	Discussion on Ambient IoT device 1 RF requirements

Agenda item:	7.24.2.2

Document for:	Approval

- Introduction

In RAN#106, a new WI “Solutions for Ambient IoT (Internet of Things) in NR” [1] is approved. This WI is based on the SI study in RAN WGs. The scope of the WI is limited that only device 1 is studied.

Meanwhile, in RAN4 #113 meeting, a WF [2] on RF requirements for A-IoT was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

This contribution provides our analysis for device 1 RF requirements based on the discussion in SI phase [2].

- Discussion

  - Tx requirements

The open issues on RF requirements for A-IoT were discussed during the previous meetings, as shown below:

### Issue 2-4: TX Agreement: - For R2D transmission requirements, LP-WUS requirements can be considered as reference. - Whether requirements for A-IoT device RX RF potentially needed or not is summarized in the following table: RF Requirement for A-IoT BS- TX requirements; Potentially needed or not. Base station output power; YES. Output power dynamic; RE power control dynamic range; TBD in WI phase. Total power dynamic range; TBD in WI phase. Power boosting; NO. Some requirements like Transmit ON/OFF power; Transmitter OFF power; TBD in WI phase. Transmitter transient period; TBD in WI phase. Transmit signal quality; Frequency error; YES. Some requirements like EVM (Modulation quality); YES. TAE; NA. Unwanted emissions; Occupied bandwidth; YES. ACLR; YES. IBE; NO. Operating band unwanted emissions; YES. Transmitter spurious emissions; YES. Transmitter intermodulation; TBD in WI phase. ### Issue 2-5: RX Agreement: - Capture something like the following in the general section for BS: For A-IOT BS receiver requirements, some requirements can be different for inside and outside topology respectively - Whether requirements potentially needed or not is summarized in the following table: A-IoT BS- RX RF Requirement; Potentially needed or not. Reference sensitivity level; YES. Dynamic range; TBD in WI phase. Adjacent Channel Selectivity; YES. In-band blocking; general blocking; YES. narrowband blocking; YES. Out-of-band blocking; YES. Receiver spurious emission; YES. Receiver intermodulation; General intermodulation; Wait for RAN1 conclusion.. Narrowband intermodulation; YES. In-channel selectivity; TBD in WI phase.

According to the WF in SI phase, RAN4 has agreed that the following Tx requirements will be defined for Device 1:

- backscattering power or power backscattering loss

- Occupied bandwidth

However, some of the Tx requirements still need more discussion:

- Modulation quality requirements

- Unwanted emissions or SEM and spurious emissions

For modulation quality requirements, according to RAN1#118 meeting agreement, the OOK and BPSK for baseband modulation are the potential modulation scheme for D2R devices. Since the modulation quality requirements are mainly used to limit the amplitude and phase downgrade of a signal, we believed that the modulation quality requirements are still required for Device 1.

However, the modulation quality requirements for Device 1 might be slightly different from NR, depending on the modulation scheme. For OOK modulation, EVM measurements can only be conducted when the transmitter is switched ON (no signal is transmitted when the transmitter is switched OFF), so how to apply the EVM requirements to Device 1 may still need to be investigated. Meanwhile, the above problems do not exist for BPSK modulation, so the current NR EVM requirements can be considered as a good starting point.

Therefore, we believe that Device 1 could introduce the modulation quality requirements, and further discussion is needed as to how to define the modulation quality requirements.

Proposal 1: Modulation quality requirement could be defined for Device 1.

For Unwanted emissions or SEM and spurious emissions requirements, since Device 1 is considered as a passive device, the transmitted signal of a passive device is generated by reflection through impedance matching, and the very simple implementation makes its performance also less varied and relatively easy to control. Due to the relatively simple structure, the unwanted emissions of Device 1 may not be too complex. In addition, considering that Device 1 has a structure similar to RFID tags, we believed that the ETSI RFID study could be considered as a good reference and starting point.

Referring to the ETSI EN 302 208, the emissions requirements are captured below (aka spectrum mask):

The ETSI RFID emission requirements includes spectrum masks (indicated the emission range of SEM) and spurious emissions, hence we believed that the emission requirements for Device 1 could consider it as a reference as well.

Proposal 2: Emission requirements should be defined for Device 1. Meanwhile, the requirement should cover the emission range of SEM and spurious emissions.

  - Rx requirements

According to the WF in SI phase, RAN4 has agreed that the following Rx requirements will be defined for Device 1:

- Reference sensitivity

However, some of the Rx requirements still need more discussion:

- Maximum input power

- Interference rejection

For maximum input power, this requirement might be related to the maximum coupling loss (MCL). The maximum input power level for device could be calculated by maximum A-IoT BS Tx power minus the MCL, so if the Maximum Input Power requirements need to be discussed, the MCL needs to be determined first.

We noticed that the MCL has been discussed during the coexistence study of ambient IoT SI phase and was captured in TR 38.769. Therefore, we believed that the MCL value in TR 38.769 could be considered as a good reference and starting point.

Proposal 3: The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered.

The interference rejection requirement is originally proposed to consider the interference from NR system, and the concept of interference rejection requirements are defined for RFID tags. However, the R19 WI only includes the D1T1-B scenario, which means that there is no NR system deployed close to ambient IoT devices (such as no ACS requirements for Device 1). Therefore, we think interference rejection requirement is not needed for R19 WI.

Proposal 4: Interference rejection requirement is not needed for the D1T1-B deployment scenario.

- Conclusion

This contribution provides our initial views on A-IoT Device 1 requirements. The following proposals are concluded as follows:

Proposal 1: Modulation quality requirement could be defined for Device 1.

Proposal 2: Emission requirements should be defined for Device 1. Meanwhile, the requirement should cover the emission range of SEM and spurious emissions.

Proposal 3: The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered.

Proposal 4: Interference rejection requirement is not needed for the D1T1-B deployment scenario.

- References

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106

[2] TR 38.769

[3] R4-2420470, “WF on RF requirements for A-IoT”, RAN4#113, Huawei