3GPP TSG-RAN WG4 Meeting #114                                                                                             R4-2501008

Athens, GR, Feb. 17 – Feb. 21, 2025

Source: 	CATT

Title: 	Discussion on UE Tx requirement for NTN support HPUE

Agenda item:	7.10.2.2.1

Document for:	Approval

- Introduction

In RAN4 #113 meeting, a WF [1] on UE Tx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE Tx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Tx requirements for NTN support HPUE was concluded during the previous meetings, as shown below:

# Open issues Issue 2-5: NR-NTN PC2 UE Agreement in #112bis meeting: Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE Agreement: - Follow previous agreement Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - ..... - Proposals - Option 1: In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices. (P1a/R4-2417747) - Option 2: For handheld UE, 1Tx and 2Tx are covered with existing TN 26dBm and 23dBm PAs respectively. (P1/R4-2419521) Issue 2-6: NR-NTN PC1.5 UE Agreement in #112bis meeting: Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE Agreement: - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not. - Proposals PC1.5 UE; 1Tx; 2Tx. Handheld; To specify: Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015, P1/R4-2419521 FFS:; To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532 Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015 FFS:. Non-handheld; To specify: Not to specify: P1/R4-2419668, P3/R4-2419072 FFS: P5/R4-2418200, P1/R4-2419521; To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532, P2/R4-2419521 Not to specify: P1/R4-2419668, P3/R4-2419072, P1b/R4-2417747 FFS:. Issue 2-7: NR-NTN PC1 UE Agreement in #112bis meeting: Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.. - Proposals PC1; 1Tx; 2Tx; 3Tx; 4Tx. Handheld; To specify: Not to specify: P1/R4-2419521 FFS:; To specify: Not to specify: P5/R4-2418200, P1/R4-2419521 FFS:; To specify: P1/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200; To specify: Not to specify: FFS:. Non-handheld Only non-handheld: P2/R4-2419015, P1/R4-2419072; To specify: P1a/R4-2417747, P2/R4-2419521 Not to specify: FFS:; To specify: Not to specify: P5/R4-2418200 FFS:; To specify: P2/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200; To specify: P4/R4-2419072 Not to specify: FFS:.

For TN HPUE that support PC 2, both handheld UE and non-handheld UE have been deployed and can be considered as commercially available, with 1Tx or 2Tx architecture. Therefore, for NTN HPUE we believe that the same architecture should be feasible as well.

Proposal 1: If NR NTN PC2 HPUE with 1Tx architecture is commercially available and widely supported, then both the 1Tx and 2Tx architectures can be considered for handheld and non-handheld NR NTN PC2 HPUE.

For TN HPUE that support higher power levels such as PC 1.5, only non-handheld UEs will be deployed. However, similar to PC 2 handheld HPUE, the 2Tx PC 1.5 handheld UE is also possible since the state-of-the-art RF front-end implementations already made some progress (such as deployed by using 2 Tx RF components with 26 dBm capability). For PC 1.5 non-handheld UE, the implementation could use more Tx chains. However, as mentioned above, there are no significant challenges to adopting 2 Tx implementation for non-handheld PC 1.5 HPUE. Therefore, 2 Tx architecture could be considered for both handheld and non-handheld NR NTN HPUE.

Proposal 2: If NR NTN PC2 HPUE with 1Tx architecture is commercially available and widely supported, then for NR NTN PC 1.5 HPUE, the 2 Tx architecture could be considered for both handheld and non-handheld.

- Conclusion

This contribution provides our views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: If NR NTN PC2 HPUE with 1Tx architecture is commercially available and widely supported, then both the 1Tx and 2Tx architectures can be considered for handheld and non-handheld NR NTN PC2 HPUE.

Proposal 2: If NR NTN PC2 HPUE with 1Tx architecture is commercially available and widely supported, then for NR NTN PC 1.5 HPUE, the 2 Tx architecture could be considered for both handheld and non-handheld.

- References

- R4-2420403, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #113