3GPP TSG-RAN WG4 Meeting #114	R4-2501023

# Athens , GR, 17th – 21st Feb, 2025

Title: 	Discussion on (e)RedCap RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.29.4.1

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part1 was approved and some agreements were reached, but there still are a few issues left [1].

This document will discuss (e)RedCap RRM requirements for Rel-19 NTN phase 3 and give our understanding and proposals.

- Discussion

  - General consideration

- 2.1.1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Further discussion on cell reselection requirements

In RAN4 #112-bis meeting, RAN4 further discussed cell reselection requirements for (e)RedCap UE with FR1-NTN bands and reached the following agreements, but there was still FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement. In RAN4#113 meeting, the Way Forward is also copied as below [1]:

RAN4#112-bis: <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement. RAN4#113: <Way Forward>: - Proposal 1 (CATT, Xiaomi, LG, Samsung CMCC, Apple, Ericsson, HW, ZTE): - RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN. - Proposal 2 (vivo): - RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

In our view, since the measurement requirements for inter-RAT (NR NTN to E-UTRA TN) measurement has been defined in NTN for normal UE, it is reasonable to define them for (e)RedCap UE with FR1-NTN bands, and there is no much differences between normal UE and RedCap UE regarding this aspect.

Proposal 1: The measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement can be defined for (e)RedCap UE with FR1-NTN bands.

The Requirements to be further determined

In RAN4#112-bis meeting, RAN4 agreed to define NR Rel-17 and Rel-18 Conditional Handover requirements, RACH-less Handover requirements and  SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state, some FFS were still left. The Way Forward for SDT and MDT in the last meeting is copied as follows [1]:

<Way Forward>: - For SDT requirements in RRC_INACTIVE state, - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements. - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission. - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable) - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be: Measurement; FR1. RSRP1; (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)). RSRP2; (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2. Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.. - For MDT requirements in RRC_IDLE and RRC_INACTIVE state - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK): - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system. - Proposal 1a (CATT): Deprioritize to define. - Proposal 2 (CMCC): - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec. - Proposal 3 (ZTE): - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

SDT requirements

Regarding CG-SDT requirements, TA validation requirements has been introduced for RedCap UE with TN and the TA validation was based on the RSRP change criterion. The RSRP change was used to reflect the change of distance between UE and BS. When the change of distance exceeds the threshold, TA will be considered invalid.

But for NTN, the change of distance has already been compensated by UE in UE specific TA, so RSRP change is not a good indicator for TA verification. Therefore, we support to remove the TA validation based on the RSRP change criterion and the requirements can refer to the PUR requirements for IoT for NTN.

Observation 1: TA validation in CG-SDT requirements has been introduced for RedCap UE with TN and the TA validation was based on the RSRP change criterion.

Observation 2: In NTN scenario, the change of distance has already been compensated by UE in UE specific TA, so RSRP change is not a good indicator for TA verification.

Proposal 2: For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN.

MDT requirements

In our view, the purpose of MDT is for network optimization. Although MDT requirements are important for NTN scenario as proponent said, if drive tests are needed in NTN scenarios, it is more resonable to use normal UE or even dedicated terminals for this task and we don’t think a RedCap UE will be used for drive tests unless in private network. From the perspective of practical use and scenario, the necessity of defining MDT requirements for (e)RedCap UE with FR1-NTN is not strong, and MDT requirements for normal UE is enough. Therefore, we support to deprioritize to define MDT requirements for (e)RedCap UE with FR1-NTN bands

Observation 3: The purpose of MDT is for network optimization.

Observation 4: Although MDT requirements are important for NTN scenario, if drive tests are needed in NTN scenarios, it is more resonable to use normal UE or even dedicated terminals for this task and a RedCap UE will not be used for drive tests unless in private network.

Proposal 3: Deprioritize to define MDT requirements for (e)RedCap UE with FR1-NTN bands.

- 2.1.2 Features to be considered for (e)RedCap UE with FR1-NTN

Relaxed measurements

For R17 relaxed measurements, they were introduced for R17 RedCap UEs in TN network, including stationary/ low mobility/not-at-cell-edge criteria and their combinations in RRC_IDLE/RRC_INACTIVE mode. In current NTN requirements, only R16 TN relaxed measurements were introduced in RRC_IDLE/RRC_INACTIVE mode, whether to consider these R16/R17 relaxed measurements for (e)RedCap UEs with FR1-NTN bands should be discussed.

In the last meeting, RAN4 discussed relaxed measurements related issues for RedCap UEs with FR1-NTN, and the agreement is copied as below [2]:

< Agreement >: - For supported types of relaxations for (e)RedCap UE with FR1-NTN, - Only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection. - RAN4 to further discuss the thresholds and specific relaxation factor.

For supported types of relaxations for (e)RedCap UE with FR1-NTN, RAN4 has agreed only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection due to near-far effect in NTN, which mainly emphasizes the framework of criterion including low mobility criterion and not-at-cell edge criterion. For the thresholds in supported relaxations criterion and specific value of measurement relaxation factor K3, we support to reuse the parameters used in R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation).

Proposal 4: For the thresholds in supported relaxations criterion and specific value of measurement relaxation factor K3, we support to reuse the parameters used in R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation).

eDRX configuration

In RAN4#112-bis meeting, RAN4 discussed the eDRX related issues and reached agreements for GSO. In the last meeting, RAN4 further discussed eDRX for LEO, but there was some FFS for quasi-earth fixed LEO scenario. The previous agreements are copied as below [2]:

RAN4#112-bis: <Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO. RAN4#113: < Agreement >: - For earth-moving LEO scenario: - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario. - For quasi-earth fixed LEO scenario: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - Requirements are applicable for 2.56s and 5.12s eDRX cycle - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

For quasi-earth fixed cell LEO deployment, as far as we know, the serving time of satellite is about several minutes (about 300s ~ 500s). According to Table 4.2B.2.3-2 in TS 38.133 [4], for cell Re-selection in IDLE mode, the existing Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap defined for RedCap UE configured with eDRX_IDLE cycle are copied as follows:

Table 4.2B.2.4-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table corresponds to the DRX cycles within PTWs, when PTW is configured. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |

Technically speaking, according to the values of Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap defined for RedCap UE, the serving time of a satellite can cover the time for UE to perform cell reselection for up to 10.24s eDRX cycle. However, for inter-frequency NR cells, the number of NR inter-frequency carriers also needed to be considered, so some margin of measurement time is needed. Therefore, we beilieve that at least 2.56s eDRX cycle length can be supported and 10.24s eDRX cycle length will be excluded. For 5.12s eDRX cycle length, it can be used in some cases such as intra-frequency measurement, so it can be introduced and whether it is configured depends on the network.

Observation 5:For quasi-earth fixed cell LEO deployment, for inter-frequency NR cells measurement, the number of NR inter-frequency carriers also needed to be considered, so some margin of time is needed.

Proposal 5: For quasi-earth fixed cell LEO deployment,

- 2.56s eDRX cycle length can be supported.

- 10.24s eDRX cycle length will be excluded.

- 5.12s eDRX cycle length can be used in some cases and whether it is configured depends on NW.

The number of searchers and CSSF

In the last meeting, the WF of the number of searchers and CSSF is copied as follows [2]:

<Way Forward>: - Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Regarding the searcher assumption and CSSF for (e)RedCap UEs in TN network, 1Rx and 2Rx (e)RedCap UEs use only a single cell search and measurement engine, with no CA in mind. The single searcher was shared by all of intra- and inter-frequency layers. For CSSF, its values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i. The legacy design of CSSFwithin gap were reused for (e)RedCap UEs in TN, and CSSFoutside_gap,I were defined as follows:

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap_RedCap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, will be specified as follows: CSSFoutside_gap,i =1, if only one measurement object is configured to be measured outside of MG for RedCap. CSSFoutside_gap,i =2 for intra-frequency measurement, and CSSFoutside_gap,i = 2*Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. If intra-frequency measurement is with measurement gap, CSSFoutside_gap,i = Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG.

For NTN requirements, the legacy searcher assumption and CSSF in TN were reused for normal UE. In our view, a single searcher and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN since they are related to the UE capability to process and depend on the type of UEs.

Observation 6: The legacy searcher assumption and CSSF are related to the capability of the UE to process and depend on the type of UEs.

Proposal 6: Single searcher assumption and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN.

FH in PRS measurement for NW verified location requirements

In the last meeting, the WF of FH in PRS measurement for NW verified location requirements is copied as follows [2]:

<Way Forward>: - Proposal 1 (HW): RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands. - Proposal 2 (Ericsson, OPPO): Not support FH in PRS measurement for RedCap for NTN.

In our view, using frequency hopping can effectively combat system interference in traditional larger bandwidths and FH was introduced in PRS measurement for RedCap in TN.

However, the configurable bandwidth of NTN in FR1 is only 20MHz, so the gain of FH is very small, so we support not to introduce FH in PRS measurement for (e)RedCap UE with FR1-NTN.

Observation 7: The configurable bandwidth of NTN in FR1 is only 20MHz, so the gain of FH in PRS measurement is very small.

Proposal 7: Not support FH in PRS measurement for (e)RedCap UE with FR1-NTN.

NCD-SSB

In RAN4#112 meeting, RAN4 has already reached the following agreements [1]:

<Agreement> - For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

In our view, even though NCD-SSB can bring benefits to RedCap UE in NTN, but considering that the NTN bandwidth is too small since the configurable bandwidth of NTN is only 20MHz, so the benefit of NCD-SSB is not obvious, so we support to still keep the above agreements.

Another point we suggest to clarifying is the meaning of ‘no need to define NCD-SSB specific measurement requirements’ in the agreement. To facilitate the writing of CR, RAN4 to only use ‘CD-SSB’ to define the requirements or use ‘SSB’ should be discussed. For this issue, the WF in the last meeting is copied as follows [3]:

<Way Forward>: - For the terminology of SSB used in the spec, - Proposal 1 (CATT, Ericsson, HW, CMCC): Use the terminology of SSB without differentiating CD-SSB/NCD-SSB. - For whether to add additional requirement applicability about CD-SSB/NCD-SSB, - Option 1 (CATT, Ericsson, HW): Add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be: - The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB. - Option 2 (CMCC): No need to add additional requirement applicability about CD-SSB/NCD-SSB.

For the terminology of SSB used in the spec, we support to use the terminology of SSB without differentiating CD-SSB/NCD-SSB in the spec, which seems to be uncontroversial.

For whether to add additional requirement applicability about CD-SSB/NCD-SSB, we think that the types of SSB need to be clarified. Considering that the network configuration can’t be restricted, so it is better to add a clarification of applicability in a separate clause to reflect the related agreement on this aspect (such as clause 3 in TS 38.133[4]), and the wording can be: the requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB. If there is no clarification on the types of SSB, the applicability of the requirements is not very clear since we have no requirements for NCD-SSB.

Proposal 8: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

- The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

  - Reduction in the number of UE Rx branches

- 2.2.1 Mobility requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Satellite switching with re-sync

In the last meeting, RAN4 has agreed the requirements for HO and CHO for 1Rx/2Rx (e)RedCap UEs with FR1-NTN, and also discussed satellite switching with re-sync requirements, there was still some square brackets and FFS. The agreement in the last meeting is copied as below [1]:

< Agreement >: - For Satellite switching with re-sync requirements, - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy Satellite switching with re-sync requirements for normal UEs (including both hard satellite switch with re-sync and soft satellite switch with re-sync). - For hard Satellite switching with re-sync requirements, for 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tsearch by extending one more [SSB or SMTC] periodicity on the top of Tfirst_SSB: - Tsearch = Tfirst_SSB + [TSSB or TRS] ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite - FFS: whether to define soft Satellite switching with re-sync requirements for 1Rx (e)RedCap UEs with FR1-NTN bands.

For hard Satellite switching with re-sync requirements, RAN4 has agreed to extend one more [SSB or SMTC] periodicity on the top of Tfirst_SSB for Satellite switching with re-sync for 1Rx (e)RedCap UEs with FR1-NTN bands. Since Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, and the location of it is determined by the periodicity and location of SSB of the source satellite, relaxing by 1 sample cannot be directly multiplied by two, and we support Tsearch = Tfirst_SSB + TSSB ms for 1 Rx antenna, where TSSB is the periodicity of the SSB of the target NR SAN cell.

Proposal 9: For hard Satellite switching with re-sync requirements, for 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB:

- Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the target NR SAN cell.

- 2.2.2 Measurement Procedure

L1/L3 Measurement requirements

In the last meeting, the Way Forward of L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN is copied as follows [1]:

<Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, HW): RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level. - Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE, HW): The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter. - Not to extend the lower boundary. - Proposal 1a (QC): Extended delays for PSS/SSS detection. - Proposal 1b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 2 (CATT, [Samsung?], CMCC, HW): The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter. - Extend the lower boundary from ‘120 ms’ to ‘160ms’. - Proposal 2a (QC): Extended delays for SSB index identification (PBCH decoding). - Proposal 2b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 3 (CATT, CMCC, HW): The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples for T SSB_measurement_period_inter. - Proposal 4 (CATT, CMCC, ZTE, HW): The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - Proposal 5 (CATT, CMCC, HW): The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

For the measurement requirements, the relaxation on the time period for PSS/SSS detection, SSB index identification (PBCH decoding) and measurement period have been considered for 1Rx (e)RedCap UEs compared to those defined for 2Rx (e)RedCap UEs in TN network, According to TS 38.133 [4], the specific relaxation principles for 1Rx (e)RedCap UEs compared to 2Rx (e)RedCap UEs in TN are listed as following:

|  | Intra-f without measurement gaps | Intra-f with measurement gaps | Inter-f without measurement gaps | Inter-f with measurement gaps |
| --- | --- | --- | --- | --- |
| Time period for PSS/SSS detection | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘8’ to ‘10’ - Not to extend the lower boundary ‘600ms’ |
| Time period for SSB time index detection | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ |
| Measurement period | No relaxation | No relaxation | No relaxation | Relax for 1Rx - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples. |

According to the above, we believe that the same relaxation can be reused as baseline for 1Rx (e)RedCap UEs with FR1-NTN.

Proposal 10: For 2Rx (e)RedCap UEs with FR1-NTN bands,

- RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level.

- Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised.

Proposal 11: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(7 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

| Condition NOTE1 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 10 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(10 *1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 10 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 12: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(160ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(160ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(160ms, 6 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(160ms, Ceil(6 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 13: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

| Condition NOTE1 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400ms, 8 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(400ms, ceil(1.5 8) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 14: The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

For SSB and CSI-RS based L1-RSRP measurement requirements, there were no extensions on the measurement period TL1-RSRP_Measurement_Period_SSB_RedCap and TL1-RSRP_Measurement_Period_CSI-RS_RedCap for FR1 for 1 Rx RedCap UE in TN network [4]. Therefore, we believe that RAN4 doesn’t need to extend the measurement period TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS for 1Rx (e)RedCap UEs with FR1-NTN bands, that is, reuse those defined for 2Rx.

Proposal 15: The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

- 2.2.3 Measurement accuracy requirements

L1/L3 Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

In the last meeting, the Way Forward of L1/L3 Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN is copied as follows [1]:

<Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE): RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Xiaomi, Samsung, [QC?], CMCC, ZTE, [OPPO?], HW): RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB.

The specific measurement accuracy requirements were defined in clauses 10.1A and 10.2A in TS 38.133[4] for (e)RedCap UEs in TN, which include NR measurements for RedCap and E-UTRAN measurements for RedCap. For 2Rx (e)RedCap UEs in TN network, the legacy accuracy requirements were reused. For 1Rx (e)RedCap UEs in TN network, 1 dB relaxation on SS-RSRP/ SS-RSRQ/ SS-SINR accuracy requirements was considered for L3 intra- and inter-frequency measurement in FR1, and 3dB relaxation on L1-RSRP accuracy requirements was considered for SSB based and CSI-RS based L1 measurement in FR1.

In current NTN requirements, the specific measurement accuracy requirements were also defined with the suffix C in TS 38.133[4]. SS-RSRP/ SS-RSRQ/ SS-SINR accuracy requirements were considered for L3 intra- and inter-frequency measurement in FR1, and SSB based L1-RSRP accuracy requirements was considered in FR1. All of the values of accuracy are the same as the legacy.

Therefore, we believe that RAN4 can reuse the accuracy requirements defined in NTN for 2Rx (e)RedCap UEs with FR1-NTN bands, and consider the same relaxation as TN for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Proposal 16:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

- The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

For SSB_RP and SSB Ês/Iot, we believe that RAN4 can reuse the conditions defined in NTN for a corresponding Band. For 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs, the values for SSB_RP and SSB Ês/Iot are the same.

Proposal 17: RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

  - The impact of HD-FDD

RAN4 has agreed that the TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point [3]. According to the RedCap WI, the following  RRM requirements are affected by HD-FDD for (e)RedCap UE in TN network, so the same requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN.

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 18: At least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

For the impact on NR Handover due to HD-FDD, we think that the legacy applicability can be reused for RACH-based/RACH-less HO, Time/location-based CHO and Satellite switching with re-synchronization.

Proposal 19: For RACH-based/RACH-less HO, Time/location-based CHO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

- For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

Since RAN1 is still discussing the enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, which may impact scheduling availability in RRM requirement, and the current progress is listed for information, the HD-FDD related enhancement on legacy requirement applicability should wait for more RAN1 progress.

RAN1#116: Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching - At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs - Error cases in case 3 and case 4 - SIB19 reception collides with UL transmission - Slot counting for UL repetition transmission colliding with SSB reception - Invalid symbol determination for PUSCH repetition type B - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling - CPU occupation due to omitted DL reception or UL transmission Note: Both GSO and Non-GSO should be considered. RAN1#117: Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN. RAN1#118: Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied RAN1#118bis: Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS RAN1#119: Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL

Observation 8: For collision case 3, RAN1 agreed that it left to UE implementation whether to prioritize UL or prioritize DL with the some constraint.

Observation 9: There was still FFS on priority rule for collision case 4 in RAN1.

Proposal 20: Whether the legacy HD-FDD related scheduling availability in RRM requirement can be reused in NTN directly should wait for more RAN1 progress.

  - Others

Specification structure

To facilitate the writing of CR, the specification structure also should be discussed. In the last meeting, the candidate proposals about specification structure are copied as below [3]:

<Way Forward>: - Proposal 1 (CATT, Ericsson, HW): - Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. - Utilize the reference method for the requirements that can be reused. - Proposal 2 (Nokia): - Adopt RedCap subclauses under NTN sections (under Suffix C).

We support to define the requirements for (e)RedCap UEs with FR1-NTN in the new sections of section number with new suffix X in the specification, like current way used for NTN requirements and (e)RedCap requirements. It is more clear and simple way. In addition, RAN4 will utilize the reference method for the requirements that can be reused, thereby reducing the redundancy of the spec to a certain extent.

For the method of introducing new sub clauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes [4], so a new suffix after NTN suffix may need to be added, and we’re not sure it is a permitted and general methods.

Observation 10: For introducing new subclauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes, so a new suffix after NTN suffix may need to be added.

Proposal 21: To use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

UE capabilities for (e)RedCap UE with FR1-NTN bands

In the last meeting, the WF on UE capabilities for (e)RedCap UE with FR1-NTN bands is copied as below [1]:

<Way Forward>: - Proposal 1 (Nokia): RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17) - Proposal 1a (CMCC): Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 1b (Ericsson): RAN4 to define a new capability declaring (e) RedCap for NTN.

In our view, the advantage of Proposal 1a is simplicity, convenience, and greater rationality, as currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some features may need to be modified. Afterwards, we can define corresponding signaling for certain UE capabilities that need to be modified or added.

Observation 11: Currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some features may need to be modified.

The company that support Proposal 1b is mainly concerned that some of the requirements defined for (e)RedCap UE in TN networks may not be supported or defined in NTN scenarios, and some of the requirements defined for ordinary UE in NTN networks may no longer be supported for (e)RedCap UE. Therefore, they want to define a new set of UE capabilities for (e)RedCap UE with FR1-NTN. However, this is all from the RAN4 perspective of defining requirements.

However, from the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded. In addition, most of the features of (e)RedCap UE in NTN will reuse previous UE capabilities, so it may not be necessary to define new capabilities as a whole, as there may be duplication and waste.

Observation 12: From the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded.

Observation 13: Most of the features of (e) RedCap UE in NTN will reuse previous UE capabilities, so it may not be necessary to define new capabilities as a whole.

Proposal 22: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Conclusion

This document discussed the issues for (e)RedCap RRM requirements for R19 NR NTN and presented following observations and proposals:

Proposal 1: The measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement can be defined for (e)RedCap UE with FR1-NTN bands.

Observation 1: TA validation in CG-SDT requirements has been introduced for RedCap UE with TN and the TA validation was based on the RSRP change criterion.

Observation 2: In NTN scenario, the change of distance has already been compensated by UE in UE specific TA, so RSRP change is not a good indicator for TA verification.

Proposal 2: For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN.

Observation 3: The purpose of MDT is for network optimization.

Observation 4: Although MDT requirements are important for NTN scenario, if drive tests are needed in NTN scenarios, it is more resonable to use normal UE or even dedicated terminals for this task and a RedCap UE will not be used for drive tests unless in private network.

Proposal 3: Deprioritize to define MDT requirements for (e)RedCap UE with FR1-NTN bands.

Proposal 4: For the thresholds in supported relaxations criterion and specific value of measurement relaxation factor K3, we support to reuse the parameters used in R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation).

Observation 5:For quasi-earth fixed cell LEO deployment, for inter-frequency NR cells measurement, the number of NR inter-frequency carriers also needed to be considered, so some margin of time is needed.

Proposal 5: For quasi-earth fixed cell LEO deployment,

- 2.56s eDRX cycle length can be supported.

- 10.24s eDRX cycle length will be excluded.

- 5.12s eDRX cycle length can be used in some cases and whether it is configured depends on NW.

Observation 6: The legacy searcher assumption and CSSF are related to the capability of the UE to process and depend on the type of UEs.

Proposal 6: Single searcher assumption and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN.

Observation 7: The configurable bandwidth of NTN in FR1 is only 20MHz, so the gain of FH in PRS measurement is very small.

Proposal 7: Not support FH in PRS measurement for (e)RedCap UE with FR1-NTN.

Proposal 8: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

- The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

Proposal 9: For hard Satellite switching with re-sync requirements, for 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB:

- Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the target NR SAN cell.

Proposal 10: For 2Rx (e)RedCap UEs with FR1-NTN bands,

- RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level.

- Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised.

Proposal 11: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

Proposal 12: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

Proposal 13: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

Proposal 14: The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 15: The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 16:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

- The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

Proposal 17: RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 18: At least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 19: For RACH-based/RACH-less HO, Time/location-based CHO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

- For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

Observation 9: There was still FFS on priority rule for collision case 4 in RAN1.

Proposal 20: Whether the legacy HD-FDD related scheduling availability in RRM requirement can be reused in NTN directly should wait for more RAN1 progress.

Observation 10: For introducing new subclauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes, so a new suffix after NTN suffix may need to be added.

Proposal 21: To use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

Observation 11: Currently no new features have been introduced for (e)RedCap UE with FR1-NTN, only some features may need to be modified.

Observation 12: From the perspective of RAN2’s signaling, all features introduced for (e)RedCap UEs in TN and normal UEs in NTN have not been excluded.

Observation 13: Most of the features of (e) RedCap UE in NTN will reuse previous UE capabilities, so it may not be necessary to define new capabilities as a whole.

Proposal 22: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Reference

- R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- 3GPP TS 38.133, NR Requirements for support of radio resource management