3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2501026

Athens, GR, 17th – 21st February, 2025

Title: 	Discussion on modification of existing Tx requirements for SBFD operation

Source: 	CATT

Agenda item:	7.23.2.2

Document for:	Discussion

- Background

WF [1] of RAN4#113 concluded that further studies are needed for some TX RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Tx RF Requirement

  - ACLR and OBUE

In WF [1], WF of ACLR and OBUE are shown following.

WF:

- Issue 2: Unwanted emissions

Agreement:

- FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

We agree with 1dB degradation of victim BS sensitivity.

For FR1 aggressor WA BS, the output power is 49dBm, bandwidth of channel is 100MHz, if assume the degradation of victim BS sensitivity is 1dB, the required total isolation between the two BSs is 144dB, that is, the sum of ACLR, couple loss of co-location and the rejection of RF BPF used after PA must larger than 144dB.

Table 1: Feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim BS sensitivity(dB) | [1] |
| Required total isolation(dB) | 144 |
| 1st ACLR(dB) | [45] |
| Coupling loss of co-location (dB) | [30] |
| Rejection of RF BPF used after PA (dB) | 69 |

If ACLR is 45dB, coupling loss of co-location is 30dB, rejection from RF BPF at victim BS UL sub-band must be larger than 69dB.

When two SBFD-capable BSs are co-location and the operating frequency bands of the two BSs are adjacent, the interference from BS to BS of four different BS spectrum configurations is listed in table 2.

Table 2 Mutual interference between two SBFD-capable BSs of four cases

| Case | Configuration of SBFD capable BSs | SBFD BS1 DL to SBFD BS2 UL |
| --- | --- | --- |
| 1 | SBFD BS 1: {DUD} SBFD BS 2: {DUD} |  |  |
| 2 | SBFD BS 1: {UD} SBFD BS 2: {DU} |  |  |
| 3 | SBFD BS 1: {DU} SBFD BS 2: {UD} |  |  |
| 4 | SBFD BS 1: {UD} SBFD BS 2: {UD} |  |  |

From table 2, it can be seen that case 2 is the best configuration. When two SBFD-capable BSs with adjacent operating bands configured as {DU}, and two DL sub-bands are adjacent, frequency offset between aggressor channel to victim channel is max, and this configuration can optimize the rejection at victim BS UL sub-band by RF BPF used after PA. The best configuration is not a general requirement, but can be used for some scenario.

If channel bandwidth is 100MHz, the DL sub-band widths of two BSs are both 80MHz and UL sub-band width of two BSs are both 20MHz, when the two DL sub-bands are adjacent, victim BS UL sub-band is 80MHz offset to the edge of aggressor BS passband, the BPF after PA can provide 70dB suppression.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Proposal 2: For two SBFD-capable BSs that are co-located and have adjacent operating bands, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA BS.

  - Transmitter signal quality

Joint measurement requires a new complex test model, and the start time position of EVM test is very critical when switching between SBFD slot and non-SBFD slot, and the test results will be affected by this time point, so it would be beneficial to measure the mean EVM of SBFD and non-SBFD slots separately.

Proposal 3: Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

EVM is a Tx requirement, when calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account, and data from SBFD ULs needn’t to be included.

Proposal 4: When calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Proposal 2: For two SBFD-capable BSs that are co-located and have adjacent operating bands, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA BS.

Proposal 3: Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Proposal 4: When calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account.

- Reference

[1] R4-2419908, “Way Forward for [113][307] NR_duplex_evo_BSRF”, Huawei, RAN4#113