3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2501033

Athens, GR, 17th – 21st February, 2025

Source: 	CATT

Title: 	Discussion on BS RF OTA test scope

Agenda item:	7.13.4.2

Document for:	Discussion

- Introduction

The R19 NR_BS_RF_req_evo includes a performance objective as following,

- Identify BS excessive OTA test scope cases and reduce it with the aim to shorten the test duration time for selected RF requirements, e.g., TX IM, RX out of band blocking, for NR FR1.

This contribution provides our very preliminary consideration on this.

- Discussion

In current TS 38.141-2, the following OTA requirements should be tested,

Table 1: Requirement set summary for BS type 1-O

| Requirement | BS type 1-O | Notes |
| --- | --- | --- |
| Radiated transmit power | 6.2 | EIRP |
| OTA base station output power | 6.3 | TRP |
| OTA output power dynamics | 6.4 | Directional requirement |
| OTA transmit ON/OFF power | 6.5 | EIRP |
| OTA transmitted signal quality | 6.6 | Directional requirement |
| OTA occupied bandwidth | 6.7.2 | Directional requirement |
| OTA ACLR | 6.7.3 | TRP |
| OTA out-of-band emission | 6.7.4 | TRP |
| OTA transmitter spurious emission | 6.7.5 | TRP based on directional power measurements 9 kHz-26 GHz needs to be tested CLTA related |
| OTA transmitter intermodulation | 6.8 | TRP 9 kHz-26 GHz needs to be tested CLTA related |
| OTA sensitivity | 7.2 |  |
| OTA reference sensitivity level | 7.3 |  |
| OTA dynamic range | 7.4 |  |
| OTA in-band selectivity and blocking | 7.5 |  |
| OTA out-of-band blocking | 7.6 | Requirement apply from 30 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz. The CW interfering signal shall be swept with a step size of 1 MHz within the frequency range specified in clause 7.6.5.1.1 |
| OTA receiver spurious emission | 7.7 | TRP based on the directional power measurements 30 MHz-26 GHz needs to be tested |
| OTA receiver intermodulation | 7.8 |  |
| OTA in-channel selectivity | 7.9 |  |

In general, OTA test is more time consuming and higher cost than conducted requirement. From the summary in Table 1, the test for two Tx requirements and two Rx requirements are more challenge. For Tx, OTA transmitter spurious emission and OTA transmitter intermodulation requirements are related to CLTA. CLTA is in the discussion scope in current specification, even using current approach, the measurement is very complicated. Furthermore, these two requirements are based on TRP, and a very wide frequency range should be tested for the requirement. If some simplification method can be found, it’ll reduce much test time and also the test cost. For Rx, similarly two requirements related to sweeping a wide frequency range should be studied, i.e. OTA out-of-band blocking and OTA receiver spurious emission.

Proposal 1: The following requirements can be studied to reduce the test scope for FR1 OTA conformance test,

- For Tx: OTA transmitter spurious emission, OTA transmitter intermodulation

- For Rx: OTA out-of-band blocking, OTA receiver spurious emission

For how to simplify the test, our suggestion is that firstly maybe it can be studied that if some requirements can be removed. For example, out of band blocking requirement is derived from the scenario of co-existence with GSM. Or alternatively, some simplification like choosing typical frequency points to test the requirement can also be considered. For example, if OOB requirement is still kept after discussion, then the test step size may be consider to be changed.

In general, the farther away from the filter passbands, the greater the rejection, so we can make some choices at the test points for the transmitter spurious emission and Rx out-of-band blocking requirements according to this characteristic of the filter.

Proposal 2: RAN4 to discuss if some requirements can be removed firstly, and then discuss the test method improvement.

Proposal 3: Choosing some typical frequency points or changing the step size for the whole frequency test range can be considered to reduce the test effort.

Proposal 4: In general, the farther away from the filter passbands, the greater the rejection, so we can make some choices at the test points for the transmitter spurious emission and Rx out-of-band blocking requirements according to this characteristic of the filter.

- Conclusion

This contribution provides our preliminary consideration on FR1 OTA test scope. We have the following proposals,

Proposal 1: The following requirements can be studied to reduce the test scope for FR1 OTA conformance test,

- For Tx: OTA transmitter spurious emission, OTA transmitter intermodulation

- For Rx: OTA out-of-band blocking, OTA receiver spurious emission

Proposal 2: RAN4 to discuss if some requirements can be removed firstly, and then discuss the test method improvement.

Proposal 3: Choosing some typical frequency points or changing the step size for the whole frequency test range can be considered to reduce the test effort.

Proposal 4: In general, the farther away from the filter passbands, the greater the rejection, so we can make some choices at the test points for the transmitter spurious emission and Rx out-of-band blocking requirements according to this characteristic of the filter.

- References

[1] RP-242010, “Revised WID: NR base station (BS) RF requirement evolution for R1/FR2 and testing”, ZTE, RAN#105